3GPP TSG-SA5 Meeting #164	S5-255404

Dallas, USA, 17 - 21 November 2025

Title:	LS on Analysis on the Security impacts when the same TR/TRSR is resent multiple times over the air interface

Release:	Rel-19

Work Item:	TraceQoE_OAM

Source:	SA5#164

To:	SA3

Cc:	RAN3

Contact person:	Sreekumar Pothera Kalloor

sreekumar.pk@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# 1	Overall description

SA5 agreed on a solution on Continuous MDT (C-MDT), in which the same UE provides measurements to the network across RRC States and across connected mode mobility. According to the solution introduced in TS 32.422, the activation mechanism for C-MDT is the following:

According to the solution, the management system activates C-MDT with a Trace Activation by configuring all gNBs in a given area with a given Trace Reference (TR). Once a gNB selects a UE for C-MDT, it configures the UE with both Logged and Immediate MDT. When a UE configured for C-MDT transitions from RRC_Idle or RRC_Inactive to RRC_Connected and the UE has Logged MDT measurements available it indicates availability for these Logged MDT measurements to the gNB according to legacy methods. The gNB requests the Logged MDT measurements and the UE reports these to the gNB including the TR (Trace Reference) and TRSR (Trace Recording Session Reference) of the Logged MDT Configuration. If the gNB determines (through the reported TR) that the UE is a Continuous MDT UE, it reconfigures it with both Logged MDT and Immediate MDT with the same Continuous MDT configuration and with the same TR and TRSR. This means that for every transition to RRC_Connected the UE is reconfigured with the same TR and TRSR. Also, as a consequence of this reconfiguration, the logging duration of 2 hours is renewed each time the logged MDT is reconfigured in the UE. C-MDT stops within 2 hours after the management system sends a Trace Deactivation to all the involved gNBs (duration of the logged MDT configuration in the UE).

SA5 would like to understand if there are security impacts with the above-mentioned solution in TS 32.422 in SA5. Especially for the case the same TR/TRSR is sent multiple times over the air interface.

Additionally, SA5 considers updating the current solution according to the following options:

- By updating the existing solution such that C-MDT for a selected UE continues over the duration of legacy logged MDT (2 hours), that is, by removing reconfiguration of logged MDT.

- By updating existing solution in the following two directions:

  - by limiting the duration of C-MDT to e.g. a configurable duration and

  - by mandating that every time logged MDT in the UE is reconfigured by the gNB a new TRSR shall be used.

Option 1 corresponds to reuse of legacy logged MDT for C-MDT. SA5 would like to ask SA3 whether option 2 is useful e.g., in case of a rogue gNB scenario.

# 2	Actions

To SA3

ACTION: SA5 requests SA3 to provide guidance on the possible security impacts related to sending the same TR and TRSR in the UL and DL for a given UE and for an indefinite amount of time. SA5 would also like to ask SA3 to provide feedback on the possible alternative options.

# 3	Dates of next TSG SA WG5 meetings

SA5#165		9 February - 13 February 2026		India

SA5#166		13 April - 17 April 2026			Malta