3GPP TSG-SA5 Meeting #165	S5-260362

Goa, India, 9 - 13 February 2026

Source:	Ericsson (China)

Title:	Add the use case of MDA analytics priority to MDAS capabilities

Document for:	Approval

Agenda item:	6.20.7

Spec:	3GPP TR 28.886

Version:	0.3.0

Work Item:	FS_eMDAS_Ph4

Comments

This pCR is to add use case and requirements for MDA analytics priority which relates to

- WT-3 Investigate new and enhanced analytics related capabilities

  - WT-3.4 Study enhancements to MDAS capabilities to allow MDAS consumers to provide further input when requesting analytics. The consumer input may be used to optimize the MDAS analytics to align with the needs of the MDAS consumer and to improve MDAS recommendations

.

Proposed Changes

* * * First Change * * * *

# 5. 		Use Cases

### 5.3	Investigate new and enhanced analytics related capabilities

### 5.3.X	Use case X: MDA analytics priority

#### 5.3.X.1	Description

Currently if the consumer has a task to complete, they can make an MDARequest for analytics. The MDAF/MDAS then performs analytics and provides output to the consumer which can contain recommended actions which the consumer can act upon. However, each consumers goal/priority may be different. It could be the case that the consumer is under no time constraints and would like the most energy efficient actions for completing the task, or that the consumer wants the actions that can complete the task as quickly as possible. Introduction of the priority attribute allows the consumer to specify their goal in relation to the analytics and helps the MDA producer to provide the recommended actions based on the consumer’s input.

E.g. If the consumer wants the actions to complete the task as quickly as possible, “time” could be their priority.

#### 5.3.X.2	Potential requirements

REQ-MAP-1: The MnS consumer for MDA should include capability to state their highest priority/goal for the MDA output.

#### 5.3.X.3	Potential solutions

To support MDA analytics priority. It can introduce a new attribute “priority” added to the MDARequest IOC (TS 28.104). The MDA MnS consumer may state their priority when making an MDARequest. This may contain their highest priority/goal for the MDA output. The MDAS producer can consider this priority when providing output including recommended actions.

#### 5.3.X.4	Evaluation of solutions

TBA

* * * End of Changes * * * *