3GPP TSG SA5 Meeting #165	S5-260446

Goa, India, 9-13 February 2026

Source:	Ericsson

Title:	Pseudo-CR on Solution topic 1 issue 1 using SBA

Document for:	Approval

Agenda item:	7.5.2

Spec:	3GPP TR 32.872

Version:	0.2.0

Work Item:	FS_RoamRE_CH

Comments

Addition of using service-based architecture for V-CHF detected CTF failure and using service-based architecture for CTF detected V-CHF failure solutions.

Proposed Changes

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 32.255: "5G data connectivity domain charging stage 2".

[3]	3GPP TS 32.290: "5G system; Services, operations and procedures of charging using Service Based Interface (SBI)".

[x]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

* * * Next Change * * * *

#### 5.1.4.x	Solution #1.x: Using service-based architecture for V-CHF detected CTF failure

A possible solution for key issue #1 covering requirements REQ-3GPPCH-LBIC-1, enhanced failure handling for scenarios where a failure is detected between the CTF and the V-CHF.

When the V-CHF detects a failure, it may use the procedure outlined in TS 29.500 [x] clause 6.5.3.2, to restore a connection to the CTF. If the restoring of the connection fails it is operator specific if the connection between V-CHF and H-CHF is to be maintained.

* * * Next Change * * * *

#### 5.1.4.y	Solution #1.y: Using service-based architecture for CTF detected V-CHF failure

A possible solution for key issue #1 covering requirements REQ-3GPPCH-LBIC-1, enhanced failure handling for scenarios where a failure is detected between the CTF and the V-CHF.

When the CTF detects a failure, it may use the procedure outlined in TS 29.500 [x] clause 6.5.3.3, to restore a connection V-CHF.

* * * End of Changes * * * *