3GPP TSG SA5 Meeting #165	S5-260653

Goa, India, 9-13 February 2026

Source:	Huawei

Title:	Rel-20 pCR on TR 28.881 Solution, evaluation and conclusion on intent expectation satisfied information

Document for:	Approval

Agenda item:	6.20.1

Spec:	3GPP TR 28.881

Version:	1.0.0

Work Item:	FS_IDMS_MN_Ph4

Comments

This contribution proposes to add solution, evaluation and conclusion for the requirements documented in clause 4.14.2 regarding the enhancement of intent and intent report.

Proposed Changes

* * * First Change * * * *

4.14 Use case #14:  intent expectation satisfied information

4.14.1	Description

In 3GPP TS 28.312 [1], the existing intentReport allows the IDMS consumer to obtain the values of KPI as indicated by the corresponding expectation targets. However, this information may not accurately reflect the actual situation. For example, considering an intent that includes a radio network expectation with the target aveDLRANUEthptTarget, the existing intent report provides the average value of the downlink RAN UE throughput and tells whether the intent expectation is fulfilled or not based on the average value. However, in the case where the average value is impacted by some extreme values, the fulfilled intent report will miss the information that the majority of UE experience doesn’t fulfil the UE throughput specified in the target. Such information matters because it helps the network operator to have a better understanding on the intent expectation satisfaction. The report can include the information besides the average information, which can be entity based, or time based, and is derived through statistical measurement or calculation, such as distributions, range or std deviation.

4.14.2	Potential requirements

REQ-Intent_IESI-1: The intent driven MnS producer should have a capability providing information besides average value in the intent report.

4.14.3	Potential solutions

4.14.3.1 Solution 1: Enhance Intent Report and Intent Report Control

This solution proposes to enhance the expectation fulfilment results included in intent fulfilment report defined in clause 6.2.1.3.6, and intentReportControl defined in clause 6.2.1.3.16.

Enhancement Aspect #1: In order to enable the MnS consumer to receive more fulfilment information besides the binary fulfilment result, the detailed satisfied information is introduced in the existing data type ExpectationFulfilmentResult, which reflects how comprehensive and consistent the expectation objects achieve the target requirements during the intent fulfilment phase.

The data type ExpectationFulfilmentResult is enhanced with:

(1) The detailed satisfied information is conveyed through the following.

- FulfilmentStatisticsInfo: an attribute which contains the fulfilment information besides the average value. It may contain statistical measurement or calculation result. Examples could include fulfilment ratio, distributions, range or std deviation.

4.14.4	Evaluation of potential solutions

Enhancement Aspect #1, i.e., adding one new attribute to cover satisfied information in ExpectationFulfilmentResult is feasible and independently to address the requirement REQ-Intent_IESI-1It is recommended to proceed to normative phase.

* * * Next Change * * * *

# 5	Conclusions and Recommendations

Editor's note: this clause will contain conclusions and recommendations for corresponding key issues identified in clause 4.

## 5.14 Use case #14: intent expectation satisfied information

The description, requirements and potential solutions for intent expectation satisfied information are described in clause 4.14.  This use case proposes to update intent report to support satisfied information. It is recommended to enhance the ExpectationFulfilmentResult defined in 3GPP TS 28.312 [1] to support above management requirement. The detailed solution in clause 4.1.3 is used as baseline for normative work.

* * * End of Changes * * * *