3GPP TS 32.532 V9.2.0 (2009-12)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Telecommunication management;

Software management Integration Reference Point (IRP); Information Service (IS)

(Release 9)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.	
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.	
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

Management, OAM

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2009, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI currently being registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	7

Introduction	7

1	Scope	8

2	References	8

3	Definitions and abbreviations	8

3.1	Definitions	8

3.2	Abbreviations	8

4	Information Object Classes	9

4.1	Imported information entities and local labels	9

4.2	Class diagram	9

4.2.1	Attributes and relationships	9

4.2.2	Inheritance	10

4.3	Information object class definitions	11

4.3.1	GenManCapability	11

4.3.1.1	Definition	11

4.3.1.2	Attributes	11

4.3.1.3	Notifications	11

4.3.2	GenManProfile	12

4.3.2.1	Definition	12

4.3.2.2	Attributes	12

4.3.2.3	Notifications	12

4.3.3	GenManProcess	13

4.3.3.1	Definition	13

4.3.3.2	Attributes	13

4.3.3.3	Notifications	13

4.3.4	SwMCapability	14

4.3.4.1	Definition	14

4.3.4.2	Attributes	14

4.3.4.3	Attribute constraints	14

4.3.4.4	Notifications	14

4.3.5	SwMProfile	15

4.3.5.1	Definition	15

4.3.5.2	Attributes	15

4.3.5.3	Attribute constraints	15

4.3.5.4	Notifications	15

4.3.6	SwMProcess	16

4.3.6.1	Definition	16

4.3.6.2	Attributes	16

4.3.6.3	Notifications	16

4.3.7	SwMIRP	17

4.3.7.1	Definition	17

4.3.7.2	Attributes	17

4.3.7.3	Notifications	17

4.3.8	SwMManagedEntity	17

4.3.8.1	Definition	17

4.4	Information relationship definitions	17

4.4.1	relation-swMIRP-swMCapability (M)	17

4.4.1.1	Definition	17

4.4.1.2	Roles	17

4.4.1.3	Constraints	17

4.4.2	relation-SwmIRP-swMProfile (M)	18

4.4.2.1	Definition	18

4.4.2.2	Roles	18

4.4.2.3	Constraints	18

4.4.3	relation-swMIRP-swMProcess (M)	18

4.4.3.1	Definition	18

4.4.3.2	Roles	18

4.4.3.3	Constraints	18

4.4.4	relation-swMCapabilites-swMProfile (M)	19

4.4.4.1	Definition	19

4.4.4.2	Roles	19

4.4.4.3	Constraints	19

4.4.5	relation swMProfile-swMProcess (M)	19

4.4.5.1	Definition	19

4.4.5.2	Roles	19

4.4.5.3	Constraints	19

4.5	Information attribute definitions	20

4.5.1	Definition and legal values	20

4.5.2	Constraints	22

5	IRP descriptions: Interface Definitions	23

5.1	Class diagram representing interfaces	23

5.2	Generic rules	24

5.3	SwMIRPOperations_1 Interface (M)	25

5.3.1	Operation listSwMCapabilities (M)	25

5.3.1.1	Definition	25

5.3.1.2	Input parameters	25

5.3.1.3	Output parameters	25

5.3.1.4	Post-condition	25

5.3.1.5	Exceptions	26

5.3.1.5.1	operation_failed	26

5.3.2	Operation listSwMProfiles (M)	26

5.3.2.1	Definition	26

5.3.2.2	Input parameters	26

5.3.2.3	Output parameters	26

5.3.3	Operation createSwMProfile (M)	26

5.3.3.1	Definition	26

5.3.3.2	Input parameters	27

5.3.3.3	Output parameters	27

5.3.4	Operation deleteSwMProfile (M)	27

5.3.4.1	Definition	27

5.3.4.2	Input parameters	27

5.3.4.3	Output parameters	27

5.3.5	Operation listSwMProcesses (M)	28

5.3.5.1	Definition	28

5.3.5.2	Input parameters	28

5.3.5.3	Output parameters	28

5.3.6	Operation resumeSwMProcess (M)	28

5.3.6.1	Definition	28

5.3.6.2	Input parameters	28

5.3.6.3	Output parameters	29

5.3.7	Operation swFallback (M)	29

5.3.7.1	Definition	29

5.3.7.2	Input parameters	29

5.3.7.3	Output parameters	29

5.3.8	Operation terminateSwMProcess (M)	30

5.3.8.1	Definition	30

5.3.8.2	Input parameters	30

5.3.8.3	Output parameters	30

5.4	SwMIRPOperations_2 Interface (O)	31

5.4.1	Operation changeSwMProfile (O)	31

5.4.1.1	Definition	31

5.4.1.2	Input parameters	31

5.4.1.3	Output parameters	31

5.4.1.4	Constraints	32

5.5	SwMIRPNotifications_1 Interface (M)	33

5.5.1	Notification notifySwMProfileCreation (M)	33

5.5.1.1	Definition	33

5.5.1.2	Input parameters	33

5.5.2	Notification notifySwMProfileDeletion (M)	33

5.5.2.1	Definition	33

5.5.2.2	Input parameters	33

5.5.3	Notification notifySwMProcessCreation (M)	34

5.5.3.1	Definition	34

5.5.3.2	Input parameters	34

5.5.4	Notification notifySwMProcessStage (M)	34

5.5.4.1	Definition	34

5.5.4.2	Input parameters	34

5.5.5	Notification notifySwMProcessDeletion (M)	35

5.5.5.1	Definition	35

5.5.5.2	Input parameters	35

5.5.6	Notification notifyNewSwAvailability (M)	35

5.5.6.1	Definition	35

5.5.6.2	Input parameters	35

5.6	SwMIRPNotifications_2 Interface (O)	36

5.6.1	Notification notifySwMProfileChange (C/O)	36

5.6.1.1	Definition	36

5.6.1.2	Input parameters	36

5.7	SwMIRPOperations_3 Interface (M)	36

5.7.1	Operation downloadNESw (M)	36

5.7.1.1	Definition	36

5.7.1.2	Input parameters	37

5.7.1.3	Output parameters	37

5.7.1.4	Pre condition	38

5.7.1.5	Post-condition	38

5.7.1.6	Exceptions	38

5.7.2	Operation activateNESw (M)	38

5.7.2.1	Definition	38

5.7.2.2	Input parameters	38

5.7.2.3	Output parameters	39

5.7.2.4 Pre condition	39

5.7.2.5 Post-condition	39

5.7.2.6 Exceptions	39

5.8	SwMIRPOperations_4 Interface (O)	39

5.8.1	Operation installNESw (O)	39

5.8.1.1	Definition	39

5.8.1.2	Input parameters	40

5.8.1.3	Output parameters	40

5.8.1.4	Pre condition	40

5.8.1.5	Post-condition	40

5.8.1.6	Exceptions	41

5.9	SwMIRPNotifications_3 Interface (M)	42

5.9.1	Notification notifyDownloadNESwStatusChanged (M)	42

5.9.1.1	Definition	42

5.9.1.2	Input parameters	43

5.9.1.3	Triggering Event	43

5.9.1.3.1	From State	43

5.9.1.3.2	To State	43

5.9.1.4	Constraints	43

5.9.2	Notification notifyActivateNESwStatusChanged (M)	44

5.9.2.1	Definition	44

5.9.2.2	Input parameters	44

5.9.2.3	Triggering Event	45

5.9.2.3.1	From State	45

5.9.2.3.2	To State	45

5.9.2.4	Constraints	45

5.10	SwMIRPNotifications_4 Interface (O)	45

5.10.1	Notification notifyInstallNESwStatusChanged (O)	45

5.10.1.1	Definition	45

5.10.1.2	Input parameters	46

5.10.1.3	Triggering Event	46

5.10.1.3.1	From State	46

5.10.1.3.2	To State	46

5.10.1.4	Constraints	46

Annex A (informative):	Change history	47

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

The present document is part of a TS-family covering the 3rd Generation Partnership Project Technical Specification Group Services and System Aspects, Telecommunication management; as identified below:

32.531:	Software management; Concepts and Integration Reference Point (IRP) Requirements

32.532:	Software management Integration Reference Point (IRP); Information Service (IS)

32.533:	Software management Integration Reference Point (IRP); Common Object Request Broker Architecture (CORBA) Solution Set (SS)

# 1	Scope

The present document contains the Software Management Interface IRP Information Services descriptions.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 32.101: "Telecommunication management; Principles and high level requirements".

[3]	3GPP TS 32.102: "Telecommunication management; Architecture".

[4]	3GPP TS 21.905: "Vocabulary for 3GPP Specifications".

[5]	3GPP TR 32.816: "Telecommunication management; Study on Management of Evolved Universal Terrestrial Radio Access Network (E-UTRAN) and Evolved Packet Core (EPC)".

[6]	3GPP TS 32.531: "Telecommunication management; Software management; Concepts and Integration Reference Point (IRP) Requirements".

[7]	3GPP TS 32.622: "Telecommunication management; Generic network resources Integration Reference Point (IRP); Network Resource Model (NRM)".

[8]	3GPP TS 32.312: "Telecommunication management; Generic Integration Reference Point (IRP) management: Information Services".

[9]	3GPP TS 32.302: Telecommunication management; Configuration Management (CM); Notification Integration Reference Point (IRP); Information Service (IS).

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TS 32.101 [2], TS 32.102 [3] and TR 21.905 [4] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TS 32.531 [6], TS 32.101 [1], TS 32.102 [2] and TS 21.905 [4], in that order.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [4], TS 32.531 [6] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [4] and TS 32.531 [6].

# 4	Information Object Classes

## 4.1	Imported information entities and local labels

| Label reference | Local label |
| --- | --- |
| 3GPP TS 32.622 [7], information object class, Top | top |
| 3GPP TS 32.312 [8], information object class, managedGenericIRP | managedGenericIRP |

## 4.2	Class diagram

### 4.2.1	Attributes and relationships

The diagram reflects the definitions in the text of the following clauses. In case of conflict text takes precedence.

### 4.2.2	Inheritance

The diagram reflects the definitions in the text of the following clauses. In case of conflict text takes precedence.

ManagedGenericIRP

(from TS 32.312)

<<InformationObjectClass>>

SwManagement IRP

<<InformationObjectClass>>

Top

(from TS 32.622)

<<InformationObjectClass>>

SwMCapabilities

swVersionToBeInstalledOfferList

<<InformationObjectClass>>

SwMProfile

swVersionToBeInstalled

<<InformationObjectClass>>

SwMProcess

<<InformationObjectClass>>

GenManCapabilities

id

nEInformation

offeredFinalAdministrativeStateInformation

stepsAndOfferedStopPointList

<<InformationObjectClass>>

GenManProcess

id

nEIdentification

profileId

stepInfoList

<<InformationObjectClass>>

GenManProfile

id

selectedFinalAdministrativeState

nEInformation

stepsAndSelectedStopPointList

versionNumber

<<InformationObjectClass>>

## 4.3	Information object class definitions

### 4.3.1	GenManCapability

#### 4.3.1.1	Definition

This object class is a support object class. Sub-classes of this IOC represent the IRPAgent’s capability in support of automated management.

It is created by the IRPAgent and cannot be modified by the IRPManager. 
An instance of a sub-class of genManCapability object is valid for a certain NE type or a set of NE types. 
Multiple genManCapability objects may be instantiated in the IRPAgent.

The object identifies 
a) the sequence of the self-configuration steps
and for each step
a.1) the possibility, whether before the step a stop point can be selected, such that the self-configuration step is suspended and waits for a request by the IRPManager to resume.
b) the final administrativeState (ITU-T X.731) of the NE after successful self-configuration.

#### 4.3.1.2	Attributes

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| id | M | M | - |
| nEInformation | M | M | - |
| stepsAndOfferedStopPointList | M | M | - |
| offeredFinalAdministrativeStateInformation | M | M | - |

#### 4.3.1.3	Notifications

None.

### 4.3.2	GenManProfile

#### 4.3.2.1	Definition

This object class is a support object class. Sub-classes of this IOC represent the IRPManager’s decision related to automated management.

An instance of a sub-class of GenManProfile is valid for a certain NE type or a set of NE types. 
For an NE starting its self-configuration process (see genManProcess) there shall be no ambiguity which instance of a sub-class of GenManProfile is valid for a certain NE type or a set of NE types. 
Multiple instances of sub-classes of GenManProfile objects may be instantiated in the IRPAgent.

By using an instance of a sub-class of this object the IRPManager decides which of the possible stop points offered in the related instance of a sub-class of genManCapability are used to suspend the automated management process of the specified NE type (or set of NE types) and which of the offeredFinalAdministrativeStateInformation is selected.

#### 4.3.2.2	Attributes

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| id | M | M | - |
| versionNumber | M | M | - |
| nEInformation | M | M | - |
| stepsAndSelectedStopPointList | M | M | - |
| selectedFinalAdministrativeState | M | M | - |

#### Notifications

None.

### 4.3.3	GenManProcess

#### 4.3.3.1	Definition

This object class is a support object class. Sub-classes of this IOC describe the automated management process for an NE. They allow the IRPManager to be informed about the current progress of the process and where stop points are set. No intervention of the IRPManager is foreseen except resume after a stop point was reached or termination of the self-configuration.

When the automated management process for an NE starts, an instance of the sub-class of genManProcess is created automatically.

The steps in the stepInfoList shall conform to the content of the relevant sub-class of genManProfile instance.
Example: 
If the stepsAndOfferedStopPointList of a sub-class instance of genManProfile indicates stopPointCanBeSetBeforeThisStep for step X, then the entry for step X in the stepInfoList of the sub-class instance of genManProcess can only have the value stopPointIsNotSet .

When there is no relevant genManProfile at creation time of genManProcess, then the IRPAgent creates the genManProcess based on the relevant genManCapability. In this case preferably no stop point shall be set in the self configuration process.

When the last step of the self configuration process is completed successfully, the genManProcess instance is deleted automatically.

When self configuration process is terminated by the IRPManager, the genManProcess instance is deleted automatically.

#### 4.3.3.2	Attributes

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| id | M | M | - |
| nEIdentification | M | M | - |
| profileId | M | M | - |
| stepInfoList | M | M | - |

#### Notifications

None.

### 4.3.4	SwMCapability

#### 4.3.4.1	Definition

This object class is a sub-class of genManCapability and represents the IRPAgent’s capability in support of SWM.

It is created by the IRPAgent and cannot be modified by the IRPManager. 
A SwMManagementCapability object is valid for a certain NE type or a set of NE types with a certain SW version or set of versions. For an NE there shall be no ambiguity which SwMManagementCapability object is valid for the NE.
Multiple SwMManagementCapability objects may be instantiated in the IRPAgent.

The object identifies 
a) the sequence of the self-configuration steps
and for each step
a.1) the possibility, whether before the step a stop point can be selected, such that the self-configuration step is suspended and waits for a request by the IRPManager to resume.
b) the final administrativeState of the NE after successful self-configuration.

Information on Requirements Traceability:

| Referenced TS | Requirement label | Comment |
| --- | --- | --- |
| 3GPP TS 32.531 [6] | REQ_SWM_FUN_5 |  |
| 3GPP TS 32.531 [6] | REQ_ASWM_FUN_1 |  |

#### 4.3.4.2	Attributes

All attributes inherited from IOC GenManCapability.

Additional attributes:

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| swVersionToBeInstalledOfferList | CM *) | M | - |

*) Condition: objectClass_is_swMCapability

#### 4.3.4.3	Attribute constraints

| Name | Definition |
| --- | --- |
| objectClass_is_swMCapability | objectClass is equal to swMCapabilities |

#### 4.3.4.4	Notifications

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyNewSwAvailability | O |  |

### 4.3.5	SwMProfile

#### 4.3.5.1	Definition

This object class is a sub-class of genManProfile. It allows the IRPManager to select from the stop points offered in the swMCapabilites object those which should be used to stop the SW management process for NEs, which fit to the nEInformation and swVersionToBeInstalled, and which of the offeredFinalAdministrativeStateInformation is selected. 
For an NE starting its SWM process there shall be no ambiguity which swMManagementProfile is valid for the NE. Therefore the nEInformation of different swMProfile instances shall not intersect. Example for a not allowed intersection: profile 1 has nEInformation=(neType=eNB), profile 2 has nEInformation=((neType=eNB) and (Id=1)) ).

Information on Requirements Traceability:

| Referenced TS | Requirement label | Comment |
| --- | --- | --- |
| 3GPP TS 32.531 [6] | REQ_ASWM_FUN_1 |  |
| 3GPP TS 32.531 [6] | REQ_ASWM_FUN_2 |  |
| 3GPP TS 32.531 [6] | REQ_ASWM_FUN_3 |  |
| 3GPP TS 32.531 [6] | REQ_ASWM_FUN_4 |  |
| 3GPP TS 32.531 [6] | REQ_SWM_FUN_7 | The part of the requirement to avoid service impact can be fulfilled by creating a swmProfile with selectedFinalAdministrativeState equal locked |

#### 4.3.5.2	Attributes

All attributes inherited from IOC GenManProfile.

Additional attributes:

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| swVersionToBeInstalled | CM | M | - |

Condition: objectClass_is_swMProfile

#### 4.3.5.3	Attribute constraints

| Name | Definition |
| --- | --- |
| objectClass_is_swMProfile | objectClass is equal to swMProfile |

#### 4.3.5.4	Notifications

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifySwMProfileCreation | M |  |
| notifySwMProfileChange | CM | Condition: Present if operation changeSwMProfile is supported. |
| notifySwMProfileDeletion | M |  |

### 4.3.6	SwMProcess

#### 4.3.6.1	Definition

This object class is a sub-class of genManProcess . It describes the SW management process for an NE. It allows the IRPManager to be informed about the current progress of the SWM process and where stop points are set. No intervention of the IRPManager is foreseen except to provide indication to resume after a stop point was reached or to abort the self-configuration.

When the automated management process for an NE starts, an instance of the swMProcess is created automatically.

The id of the swMProcess shall be identical to the identifier of the NE and identify the swMProcess instance uniquely.

The steps in the stepInfoList shall conform to the content of the relevant swMProfile instance.
Example: 
If the stepsAndOfferedStopPointList of swMProfile indicates stopPointCanBeSetBeforeThisStep for step X, then the entry for step X in the stepInfoList of swMProcess can only have the value stopPointIsNotSet .

When there is no relevant swMProfile at creation time of swMProcess, then the IRPAgent creates the swMProcess based on the relevant swMCapability. In this case preferably no stop point shall be set in the self configuration process.

When the last step of the self configuration process is completed successfully, the swMProcess instance is deleted automatically.

When self configuration process is terminated by the IRPManager, the swMProcess instance is deleted automatically.

Information on Requirements Traceability:

| Referenced TS | Requirement label | Comment |
| --- | --- | --- |
| 3GPP TS 32.531 [6] | REQ_ASWM_FUN_2 |  |
| 3GPP TS 32.531 [6] | REQ_ASWM_FUN_3 |  |
| 3GPP TS 32.531 [6] | REQ_ASWM_FUN_4 |  |
| 3GPP TS 32.531 [6] | REQ_ASWM_FUN_5 |  |
| 3GPP TS 32.531 [6] | REQ_ASWM_FUN_6 |  |

#### 4.3.6.2	Attributes

All attributes inherited from IOC GenManProcess.

Additional attributes: None.

#### 4.3.6.3	Notifications

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifySwMProcessCreation | M |  |
| notifySwMProcessStage | M |  |
| notifySwMProcessDeletion | M |  |

### 4.3.7	SwMIRP

#### 4.3.7.1	Definition

This information object represents a Software Management IRP. It inherits from IOC managedGenericIRP.

#### 4.3.7.2	Attributes

All attributes inherited from IOC managedGenericIRP.

Additional attributes: None.

#### 4.3.7.3	Notifications

All notifications inherited from IOC managedGenericIRP.

Additional notifications: None.

### 4.3.8	SwMManagedEntity

#### 4.3.8.1	Definition

The proxy IOC, SwMManagedEntity represents the role that can be played by an instance of a managed element. The objectClass and objectInstance identify a managed element instance.

## 4.4	Information relationship definitions

### 4.4.1	relation-swMIRP-swMCapability (M)

#### 4.4.1.1	Definition

This represents the relationship between SwMIRP and SwMCapability.

#### 4.4.1.2	Roles

| Name | Definition |
| --- | --- |
| theSwmIRP | It represents the SwmIRP. |
| theSwMCapability | It represents the SwMCapability |

#### 4.4.1.3	Constraints

There is no constraint for this relationship.

### 4.4.2	relation-SwmIRP-swMProfile (M)

#### 4.4.2.1	Definition

This represents the relationship between SwmIRP and SwMProfile.

#### 4.4.2.2	Roles

| Name | Definition |
| --- | --- |
| theSwmIRP | It represents the SwmIRP. |
| theSwMProfile | It represents the SwMProfile. |

#### 4.4.2.3	Constraints

There is no constraint for this relationship.

### 4.4.3	relation-swMIRP-swMProcess (M)

#### 4.4.3.1	Definition

This represents the relationship between SwmIRP and SwMProcess.

#### 4.4.3.2	Roles

| Name | Definition |
| --- | --- |
| theSwMIRP | It represents the SwMIRP. |
| theSwMProcess | It represents the SwMProcess. |

#### 4.4.3.3	Constraints

There is no constraint for this relationship.

### 4.4.4	relation-swMCapabilites-swMProfile (M)

#### 4.4.4.1	Definition

This represents the relationship between swMCapability and swMProfile.

#### 4.4.4.2	Roles

| Name | Definition |
| --- | --- |
| theSwMCapability | It represents the swMCapability. |
| theSwMProfile | It represents the swMProfile. |

#### 4.4.4.3	Constraints

A relation can only exist between a SwMProfile and a SwMCapability when

a) all steps which are entries in the stepsAndSelectedStopPointList of SwMProfile have stopPointCanBeSetBeforeThisStep = Yes in the stepsAndOfferedStopPointList of the SwMCapability.

b) nEInformation of SwMProfile is a subset of nEInformation of  SwMCapability.

### 4.4.5	relation swMProfile-swMProcess (M)

#### 4.4.5.1	Definition

This represents the relationship between SwMProfile and SwMProcess.

#### 4.4.5.2	Roles

| Name | Definition |
| --- | --- |
| theSwMProfile | It represents the theSwMProfile. |
| theSwMProcess | It represents the SwMProcess. |

#### 4.4.5.3	Constraints

A SwMProcess shall perform all self-configuration steps according to stepsAndOfferedStopPointList of SwMProfile.

A relation can only exist between a SwMProcess and a SwMProfile when

nEIdentification of SwMProcess falls into nEInformation of SwMProfile.

## 4.5	Information attribute definitions

### 4.5.1	Definition and legal values

| Attribute Name | Definition | Legal Values |
| --- | --- | --- |
| id | It identifies uniquely an instance of its object class. |  |
| nEIdentification | This attribute identifies the NE for which the self management activity is done. |  |
| nEInformation | This attribute defines the neType or NE instance/s - with optional software identification information - , for which this capability/profile instance is valid. | ”NE instance/s” only applies for instance/s already known to the IRPManager, e.g. in case of re-configuration or SW update. |
| startStepName | nameOfStep, this attribute defines the start step for resume operation. | The legal value of startStepName could be one of the step which defined in stepsAndOfferedStopPointList |
| swVersionToBeInstalled | This attribute describes which SW identification information shall be used at the end of self management in NEs for which this swMCapability/swMProfile applies. |  |
| stepsAndOfferedStopPointList | Each entry in the list contains for each step the following information: - nameOfStep: This list shall be exhaustive; if a certain step is not visible or not supported in the SWM process, then it shall not be shown (listed) in the stepsAndOfferedStopPointList. - sequenceNumberInProcess - stopPointCanBeSetBeforeThisStep | nameOfStep: swDownload, swInstallation, swActivation More values for nameOfStep may be used by other IRPs. All steps may be offered as stop points. sequenceNumberInProcess: Positive Integer stopPointCanBeSetBeforeThisStep: Yes, No |
| stepsAndSelectedStopPointList | Each entry in the list contains for each step the following information: - nameOfStep: - sequenceNumberInProcess - stopPointSetIndication | nameOfSwMStep, sequenceNumberInProcess: see stepsAndOfferedStopPointList stopPointSetIndication: stopPointIsSetBeforeThisStep, stopPointIsNotSet |
| stepInfoList | This list attribute contains information about all steps and how far they have progressed. Each entry in the list contains: - nameOfStep - sequenceNumberInProcess - stopPointSetIndication - stepProgress | nameOfSwMStep, sequenceNumberInProcess: see stepsAndOfferedStopPointList stopPointSetIndication: see stepsAndSelectedStopPointList stepProgress: notYetStarted, running, completed, awaitingResume, failure, terminated |
| swMprocessList | This attribute contains information about the instances of swMProcess . Each entry in the list contains (SET OF): - id - nEIdentification - stepInfoList | See individual definitions of the list entry content. |
| offeredFinalAdministrativeStateInformation | It describes which selection is offered regarding the administrativeState of the NE after successful automated management: If it may have the value locked or unlocked or if the value of the administrativeState may be determined by the configuration data which is uploaded in the course of the automated management. | One or more of the following values: locked, unlocked, determinedByConfigurationData The value unlocked should always be present. |
| selectedFinalAdministrativeState | Determines which of the offers made regarding the administrativeState of the NE after successful self-configuration is taken. | One of the following values: locked, unlocked, determinedByConfigurationData Default value is value unlocked. |
| swVersionToBeInstalledOfferList | This list describes for which SW version/s the capability object is valid. | Minimum size of list: 1 entry |
| versionNumber | This number is the version number of a profile. Its value is 1 when a profile is created. It is incremented by 1 each time a profile is successfully changed. | Integer |
| profileId | This parameter records the identification of the profile used by the process. It consists of two data: - id (of the profile) - versionNumber | See versionNumber |
| matchingNEInformation | This parameter records the information of the NE which was matching with the nEInformation of the profile when determining which profile is to be used for the process. | See nEInformation |
| result | This parameter records the result of an operation. | success, failure, stepNameNotMatch: The current process step doesn’t match the startStepName in the operation. nEInformationIntersection : There shall be no ambiguity which swMManagementProfile is valid for the NE. Therefore the nEInformation of different swMProfile instances shall not intersect. Example for a not allowed intersection: profile 1 has nEInformation=(neType=eNB), profile 2 has nEInformation=((neType=eNB) and (Id=1)). |

### 4.5.2	Constraints

| Name | Definition |
| --- | --- |
| FFS |  |

# 5	IRP descriptions: Interface Definitions

## 5.1	Class diagram representing interfaces

Additionally, the operations and notifications of this document are specified and grouped under Interfaces as shown in the following sections. To allow the flexible support of the necessary and sufficient operations and notifications for software management, the operations and notifications of this specification are packaged into two groups, one related to automatic software management (ASWM) and the other related to non-automated software management (NASWM).

Automatic Software Management requires the following operations and notifications:

- SwMOperations_1 and SwMNotifications_1 shall be mandatory

- SwMOperations_2 and SwMNotifications_2 shall be optional

Non Automatic Software Management requires the following operations and notifications:

- SwMOperations_3 and SwMNotifications_3 shall be mandatory

- SwMOperations_4 and SwMNotifications_4 shall be optional

Figure 5.1-2 Operations and Notifications for ASWM

Figure 5.1-3 Operations and Notifications for NASWM

## 5.2	Generic rules

-	Rule 1: each operation with at least one input parameter supports a pre-condition valid_input_parameter which indicates that all input parameters shall be valid with regards to their information type. Additionally, each such operation supports an exception operation_failed_invalid_input_parameter which is raised when pre-condition valid_input_parameter is false. The exception has the same entry and exit state.

-	Rule 2: each operation with at least one optional input parameter supports a set of pre-conditions supported_optional_input_parameter_xxx where "xxx" is the name of the optional input parameter and the pre-condition indicates that the operation supports the named optional input parameter. Additionally, each such operation supports an exception operation_failed_unsupported_optional_input_parameter_xxx which is raised when (a) the pre-condition supported_optional_input_parameter_xxx is false and (b) the named optional input parameter is carrying information. The exception has the same entry and exit state.

-	Rule 3: each operation shall support a generic exception operation_failed_internal_problem which is raised when an internal problem occurs and that the operation cannot be completed. The exception has the same entry and exit state.

NOTE:	These rules are mapped at the solution set level. Pre-conditions and exceptions, generated by these rules, need not appear explicitly in the present document.

## 5.3	SwMIRPOperations_1 Interface (M)

Editor’s Note: Whether automatic software management interface is optional or mandatory need further study.

### 5.3.1	Operation listSwMCapabilities (M)

#### 5.3.1.1	Definition

This operation allows the IRPManager to determine on the Itf-N interface which steps in the SW management are performed in NEs of a certain type, what is done by the NE in case a step does not perform normally and before which steps a stop point can be set, such that the software download halts and waits for a continuation request by the IRPManager.

Information on Requirements Traceability:

| Referenced TS | Requirement label | Comment |
| --- | --- | --- |
| 3GPP TS 32.531 [6] | REQ_ASWM_FUN_1 |  |

#### 5.3.1.2	Input parameters

| Parameter Name | Qualifier | Information type | Comment |
| --- | --- | --- | --- |
| nEInformation | M | swM.nEInformation | If this input parameter contains no information, all (offered) SwMCapability instances are to be listed in the output. |

#### 5.3.1.3	Output parameters

| Parameter Name | Qualifier | Matching Information | Comment |
| --- | --- | --- | --- |
| capabilityList | M | swM.capabilityList | Each entry in the list contains: - Id of SwMCapability - nEInformation of SwMCapability - swVersionToBeInstalledOfferList of SwMCapability - stepsAndOfferedStopPointList of SwMCapability - offeredFinalAdministrativeStateInformation of SwMCapability |
| result | M | swM.result | result=success and empty swMCapabilityList mean: No instance found. |

#### 5.3.1.4	Post-condition

| Assertion Name | Definition |
| --- | --- |
| dataDelivered | The requested data is delivered. |

#### 5.3.1.5	Exceptions

##### 5.3.1.5.1	operation_failed

| Exception Name | Definition |
| --- | --- |
| operation_failed | Condition: Pre-condition is false or post-condition is false. Returned Information: The output parameter result. Exit state: Entry state. |

### 5.3.2	Operation listSwMProfiles (M)

#### 5.3.2.1	Definition

This operation allows the IRPManager to find out which instances of SwMProfile are valid NEs of a certain type.

Information on Requirements Traceability:

| Referenced TS | Requirement label | Comment |
| --- | --- | --- |
| 3GPP TS 32.531 [6] | REQ_ASWM_FUN_1 |  |
| 3GPP TS 32.531 [6] | REQ_ASWM_FUN_2 |  |

#### 5.3.2.2	Input parameters

| Parameter Name | Qualifier | Information type | Comment |
| --- | --- | --- | --- |
| nEInformation | M | swM.nEInformation | If this input parameter contains no information, all profile instances are to be listed in the output. |

#### 5.3.2.3	Output parameters

| Parameter Name | Qualifier | Matching Information | Comment |
| --- | --- | --- | --- |
| swMProfileList | M | swM.swMProfileList | Each entry in the list contains: - Id of profile - versionNumber of swMprofile - nEInformation of profile - stepsAndSelectedStopPointList of profile - selectedFinalAdministrativeState of profile - conditionally swVersionToBeInstalled of swMprofile |
| result | M | swM.result |  |

### 5.3.3	Operation createSwMProfile (M)

#### 5.3.3.1	Definition

This operation allows the IRPManager to establish an instance of SwMProfile to be valid for NEs of a certain type.

Information on Requirements Traceability:

| Referenced TS | Requirement label | Comment |
| --- | --- | --- |
| 3GPP TS 32.531 [6] | REQ_ASWM_FUN_2 |  |

#### 5.3.3.2	Input parameters

| Parameter Name | Qualifier | Information type | Comment |
| --- | --- | --- | --- |
| id | O | swM.id | Identifier of swMprofile |
| nEInformation | M | swM.nEInformation | See 4.5 |
| swVersionToBeInstalled | M | swM.swVersionToBeInstalled | See 4.5 |
| stepsAndSelectedStopPointList | M | swM.stepsAndSelectedStopPointList | See 4.5 |
| selectedFinalAdministrativeState | M | swM.selectedFinalAdministrativeState | See 4.5 |

#### 5.3.3.3	Output parameters

| Parameter Name | Qualifier | Matching Information | Comment |
| --- | --- | --- | --- |
| id | CM | SwMProfile.id | See the definition of the result field described below |
| result | M | ENUM { success, failure, nEInformationIntersection, notAllowedBecauseOfOngoingSwmActivity } | If result = success , then parameter id contains the id of the created swMProfile. If result = failure , then parameter id is absent. If result = nEInformationIntersection, then parameter id contains the id of a swMProfile whose nEInformation would intersect with the proposed nEInformation for the new swMProfile, which was not created in this case. If result = notAllowedBecauseOfOngoingSwmOperation, then parameter id is absent. |

### 5.3.4	Operation deleteSwMProfile (M)

#### 5.3.4.1	Definition

This operation allows the IRPManager to delete an instance of swMProfile.

Information on Requirements Traceability:

| Referenced TS | Requirement label | Comment |
| --- | --- | --- |
| 3GPP TS 32.531 [6] | REQ_ASWM_FUN_2 |  |

#### 5.3.4.2	Input parameters

| Parameter Name | Qualifier | Information type | Comment |
| --- | --- | --- | --- |
| id | M | swM.id | Identifier of swMprofile |

#### 5.3.4.3	Output parameters

| Parameter Name | Qualifier | Matching Information | Comment |
| --- | --- | --- | --- |
| result | M | swM.result |  |

### 5.3.5	Operation listSwMProcesses (M)

#### 5.3.5.1	Definition

This operation allows the IRPManager to find out the status of one or several swMProcess instances

Information on Requirements Traceability:

| Referenced TS | Requirement label | Comment |
| --- | --- | --- |
| 3GPP TS 32.531 [6] | REQ_ASWM_FUN_4 |  |

#### 5.3.5.2	Input parameters

| Parameter Name | Qualifier | Information type | Comment |
| --- | --- | --- | --- |
| nEIdentification | O | swM.nEIdentification | It describes for which NE the swMprocess is to be listed. If this parameter is not present, all swMprocess instances are to be listed in the output. |

#### 5.3.5.3	Output parameters

| Parameter Name | Qualifier | Matching Information | Comment |
| --- | --- | --- | --- |
| swMprocessList | M | swM.swMprocessList | See 4.5 |
| result | M | swM.result | result=success and empty swMProcessList mean: No instance found |

### 5.3.6	Operation resumeSwMProcess (M)

#### 5.3.6.1	Definition

This operation allows the IRPManager to resume a SW management process which currently has stopped at a stop point step.

Information on Requirements Traceability:

| Referenced TS | Requirement label | Comment |
| --- | --- | --- |
| 3GPP TS 32.531 [6] | REQ_ASWM_FUN_3 |  |

#### 5.3.6.2	Input parameters

| Parameter Name | Qualifier | Information type | Comment |
| --- | --- | --- | --- |
| id | M | swM.id | Identifier of swMprocess |
| startStepName | M | swM.NameOfStep | The start step for the resume operation. If the current process step is equal to the startStepName value, the process will start from startStepName. If the current process step does not match the startStepName value, then it will be indicated in the result of the operation. Not matching startStepName value can either be the case that the process has already started from the specified step (i.e. request is too late) or that the process has not yet reached the specified step (i.e request is too early). |

#### 5.3.6.3	Output parameters

| Parameter Name | Qualifier | Matching Information | Comment |
| --- | --- | --- | --- |
| result | M | swM.result |  |

### 5.3.7	Operation swFallback (M)

#### 5.3.7.1	Definition

This operation enables the IRPManager to initiate a SW fallback.

Information on Requirements Traceability:

| Referenced TS | Requirement label | Comment |
| --- | --- | --- |
| 3GPP TS 32.531 [6] | REQ_SWM_FUN_6 |  |

#### 5.3.7.2	Input parameters

| Parameter Name | Qualifier | Information type | Comment |
| --- | --- | --- | --- |
| filter | M | swM.filter | To describe properties of the NEs to be selected. |

#### 5.3.7.3	Output parameters

| Parameter Name | Qualifier | Matching Information | Comment |
| --- | --- | --- | --- |
| nEList | M | swM.nEList | Each entry in the list contains: nEIdentification swFallbackStatus (values: fallbackSuccessful, fallbackUnsuccessful) |
| result | M | swM.result | - Success, - Partly successful – swFallbackStatus is fallbackUnsuccessful for at least one NE and fallbackSuccessful for at least one other NE - Failure Empty NEList and Result=Success means: No NEs fulfilling filter were found. |

### 5.3.8	Operation terminateSwMProcess (M)

#### 5.3.8.1	Definition

This operation allows the IRPManager to terminate a SW management process which is currently ongoing.

Information on Requirements Traceability:

| Referenced TS | Requirement label | Comment |
| --- | --- | --- |
| 3GPP TS 32.531 [6] | REQ_ASWM_FUN_6 |  |

#### 5.3.8.2	Input parameters

| Parameter Name | Qualifier | Information type | Comment |
| --- | --- | --- | --- |
| id | M | swM.id | Identifier of swMprocess . |

#### 5.3.8.3	Output parameters

| Parameter Name | Qualifier | Matching Information | Comment |
| --- | --- | --- | --- |
| result | M | swM.result |  |

## 5.4	SwMIRPOperations_2 Interface (O)

### 5.4.1	Operation changeSwMProfile (O)

#### 5.4.1.1	Definition

This operation allows the IRPManager to change an instance of SwMProfile.

A change in a profile which was already used at the start of an swMProcess does not affect that swMProcess (which is run to its completion according to the former version of the profile.

Information on Requirements Traceability:

| Referenced TS | Requirement label | Comment |
| --- | --- | --- |
| 3GPP TS 32.531 [6] | REQ_ASWM_FUN_2 |  |

#### 5.4.1.2	Input parameters

| Parameter Name | Qualifier | Information type | Comment |
| --- | --- | --- | --- |
| id | M | swM.id | Identifier of swMprofile |
| nEInformation | M | swM.nEInformation | See 4.5 |
| swVersionToBeInstalled | M | swM.swVersionToBeInstalled | See 4.5 |
| stepsAndSelectedStopPointList | M | swM.stepsAndSelectedStopPointList | See 4.5 |
| selectedFinalAdministrativeState | M | swM.selectedFinalAdministrativeState | See 4.5 |

#### 5.4.1.3	Output parameters

| Parameter Name | Qualifier | Matching Information | Comment |
| --- | --- | --- | --- |
| result | M | ENUM { success, failure, nEInformationIntersection, notAllowedBecauseOfOngoingSwmActivity } | If result = success or failure , then parameter id may be absent or contain the id of the changed swMprofile. If result = nEInformationIntersection, then parameter conflictingProfileId contains the id of a SwMProfile ”A” whose nEInformation would intersect with the proposed nEInformation for the SwMProfile”B” = input parameter id. SwMProfile”B” will not be changed in this case. If result = notAllowedBecauseOfOngoingSwmActivity, (which means that the operation is rejected because another SWM activity is ongoing for at least one NE covered by input parameter neInformation), then parameter conflictingProfileId is absent. |
| versionNumber | M | SwMProfile.versionNumber | See 4.5. This parameter has value 0 when result <> success. |
| conflictingProfileId | C *) | SwMProfile.id | See definition of result above. |

Editor Note: whether this parameter needs to be conditional or mandatory needs further discussion. The condition: result_is_nEInformationIntersection

#### 5.4.1.4	Constraints

| Name | Definition |
| --- | --- |
| result_is_nEInformationIntersection | result is equal to nEInformationIntersection |

## 5.5	SwMIRPNotifications_1 Interface (M)

### 5.5.1	Notification notifySwMProfileCreation (M)

#### 5.5.1.1	Definition

This notification conveys information about the creation of an instance of IOC swMProfile.

#### 5.5.1.2	Input parameters

| Parameter Name | Qualifiers | Matching Information | Comment |
| --- | --- | --- | --- |
| id | M,Y | swM.id | Identifier of swMprofile |
| versionNumber | M, Y | swM.versionNumber | See 4.5 |
| nEInformation | M,Y | swM.nEInformation | See 4.5 |
| swVersionToBeInstalled | M,Y | swM.swVersionToBeInstalled | See 4.5 |
| stepsAndSelectedStopPointList | M,N | swM.stepsAndSelectedStopPointList | See 4.5 |
| selectedFinalAdministrativeState | M,N | swM.selectedFinalAdministrativeState | See 4.5 |

### 5.5.2	Notification notifySwMProfileDeletion (M)

#### 5.5.2.1	Definition

This notification conveys information about the deletion of an instance of IOC swMProfile.

Information on Requirements Traceability:

| Referenced TS | Requirement label | Comment |
| --- | --- | --- |
| 3GPP TS 32.531 [6] | REQ_ASWM_FUN_2 |  |

#### 5.5.2.2	Input parameters

| Parameter Name | Qualifiers | Matching Information | Comment |
| --- | --- | --- | --- |
| id | M,Y | swM.id | Identifier of swMprofile |

### 5.5.3	Notification notifySwMProcessCreation (M)

#### 5.5.3.1	Definition

This notification conveys information about the creation of an instance of IOC swMProcess.

Information on Requirements Traceability:

| Referenced TS | Requirement label | Comment |
| --- | --- | --- |
| 3GPP TS 32.531 [6] | REQ_ASWM_FUN_2 |  |

#### 5.5.3.2	Input parameters

| Parameter Name | Qualifiers | Matching Information | Comment |
| --- | --- | --- | --- |
| id | M,Y | swM.id | Identifier of swMprocess |
| nEIdentification | M,Y | swM.nEIdentification | see 4.5 |
| profileId | M,N | swM.profileId | see 4.5 |
| matchingNEInformation | M,N | swM.matchingNEInformation | see 4.5 |
| stepInfoList | M,N | swM.stepInfoList | see 4.5 |

### 5.5.4	Notification notifySwMProcessStage (M)

#### 5.5.4.1	Definition

This notification conveys information about the stage of an instance of IOC swMProcess that has been completed or at which that process has been stopped (based on pre-set stop points).

Information on Requirements Traceability:

| Referenced TS | Requirement label | Comment |
| --- | --- | --- |
| 3GPP TS 32.531 [6] | REQ_ASWM_FUN_5 |  |

#### 5.5.4.2	Input parameters

| Parameter Name | Qualifiers | Matching Information | Comment |
| --- | --- | --- | --- |
| id | M,Y | swM.id | Identifier of swMprocess |
| stepInfoList | M,N | swM.stepInfoList | see 4.5 |

### 5.5.5	Notification notifySwMProcessDeletion (M)

#### 5.5.5.1	Definition

This notification conveys information about the deletion of an instance of IOC swMProcess

IRPAgent shall also send out this notification in case of a process termination caused by an exception, for example IRP Agent terminates the process because it had to wait too long after a suspend operation.

Information on Requirements Traceability:

| Referenced TS | Requirement label | Comment |
| --- | --- | --- |
| 3GPP TS 32.531 [6] | REQ_ASWM_FUN_2 |  |

#### 5.5.5.2	Input parameters

| Parameter Name | Qualifiers | Matching Information | Comment |
| --- | --- | --- | --- |
| id | M,Y | swM.id | Identifier of swMprocess |
| triggerForDeletion | M, Y | swM.triggerForDeletion | This parameter describes what triggered the deletion of the swMprocess instance: triggerForDeletion: irpAgentTermination, irpManagerTermination, automatedSWMSuccesfullyConcluded |
| additionalInformation | O, N | swM.additionalInformation |  |

### 5.5.6	Notification notifyNewSwAvailability (M)

#### 5.5.6.1	Definition

This notification conveys information about the availability of new SW.

Information on Requirements Traceability:

| Referenced TS | Requirement label | Comment |
| --- | --- | --- |
| 3GPP TS 32.531 [6] | REQ_SWM_FUN_3 |  |

#### 5.5.6.2	Input parameters

| Parameter Name | Qualifiers | Matching Information | Comment |
| --- | --- | --- | --- |
| nEandSWversion | M,Y | swM.NEandSWversion | Informs about new available SW, SW version and NE / NE version (types) for which it is valid |

## 5.6	SwMIRPNotifications_2 Interface (O)

### 5.6.1	Notification notifySwMProfileChange (C/O)

#### 5.6.1.1	Definition

This notification conveys information about a change of an instance of IOC swMProfile.

Information on Requirements Traceability:

| Referenced TS | Requirement label | Comment |
| --- | --- | --- |
| 3GPP TS 32.531 [6] | REQ_ASWM_FUN_2 |  |

#### 5.6.1.2	Input parameters

| Parameter Name | Qualifiers | Matching Information | Comment |
| --- | --- | --- | --- |
| id | M,Y | swM.id | Identifier of swMprofile |
| versionNumber | M,Y | swM.versionNumber | See 4.5 |
| nEInformation | M,Y | swM.nEInformation | See 4.5 |
| swVersionToBeInstalled | M,N | swM.swVersionToBeInstalled | See 4.5 |
| stepsAndSelectedStopPointList | M,N | swM.stepsAndSelectedStopPointList | See 4.5 |
| selectedFinalAdministrativeState | M | swM.selectedFinalAdministrativeState | See 4.5 |

## 5.7	SwMIRPOperations_3 Interface (M)

Editor’s Note: Whether non-automatic software management interface is optional or mandatory need further study.

### 5.7.1	Operation downloadNESw (M)

#### 5.7.1.1	Definition

This operation allows IRPManager to request an IRPAgent to download network element software entities from a specified location. IRPManager provides a unique reference where IRPAgent can download NE software from.

Note: The file transfer may not happen over Itf-N and the details on how to transfer file from IRPAgent to NE(s) is vendor specific and outside the scope of this specification.

Information on Requirements Traceability:

| Referenced TS | Requirement label | Comment |
| --- | --- | --- |
| 3GPP TS 32.531 [6] | REQ_NASWM_FUN_1 |  |
| 3GPP TS 32.531 [6] | REQ_NASWM_FUN_2 |  |

#### 5.7.1.2	Input parameters

| Parameter Name | Qualifier | Information type | Comment |
| --- | --- | --- | --- |
| swToBeDownloaded | M | List of STRUCT < swLocation, swFileSize, swFileCompression, swFileFormat > | These attributes represent information about the NE software which will be downloaded by IRPAgent. swLocation: Denotes a unique location of software. This attribute includes the name of the software or a software version. swFileSize: It identifies the size of the file. Its value is positive Integer (the unit is byte). It is optional to fill in this attribute value. swFileCompression: It identifies the name of the compression algorithm used for the file. An empty fileCompression means that there is no compression on the file. Choice of compression algorithm is vendor-specific but is encouraged to use industrial standard algorithm such as GZIP. It is optional to fill in this attribute value. swFileFormat: It identifies the encoding technique used by the file. It is optional to fill in this attribute value |
| neIdentifier | M | Distinguished Name (DN) | Identifies the destination where the software can be downloaded and can include network element, managed element or managed functionality etc. The information is represented using a full Distinguished Name according to 3GPP TS 32.300. |

#### 5.7.1.3	Output parameters

| Parameter Name | Qualifier | Matching Information | Comment |
| --- | --- | --- | --- |
| downloadProcessId | M | Integer | An Identifier generated by IRPAgent upon receiving a non-automated software management request from IRPManager. In this case, it identifies the NE software download operation request. |
| result | M | ENUM { requestAccepted, requestFailed, notAllowedBecauseOfOngoingSwmActivity } | Indicates whether SwMIRP has accepted or rejected the download request. It can have any one of the three possible values: - “requestAccepted” which means that IRPAgent would perform the NE software download operation - “requestFailed” which means that IRPAgent has failed to initiate the NE software download operation. Specific error condition can be captured in reason field - notAllowedBecauseOfOngoingSwmActivity which means that the operation is rejected because another SWM activity is ongoing for the requested NE.. |
| reason | O | String | To capture detailed error reason. The field is empty when there is no error. |

#### 5.7.1.4	Pre condition

| Assertion Name | Definition |
| --- | --- |
| swDownloadable | NE software is available which IRPAgent can download |

#### 5.7.1.5	Post-condition

| Assertion Name | Definition |
| --- | --- |
| swDownloadInProgress | The SwMIRP has accepted the download request to perform the requested operation. |
| swAvailable | This is the final state when downloadNESw operation is complete and notifyDownloadNESwStatusChanged has been generated. |

#### 5.7.1.6	Exceptions

| Exception Name | Definition |
| --- | --- |
| operationFailed | Condition: Pre-condition is false or post-condition is false. Returned Information: The output parameter status. Exit state: Entry state. |
| resourceLimitation | Condition: Operation not performed due to resource limitation. Returned Information: The output parameter status. Exit state: Entry state. |

### 5.7.2	Operation activateNESw (M)

#### 5.7.2.1	Definition

This operation allows IRPManager to activate network element software entity which has been previously downloaded or installed on the request of  IRPManager. This operation may be service affecting.

Note: activateNESw can be triggered through automatic or manual ways.

Information on Requirements Traceability:

| Referenced TS | Requirement label | Comment |
| --- | --- | --- |
| 3GPP TS 32.531 [6] | REQ_NASWM_FUN_5 |  |

#### 5.7.2.2	Input parameters

| Parameter Name | Qualifier | Information type | Comment |
| --- | --- | --- | --- |
| swVersionToBeActivated | M | swVersion | swVersion denotes the software version which would be activated. The details on how to activate a software version is vendor specific. |
| neIdentifier | M | Distinguished Name (DN) | Identifies the destination where software has to be activated. This is a full Distinguished Name according to 3GPP TS 32.300 |

#### 5.7.2.3	Output parameters

| Parameter Name | Qualifier | Matching Information | Comment |
| --- | --- | --- | --- |
| activateProcessId | M | Integer | An Identifier is generated by IRPAgent upon receiving a non-automated software management request from IRPManager. In this case, it identifies the NE software activation operation request. |
| result | M | ENUM { requestAccepted, requestFailed, notAllowedBecauseOfOngoingSwmActivity } | Indicates whether SwMIRP has accepted or rejected the NE software activation request. It can have any one of the three possible values: - “requestAccepted” which means that IRPAgent would perform the NE software activation operation - “requestFailed” which means that IRPAgent has failed to initiate the NE software activation operation. Specific error condition can be captured in reason field - notAllowedBecauseOfOngoingSwmActivity which means that the operation is rejected because another SWM activity is ongoing for the requested NE.. |
| reason | O | String | To capture specific error conditions. The field is empty when there is no error. |

#### 5.7.2.4 Pre condition

swAvailable or swInstalled

| Assertion Name | Definition |
| --- | --- |
| swAvailable | The NE software has been successfully downloaded |
| swInstalled | The NE software has been installed |

#### 5.7.2.5 Post-condition

| Assertion Name | Definition |
| --- | --- |
| swActivationInProgress | The SwMIRP has accepted the request to perform the requested activation operation. |
| swActivated | This is the final state when activateNESw operation is complete and notifyActivateNESwStatusChanged has been generated |

#### 5.7.2.6 Exceptions

| Exception Name | Definition |
| --- | --- |
| operationFailed | Condition: Pre-condition is false or post-condition is false. Returned Information: The output parameter status. Exit state: Entry state. |
| resourceLimitation | Condition: Operation not performed due to resource limitation. Returned Information: The output parameter status. Exit state: Entry state. |

## 5.8	SwMIRPOperations_4 Interface (O)

### 5.8.1	Operation installNESw (O)

#### 5.8.1.1	Definition

This operation allows IRPManager to initiate installation of NE software entity which has been previously downloaded on the request of IRPManager. Installation may also be initiated from a remote location.

Information on Requirements Traceability:

| Referenced TS | Requirement label | Comment |
| --- | --- | --- |
| 3GPP TS 32.531 [6] | REQ_NASWM_FUN_3 |  |
| 3GPP TS 32.531 [6] | REQ_NASWM_FUN_4 |  |

#### 5.8.1.2	Input parameters

| Parameter Name | Qualifier | Information type | Comment |
| --- | --- | --- | --- |
| swTobeInstalled | M | swLocation | swLocation denotes a unique location (local or remote) of software which can be a directory path or a URL and includes 1) the name of software or 2) a software version |
| neIdentifier | M | Distinguished Name (DN) | Identifies the destination where the NE software needs to be installed. This is a full Distinguished Name according to 3GPP TS 32.300 |

#### 5.8.1.3	Output parameters

| Parameter Name | Qualifier | Matching Information | Comment |
| --- | --- | --- | --- |
| installProcessId | M | Integer | An Identifier generated by IRPAgent upon receiving a non-automated software management request from IRPManager. In this case, it identifies the NE software installation operation request. |
| result | M | ENUM { requestAccepted, requestFailed, notAllowedBecauseOfOngoingSwmActivity } | Indicates whether SwMIRP has accepted or rejected the installation request. It can have any one of the twhreeo possible values: - “requestAccepted” which means that IRPAgent uld perform the NE software installation operation - “requestFailed” which means that IRPAgent has failed to initiate the NE software installation operation. Specific error condition can be captured in reason field - notAllowedBecauseOfOngoingSwmActivity which means that the operation is rejected because automatic SWM is ongoing for the requested NE.. |
| reason | O | String | To capture detailed error conditions. The field is empty when there is no error. |

#### 5.8.1.4	Pre condition

| Assertion Name | Definition |
| --- | --- |
| swAvailable | NE software is available |

#### 5.8.1.5	Post-condition

| Assertion Name | Definition |
| --- | --- |
| swInstallationInProgress | The SwMIRP has successfully accepted the request to perform the requested installation operation |
| swInstalled | This is the final state when installNESw operation is complete and notifyInstallNESwStatusChanged has been generated |

#### 5.8.1.6	Exceptions

| Exception Name | Definition |
| --- | --- |
| operationFailed | Condition: Pre-condition is false or post-condition is false. Returned Information: The output parameter status. Exit state: Entry state. |
| resourceLimitation | Condition: Operation not performed due to resource limitation. Returned Information: The output parameter status. Exit state: Entry state. |
| swNotAvailable | Condition: NE software is not available. Returned Information: The output parameter status. Exit state: Entry state. |

## 5.9	SwMIRPNotifications_3 Interface (M)

### 5.9.1	Notification notifyDownloadNESwStatusChanged (M)

#### 5.9.1.1	Definition

This notification, generated by IRPAgent conveys information about the status of the downloadNESw operation.

Information on Requirements Traceability:

| Referenced TS | Requirement label | Comment |
| --- | --- | --- |
| 3GPP TS 32.531 [6] | REQ_NASWM_FUN_1 |  |

#### 5.9.1.2	Input parameters

| Parameter Name | Qualifier | Matching Information | Comment |
| --- | --- | --- | --- |
| objectClass | M, Y | SwMManagedEntity.objectClass | Represents the network element object class generating this event. Also refer to Notification header [9]. |
| objectInstance | M,Y | SwMManagedEntity.objectInstance | - Represents the network element instance generating the event. Also refer to Notification header [9]. |
| notificationId | O,N | -- | - Refer to Notification header [9]. |
| eventTime | M,Y | -- | - Refer to Notification header [9]. |
| systemDN | C,Y | -- | - Refer to Notification header [9]. |
| notificationType | M,Y | “notifyDownloadNESwStatusChanged” |  |
| downloadProcessId | M, Y | Integer | To allow IRPManager correlate this notification with the downloadNESw operation request. Also refer to section 5.7.1 |
| downloadOperationStatus | M,Y | enum { NE_SWDOWNLOAD_SUCCESSFUL, NE_SWDOWNLOAD_FAILED, NE_SWDOWNLOAD_PARTIALLY_SUCCESSFUL } | Provides information on the status of downloadNESw operation for the network element involved. Note: When only one software entity has to be downloaded, downloadOperationStatus can be either NE_SWDOWNLOAD_SUCCESSFUL or NE_SWDOWNLOAD_FAILED. |
| downloadedNESwInfo | O,N | LIST<DownloadedNESw> | Information on where the software or version got downloaded on the NE |
| failedSwInfo | O,N | LIST<FailedSw, FailureReason> | Information on software not abled to be downloaded and the corresponding failure reason. |

#### 5.9.1.3	Triggering Event

##### 5.9.1.3.1	From State

neSwDownloadInProgress

| Assertion Name | Definition |
| --- | --- |
| neSwDownloadInProgress | IRPAgent has accepted the request to download software and downloadProcessId is available |

##### 5.9.1.3.2	To State

neSwDownloadSuccessful or neSwDownloadFailed or neSwDownloadPartiallySuccessful.

| Assertion Name | Definition |
| --- | --- |
| neSwDownloadSuccessful | Software has been successfully downloaded. When multiple software have to be downloaded, it means that all software entities have been downloaded successfully |
| neSwDownloadFailed | Software has not been downloaded. When multiple software have to be downloaded, it means that no software has been downloaded |
| neSwDownloadPartiallySuccessful | At least one of the software has not been downloaded (hence can be used only when multiple software need to be downloaded) |

#### 5.9.1.4	Constraints

None

### 5.9.2	Notification notifyActivateNESwStatusChanged (M)

#### 5.9.2.1	Definition

This notification, generated by IRPAgent conveys information about the status of the activateNESw operation.

Information on Requirements Traceability:

| Referenced TS | Requirement label | Comment |
| --- | --- | --- |
| 3GPP TS 32.531 [6] | REQ_NASWM_FUN_5 |  |

#### 5.9.2.2	Input parameters

| Parameter Name | Qualifier | Matching Information | Comment |
| --- | --- | --- | --- |
| objectClass | M, Y | SwMManagedEntity.objectClass | Represents the network element object class generating this event. Also refer to Notification header [9]. |
| objectInstance | M, Y | SwMManagedEntity.objectInstance | Represents the network element instance generating the event. Also refer to Notification header [9]. |
| notificationId | O,N | -- | Refer to Notification header [9]. |
| eventTime | M,Y | -- | Refer to Notification header [9]. |
| systemDN | C,Y | -- | Refer to Notification header [9]. |
| notificationType | M,Y | “notifyActivateNESwStatusChanged” |  |
| activateProcessId | M, Y | Integer | To allow IRPManager correlate this notification with the activateNESw operation request. Also Refer to section 5.7.2 |
| activateOperationStatus | M,Y | ENUM{ NE_SWACTIVATION_SUCCESSFUL, NE_SWACTIVATION_FAILED, NE_SWACTIVATION_PARTIALLY_SUCCESSFUL } | Provides information on the status of activateNESw operation for the network element involved. When the activation could get completed to only a certain extent, partial success may be used. |
| swVersionActivated | O, Y | swVersion | The software version which has been activated. If the sotware activation is not successful, it would be NULL. |
| failureReason | O,N | String | The error reason when the activateNESw operation is not successful. |

#### 5.9.2.3	Triggering Event

##### 5.9.2.3.1	From State

neSwActivationInProgress

| Assertion Name | Definition |
| --- | --- |
| neSwActivationInProgress | IRPAgent has accepted the request to activate software and activateProcessId is available |

##### 5.9.2.3.2	To State

neSwActivationSuccessful or neSwActivationFailed or neSwActivationPartiallySuccessful.

| Assertion Name | Definition |
| --- | --- |
| neSwActivationSuccessful | Software has been successfully activated. |
| neSwActivationFailed | Software has not been activated. |
| neSwActivationPartiallySuccessful | When activation can be completed only to a certain extent, partial success may be used. |

#### 5.9.2.4	Constraints

None

## 5.10	SwMIRPNotifications_4 Interface (O)

### 5.10.1	Notification notifyInstallNESwStatusChanged (O)

#### 5.10.1.1	Definition

This notification, generated by IRPAgent conveys information about the status of the installNESw operation.

Information on Requirements Traceability:

| Referenced TS | Requirement label | Comment |
| --- | --- | --- |
| 3GPP TS 32.531 [6] | REQ_NASWM_FUN_3 |  |

#### 5.10.1.2	Input parameters

| Parameter Name | Qualifier | Matching Information | Comment |
| --- | --- | --- | --- |
| objectClass | M, Y | SwMManagedEntity.objectClass | Represents the network element object class generating this event. Also refer to Notification header [9]. |
| objectInstance | M, Y | SwMManagedEntity.objectInstance | Represents the network element instance generating the event. Also refer to Notification header [9] |
| notificationId | O,N | -- | Refer to Notification header [9]. |
| eventTime | M,Y | -- | Refer to Notification header [9]. |
| systemDN | C,Y | -- | Refer to Notification header [9]. |
| notificationType | M,Y | “notifyInstallNESwStatusChanged” |  |
| installProcessId | M, Y | Integer | To allow IRPManager correlate this notification with the installNESw operation request. Also refer to section 5.8.1 |
| installOperationStatus | M,Y | ENUM { NE_SWINSTALLATION_SUCCESSFUL, NE_SWINSTALLATION_FAILED, NE_SWINSTALLATION_PARTIALLY_SUCCESSFUL } | Provides information on the status of installNESw operation for the network element involved. |
| installedNESwInfo | O,N | LIST<InstalledNESw> | Information on where the software or version got installed on the NE |
| failedSwInfo | O,N | LIST<FailedSw, FailureReason> | It provides information on the software which failed in installation |

#### 5.10.1.3	Triggering Event

##### 5.10.1.3.1	From State

neSwInstallationInProgress

| Assertion Name | Definition |
| --- | --- |
| neSwInstallationInProgress | IRPAgent has accepted the request to install software and installProcessId is available |

##### 5.10.1.3.2	To State

neSwInstallationSuccessful or neSwInstallationFailed or neSwInstallationPartiallySuccessful.

| Assertion Name | Definition |
| --- | --- |
| neSwInstallationSuccessful | Software has been successfully installed |
| neSwInstallationFailed | Software has not been installed |
| neSwInstallationPartiallySuccessful | At least one of software has not been installed |

#### 5.10.1.4	Constraints

None.

Annex A (informative): Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New |
| 2008-12 | SP-42 | SP-080717 |  |  | Submitted to SA#42 for information and approval | 1.0.0 | 8.0.0 |
| 2009-06 | SP-44 | SP-090408 | 001 | -- | Correction of naming errors | 8.0.0 | 8.1.0 |
| 2009-06 | SP-44 | SP-090408 | 002 | -- | Add missing start step parameter for resume operation | 8.0.0 | 8.1.0 |
| 2009-06 | SP-44 | SP-090290 | 003 | -- | To add a Non-Automated Software Management operation, downloadNESw | 8.1.0 | 9.0.0 |
| 2009-06 | SP-44 | SP-090290 | 004 | -- | To add a Non-Automated Software Management operation, activateNESw | 8.1.0 | 9.0.0 |
| 2009-06 | SP-44 | SP-090290 | 005 | -- | To add a Non-Automated Software Management operation, installNESw | 8.1.0 | 9.0.0 |
| 2009-06 | SP-44 | SP-090290 | 006 | -- | Editorial cleanup and correction of various qualifiers for TS 32.532 | 8.1.0 | 9.0.0 |
| 2009-06 | SP-44 | SP-090290 | 007 | -- | To update SWM Class diagram to include Non-Automated Software Management interfaces | 8.1.0 | 9.0.0 |
| 2009-09 | SP-45 | SP-090627 | 008 | - | Adding missing Editor’s Notes on Automatic software management | 9.0.0 | 9.1.0 |
| 2009-09 | SP-45 | SP-090627 | 009 | - | Remove duplication of SWM functionalities | 9.0.0 | 9.1.0 |
| 2009-09 | SP-45 | SP-090627 | 010 | - | To rephrase the definitions of installNESw and activateNESw operations | 9.0.0 | 9.1.0 |
| 2009-09 | SP-45 | SP-090627 | 011 | - | Addition of a new NASWM notification notifyInstallNESwStatusChanged | 9.0.0 | 9.1.0 |
| 2009-09 | SP-45 | SP-090627 | 012 | - | Addition of a new NASWM notification notifyActivateNESwStatusChanged | 9.0.0 | 9.1.0 |
| 2009-09 | SP-45 | SP-090627 | 013 | - | Adding error reason | 9.0.0 | 9.1.0 |
| 2009-09 | SP-45 | SP-090627 | 014 | - | Addition of a new NASWM notification notifyDownloadNESwStatusChanged | 9.0.0 | 9.1.0 |
| 2009-09 | SP-45 | SP-090627 | 015 | - | To update Software Management Class Diagram to include NASWM notifications | 9.0.0 | 9.1.0 |
| 2009-12 | SP-46 | SP-090719 | 016 | -- | To move editor notes related to NASWM operations | 9.1.0 | 9.2.0 |
| 2009-12 | SP-46 | SP-090719 | 017 | -- | Extend requirements traceability table for IOC swmProfile | 9.1.0 | 9.2.0 |
| 2009-12 | SP-46 | SP-090719 | 018 | -- | To update SwManagement IRP Class Diagram | 9.1.0 | 9.2.0 |