3GPP TS 32.632 V9.1.0 (2010-03)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Telecommunication management;

Configuration Management (CM);

Core network resources Integration Reference Point (IRP):

Network Resource Model (NRM)

(Release 9)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. 
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.	 
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

UMTS, management

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2010, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI currently being registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	8

Introduction	8

1	Scope	9

2	References	9

3	Definitions and abbreviations	10

3.1	Definitions	10

3.2	Abbreviations	11

4	System overview	12

4.1	Compliance rules	12

5	Modelling approach	12

6	Information Object Classes	13

6.1	Imported information entities and local labels	13

6.2	Class diagram	13

6.2.1	Attributes and relationships	13

6.2.2	Inheritance	18

6.3	Information object class definitions	21

6.3.1	MscServerFunction	21

6.3.1.1	Definition	21

6.3.1.2	Attributes	21

6.3.1.3	Notifications	21

6.3.2	HlrFunction	21

6.3.2.1	Definition	21

6.3.2.2	Attributes	21

6.3.2.3	Notifications	21

6.3.3	VlrFunction	22

6.3.3.1	Definition	22

6.3.3.2	Attributes	22

6.3.3.3	Notifications	22

6.3.4	AucFunction	22

6.3.4.1	Definition	22

6.3.4.2	Attributes	22

6.3.4.3	Notifications	22

6.3.5	EirFunction	22

6.3.5.1	Definition	22

6.3.5.2	Attributes	22

6.3.5.3	Notifications	22

6.3.6	SmsIwmscFunction	23

6.3.6.1	Definition	23

6.3.6.2	Attributes	23

6.3.6.3	Notifications	23

6.3.7	SmsGmscFunction	23

6.3.7.1	Definition	23

6.3.7.2	Attributes	23

6.3.7.3	Notifications	23

6.3.8	GmscFunction	23

6.3.8.1	Definition	23

6.3.8.2	Attributes	23

6.3.8.3	Notifications	23

6.3.9	SgsnFunction	24

6.3.9.1	Definitions	24

6.3.9.2	Attributes	24

6.3.9.3	Notifications	24

6.3.10	GgsnFunction	24

6.3.10.1	Definitions	24

6.3.10.2	Attributes	24

6.3.10.3	Notifications	24

6.3.11	BgFunction	25

6.3.11.1	Definitions	25

6.3.11.2	Attributes	25

6.3.11.3	Notifications	25

6.3.12	SmlcFunction	25

6.3.12.1	Definitions	25

6.3.12.2	Attributes	25

6.3.12.3	Notifications	25

6.3.13	GmlcFunction	25

6.3.13.1	Definitions	25

6.3.13.2	Attributes	25

6.3.13.3	Notifications	25

6.3.14	ScfFunction	26

6.3.14.1	Definitions	26

6.3.14.2	Attributes	26

6.3.14.3	Notifications	26

6.3.15	SrfFunction	26

6.3.15.1	Definitions	26

6.3.15.2	Attributes	26

6.3.15.3	Notifications	26

6.3.16	CbcFunction	26

6.3.16.1	Definitions	26

6.3.16.2	Attributes	26

6.3.16.3	Notifications	26

6.3.17	CgfFunction	27

6.3.17.1	Definitions	27

6.3.17.2	Attributes	27

6.3.17.3	Notifications	27

6.3.18	GmscServerFunction	27

6.3.18.1	Definitions	27

6.3.18.2	Attributes	27

6.3.18.3	Notifications	27

6.3.19	IwfFunction	27

6.3.19.1	Attributes	27

6.3.19.2	Attributes	27

6.3.19.3	Notifications	27

6.3.20	MnpSrfFunction	28

6.3.20.1	Definitions	28

6.3.20.2	Attributes	28

6.3.20.3	Notifications	28

6.3.21	NpdbFunction	28

6.3.21.1	Definitions	28

6.3.21.2	Attributes	28

6.3.21.3	Notifications	28

6.3.22	SgwFunction	28

6.3.22.1	Definitions	28

6.3.22.2	Attributes	28

6.3.22.3	Notifications	28

6.3.23	SsfFunction	29

6.3.23.1	Definitions	29

6.3.23.2	Attributes	29

6.3.23.3	Notifications	29

6.3.24	BsFunction	29

6.3.24.1	Definitions	29

6.3.24.2	Attributes	29

6.3.24.3	Notifications	29

6.3.25	IucsLink	30

6.3.25.1	Definitions	30

6.3.25.2	Attributes	30

6.3.25.3	Attribute constraints	30

6.3.25.4	Notifications	30

6.3.26	IupsLink	31

6.3.26.1	Definitions	31

6.3.26.2	Attributes	31

6.3.26.3	Attribute constraints	31

6.3.26.4	Notifications	31

6.3.27	IubcLink	31

6.3.27.1	Definitions	31

6.3.27.2	Attributes	31

6.3.27.3	Notifications	31

6.3.28	ALink	32

6.3.28.1	Definitions	32

6.3.28.2	Attributes	32

6.3.28.3	Notifications	32

6.3.29	GbLink	32

6.3.29.1	Definitions	32

6.3.29.2	Attributes	32

6.3.29.3	Notifications	32

6.3.30	CsMgwFunction	32

6.3.30.1	Definitions	32

6.3.30.2	Attributes	32

6.3.30.3	Notifications	32

6.3.31	Void	33

6.3.32	Void	33

6.3.33	Void	33

6.3.34	Void	33

6.3.35	BmScFunction	33

6.3.35.1	Definitions	33

6.3.35.2	Attributes	33

6.3.35.3	Notifications	33

6.3.36	Link_BmSc_Ggsn	33

6.3.36.1	Definition	33

6.3.37	Link_Ggsn_Sgsn	33

6.3.37.1	Definition	33

6.3.38	CircuitEndPointSubgroup	33

6.3.38.1	Definitions	33

6.3.38.2	Attributes	33

6.3.38.3	Notifications	34

6.4	Information relationship definitions	35

6.4.1	AssociatedWith1 (M)	35

6.4.1.1	Definition	35

6.4.1.2	Roles	35

6.4.1.3	Constraints	35

6.4.2	AssociatedWith2 (M)	36

6.4.2.1	Definition	36

6.4.2.2	Roles	36

6.4.2.3	Constraints	36

6.4.3	AssociatedWith3 (M)	36

6.4.3.1	Definition	36

6.4.3.2	Roles	36

6.4.3.3	Constraints	37

6.4.4	AssociatedWith4 (M)	37

6.4.4.1	Definition	37

6.4.4.2	Roles	37

6.4.4.3	Constraints	37

6.4.5	AssociatedWith5 (M)	37

6.4.5.1	Definition	37

6.4.5.2	Roles	37

6.4.5.3	Constraints	38

6.4.6	ConnectedTo1 (M)	38

6.4.6.1	Definition	38

6.4.6.2	Roles	38

6.4.6.3	Constraints	38

6.4.7	ConnectedTo2 (M)	38

6.4.7.1	Definition	38

6.4.7.2	Roles	38

6.4.7.3	Constraints	38

6.4.8	ConnectedTo3 (M)	39

6.4.8.1	Definition	39

6.4.8.2	Roles	39

6.4.8.3	Constraints	39

6.4.9	ConnectedTo4 (M)	39

6.4.9.1	Definition	39

6.4.9.2	Roles	39

6.4.9.3	Constraints	39

6.4.10	ConnectedTo5 (M)	40

6.4.10.1	Definition	40

6.4.10.2	Roles	40

6.4.10.3	Constraints	40

6.4.11	ConnectedTo6 (M)	40

6.4.11.1	Definition	40

6.4.11.2	Roles	40

6.4.11.3	Constraints	40

6.4.12	ConnectedTo7 (M)	41

6.4.12.1	Definition	41

6.4.12.2	Roles	41

6.4.12.3	Constraints	41

6.4.13	ConnectedTo8 (M)	41

6.4.13.1	Definition	41

6.4.13.2	Roles	41

6.4.13.3	Constraints	41

6.4.14	ConnectedTo9 (M)	42

6.4.14.1	Definition	42

6.4.14.2	Roles	42

6.4.14.3	Constraints	42

6.4.15	ConnectedTo10 (M)	42

6.4.15.1	Definition	42

6.4.15.2	Roles	42

6.4.15.3	Constraints	42

6.4.16	ConnectedTo11 (M)	43

6.4.16.1	Definition	43

6.4.16.2	Roles	43

6.4.16.3	Constraints	43

6.4.17	ConnectedTo12 (M)	43

6.4.17.1	Definition	43

6.4.17.2	Roles	43

6.4.17.3	Constraints	43

6.4.18	ConnectedTo13 (M)	44

6.4.18.1	Definition	44

6.4.18.2	Roles	44

6.4.18.3	Constraints	44

6.4.19	ConnectedTo14 (M)	44

6.4.19.1	Definition	44

6.4.19.2	Roles	44

6.4.19.3	Constraints	44

6.4.20	ConnectedTo15 (M)	45

6.4.20.1	Definition	45

6.4.20.2	Roles	45

6.4.20.3	Constraints	45

6.4.21	ConnectedTo16 (M)	45

6.4.21.1	Definition	45

6.4.21.2	Roles	45

6.4.21.3	Constraints	45

6.5	Information attribute definitions	46

6.5.1	Definition and legal values	46

6.5.2	Constraints	47

6.6	Particular information configurations	48

6.7	Common notifications	48

Annex A (informative):	Change history	49

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

The present document is part of a TS-family covering the 3rd Generation Partnership Project Technical Specification Group Services and System Aspects, Telecommunication management; as identified below:

32.631:	"Configuration Management (CM); Core network resources Integration Reference Point (IRP): Requirements".

32.632:	"Configuration Management (CM); Core network resources Integration Reference Point (IRP): Network Resource Model (NRM)".

32.633:	"Configuration Management (CM); Core network resources Integration Reference Point (IRP): Common Object Request Broker Architecture (CORBA) Solution Set (SS)".

32.635:	"Configuration Management (CM); Core network resources Integration Reference Point (IRP): Bulk CM eXtensible Markup Language (XML) file format definition".

Configuration Management (CM), in general, provides the operator with the ability to assure correct and effective operation of the 3G network as it evolves. CM actions have the objective to control and monitor the actual configuration on the Network Elements (NEs) and Network Resources (NRs), and they may be initiated by the operator or by functions in the Operations Systems (OSs) or NEs.

CM actions may be requested as part of an implementation programme (e.g. additions and deletions), as part of an optimization programme (e.g. modifications), and to maintain the overall Quality of Service (QoS). The CM actions are initiated either as single actions on single NEs of the 3G network, or as part of a complex procedure involving actions on many resources/objects in one or several NEs.

# 1	Scope

The present document is part of an Integration Reference Point (IRP) named "Core Network Resources IRP", through which an 'IRPAgent' (typically an Element Manager or Network Element) can communicate Configuration Management information to one or several 'IRPManagers' (typically Network Managers) concerning CN resources.

The present document specifies the protocol neutral Core Network Resources IRP: Network Resource Model. It reuses relevant parts of the generic NRM in 3GPP TS 32.622 [16], either by direct reuse or sub-classing, and in addition to that defines CN specific Information Object Classes.

Finally, in order to access the information defined by this NRM, an Interface IRP is needed, such as the Basic CM IRP 3GPP TS 32.60x [17]. However, which Interface IRP that is applicable is outside the scope of the present document.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies.  In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 32.101: "Telecommunication management; Principles and high level requirements".

[2]	3GPP TS 32.102: "Telecommunication management; Architecture".

[3]	3GPP TS 32.302: "Telecommunication management; Configuration Management (CM); Notification Integration Reference Point; Information Service (IS)".

[4] - [6]	Void.

[7]	ITU-T Recommendation X.710 (1991): "Common management information service definition for CCITT applications".

[8] - [10]	Void.

[11]	3GPP TS 32.111-2: "Telecommunication management; Fault Management; Part 2: Alarm Integration Reference Point: Information Service (IS)".

[12]	Void.

[13]	3GPP TS 32.300: "Telecommunication management; Configuration Management (CM); Name convention for Managed Objects".

[14]	3GPP TS 32.600: "Telecommunication management; Configuration Management (CM); Concept and high-level requirements".

[15]	3GPP TS 23.002: "Network architecture".

[16]	3GPP TS 32.622: "Telecommunication management; Configuration Management (CM); Generic network resources Integration Reference Point (IRP): Network Resource Model (NRM)".

[17]	3GPP TS 32.602: "Telecommunication management; Configuration Management (CM); Basic Configuration Management Integration Reference Point (IRP): Information Service (IS)".

[18]	3GPP TS 23.060: "General Packet Radio Service (GPRS) service description; Stage 2".

[19]	3GPP TS 23.003: "Numbering, addressing and identification".

[20]	3GPP TS 32.672: "Telecommunication Management; Configuration Management (CM); State Management Integration Reference Point (IRP): Information Service (IS)".

[21]	Void.

[22]	3GPP TS 32.642: "Telecommunication management; Configuration Management (CM); UTRAN network resources Integration Reference Point (IRP): Network Resource Model (NRM)".

[23]	3GPP TS 32.652: "Telecommunication management; Configuration Management (CM); GERAN network resources Integration Reference Point (IRP): Network Resource Model (NRM)".

[24]	ITU-T Rec. M.3100: "Generic Network Information Model" (7/95).

[25]	3GPP TS 32.772: "Telecommunication management; Home Node B Subsystem (HNS) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TS 32.101 [1], 3GPP TS 32.102 [2], 3GPP TS 32.600 [14] and the following apply:

Association: In general it is used to model relationships between Managed Objects. Associations can be implemented in several ways, such as:

(1)	name bindings;

(2)	reference attributes; and

(3)	association objects.

This IRP stipulates that containment associations shall be expressed through name bindings, but it does not stipulate the implementation for other types of associations as a general rule. These are specified as separate entities in the object models (UML diagrams).

Managed Element (ME): an instance of the Information Object Class ManagedElement defined in 3GPP TS 32.622 [16].

Managed Object (MO): in the context of the present document, a Managed Object (MO) is a software object that encapsulates the manageable characteristics and behaviour of a particular Network Resource. The MO is instance of a MO class defined in a MIM/NRM. This class, called Information Object Class (IOC) has attributes that provide information used to characterize the objects that belong to the class (the term "attribute" is taken from TMN and corresponds to a "property" according to CIM). Furthermore, the IOC can have operations that represent the behaviour relevant for that class (the term "operation" is taken from TMN and corresponds to a "method" according to CIM). The IOC may support the emission of notifications that provide information about an event occurrence within a network resource.

Management Information Model (MIM): also referred to as NRM - see the definition below.

Network Resource Model (NRM): a model representing the actual managed telecommunications network resources that a System is providing through the subject IRP
An NRM identifies and describes IOCs, their associations, attributes and operations. The NRM is also referred to as "MIM" (see above), which originates from the ITU-T TMN.

Node B: a logical node responsible for radio transmission/reception in one or more cells to/from the User Equipment
It terminates the Iub interface towards the RNC.

## 3.2	Abbreviations

For the purposes of the present document, the following abbreviations apply:

AUC	AUthentication Centre

AS	Application Server

BG	Border Gateway

BGCF	Breakout Gateway Control Function

BS	Billing System

CBC	Cell Broadcast Center

CGF	Charging Gateway Functionality

CN	Core Network

DN	Distinguished Name (see 3GPP TS 32.300 [13])

EIR	Equipment Identity Register

EM	Element Manager

FM	Fault Management

FNR	Flexible Number Register

GDMO	Guidelines for the Definition of Managed Objects

GGSN	Gateway GPRS Support Node

GMLC	Gateway Mobile Location Center

GMSC Server	Gateway MSC Server

GMSC	Gateway MSC

GPRS	General Packet Radio System

HNB GW	Home NodeB Gateway

ICSCF	Interrogating Call Session Control Function

IDL	Interface Definition Language

IMS	IP Multimedia Subsystem

IOC	Information Object Class

IRP	Integration Reference Point

ISO	International Standards Organization

IWF	InterWorking Function

ME	Managed Element

MGCF	Media Gateway Control Function

MGW	Media GateWay

MIM	Management Information Model

MNP-SRF	Mobile Number Portability-Signalling Relay Function

MO	Managed Object

MOI	Managed Object Instance

MRFC	Multimedia Resource Function Controller

MRFP	Call Session Control Function Processor

MSC Server	Mobile Services Switching Centre Server

MSC	Mobile Services Switching Centre

NE	Network Element

NM	Network Manager

NPDB	Number Portability DataBase

NR	Network Resource

NRM	Network Resource Model

OSI	Open Systems Interconnection

PCSCF	Proxy Call Session Control Function

PM	Performance Management

RDN	Relative Distinguished Name (see 3GPP TS 32.300 [13])

SCF	Service Control Function

SCSCF	Serving Call Session Control Function

SGSN	Serving GPRS Support Node

SGW	Signalling GateWay

SLF	Subscription Locator Function

SMLC	Serving Mobile Location Center

SMS	Short Message Service

SMS-GMSC	SMS Gateway MSC

SMS-IWMSC	SMS InterWorking MSC

SRF	Specialized Resource Function

SSF	Service Switching Function

TMN	Telecommunications Management Network

UML	Unified Modelling Language

UMTS	Universal Mobile Telecommunications System

UTRAN	Universal Terrestrial Radio Access Network

VLR	Visitor Location Register

# 4	System overview

## 4.1	Compliance rules

The following defines the meaning of Mandatory and Optional IOC attributes and associations between IOCs, in Solution Sets to the IRP defined by the present document:

- The IRPManager shall support all mandatory attributes/associations. The IRPManager shall be prepared to receive information related to mandatory as well as optional attributes/associations without failure; however the IRPManager does not have to support handling of the optional attributes/associations.

- The IRPAgent shall support all mandatory attributes/associations. It may support optional attributes/associations.

An IRPAgent that incorporates vendor-specific extensions shall support normal communication with a 3GPP SA5-compliant IRPManager with respect to all Mandatory and Optional information object classes, attributes and associations without requiring the IRPManager to have any knowledge of the extensions.

Given that:

- rules for vendor-specific extensions remain to be fully specified; and

- many scenarios under which IRPManager and IRPAgent interwork may exist;

it is recognized that the IRPManager, even though it is not required to have knowledge of vendor-specific extensions, may be required to be implemented with an awareness that extensions can exist and behave accordingly.

# 5	Modelling approach

The modelling approach is described in the Generic Network Resources IRP: NRM (3GPP TS 32.622 [16]).

It should be noted that this model allows for combined managed element functionality, where more than one ‘function IOCs' (inherited from ManagedFunction) modelling more specific managed element functionality may be contained in the ManagedElement IOC.

# 6	Information Object Classes

## 6.1	Imported information entities and local labels

| Label reference | Local label |
| --- | --- |
| TS 32.622 [16], information object class, Link | Link |
| TS 32.622 [16], information object class, ManagedElement | ManagedElement |
| TS 32.642 [22], information object class, RncFunction | RncFunction |
| TS 32.652 [23], information object class, BssFunction | BssFunction |
| TS 32.652 [23], information object class, ExternalBssFunction | ExternalBssFunction |
| TS 32.652 [23], information object class, ExternalGsmCell | ExternalGsmCell |
| TS 32.652 [23], information object class, GsmCell | GsmCell |

## 6.2	Class diagram

### 6.2.1	Attributes and relationships

This clause depicts the set of IOCs that encapsulate information relevant for this service. This clause provides the overview of all information object classes in UML. Subsequent clauses provide more detailed specification of various aspects of these information object classes.

The figures below show the containment/naming hierarchy and the associations of the information object classes defined in the present document.

NOTE:	The listed cardinality numbers represent transient as well as steady-state numbers, and reflect all managed object creation and deletion scenarios in all figures.

Figure 6.2.1.1: CN NRM Containment/Naming relationships 1

Figure 6.2.1.2: CN NRM Containment/Naming relationships 2

- Figure 6.2.1.3: Void

CbcFunction

<<InformationObjectClass>>

SgsnFunction

<<InformationObjectClass>>

ManagedElement

(from 32.622)

<<InformationObjectClass>>

0..1

1

0..1

1

<<names>>

0..1

1

0..1

1

<<names>>

CsMgwFunction

<<InformationObjectClass>>

0..n

1

0..n

1

<<names>>

MscServerFunction

<<InformationObjectClass>>

0..1

0..n

0..1

0..n

AssociatedWith3

0..1

1

0..1

1

<<names>>

RncFunction

(from 32.642)

<<InformationObjectClass>>

IucsLink

<<InformationObjectClass>>

0..n

1

0..n

+csMgwFunction-IucsLink

1

ConnectedTo1

0..n

1

1

<<names>>

0..1

0..n

0..1

+connectedRnc

0..n

ConnectedTo3

ExternalRNCFunction (from 32.642)

<<InformationObjectClass>>

0..1

0..n

ConnectedTo2

0..1

0..n

+connectedRnc

IupsLink

<<InformationObjectClass>>

0..n

1

1

<<names>>

0..1

0..n

0..1

+connectedRnc

0..n

ConnectedTo4

IubcLink

<<InformationObjectClass>>

0..n

1

1

<<names>>

0..1

0..n

0..1

+connectedRnc

0..n

ConnectedTo6

ExternalRNCFunction (from 32.642)

<<InformationObjectClass>>

0..1

0..n

0..1

0..n

ConnectedTo5

+connectedRnc

0..1

ConnectedTo7

+connectedRnc

0..1

NOTE:	The association between MscServerFunction and CsMgwFunction is optional and is only mandatory when they belong to different ManagedElements.

- Figure 6.2.1.4: CN UTRAN NRM Containment/Naming and Association

MscServerFunction

<<InformationObjectClass>>

CsMgwFunction

<<InformationObjectClass>>

0..n

0..1

0..n

0..1

AssociatedWith3

ManagedElement

(from 32.622)

<<InformationObjectClass>>

1

0..1

1

0..1

<<names>>

1

0..n

1

0..n

<<names>>

GsmCell

(from 32.652)

<<InformationObjectClass>>

0..1

0..n

0..1

0..n

AssociatedWith1

ExternalGsmCell

(from 32.652)

<<InformationObjectClass>>

0..n

0..1

0..n

0..1

AssociatedWith2

ALink

<<InformationObjectClass>>

0..n

1

..

1

<<names>>

0..n

0..1

0..n

+csMgwFunction-ALink

0..1

ConnectedTo8

IucsLink

<<InformationObjectClass>>

IupsLink

<<InformationObjectClass>>

SgsnFunction

<<InformationObjectClass>>

0..1

1

0..1

1

<<names>>

0..n

0..1

0..n

0..1

AssociatedWith4

0..n

0..1

0..n

0..1

AssociatedWith5

BssFunction

(from 32.652)

<<InformationObjectClass>>

0..1

0..n

0..1

+connectedBss

ConnectedTo12

0..1

0..n

0..1

+onnectedBss

ConnectedTo11

0..1

0..n

0..1

+connectedBss

ConnectedTo14

ExternalBssFunction

(from 32.652)

<<InformationObjectClass>>

0..1

0..n

0..1

+connectedBss

ConnectedTo9

0..1

0..n

0..1

+connectedBss

ConnectedTo10

0..1

0..1

+connectedBss

0..n

ConnectedTo15

GbLink

<<InformationObjectClass>>

0..n

1

0..n

1

<<names>>

0..1

0..n

0..1

+connectedBss

ConnectedTo13

0..1

0..n

0..1

+connectedBss

ConnectedTo16

NOTE 1:	The association between MscServerFunction and CsMgwFunction is optional and is only mandatory when they belong to different ManagedElements.

NOTE 2:	The association between MscServerFunction and GsmCell, and SgsnFunction and GsmCell are optional. It may be valid if both the MscServerFunction and GsmCell, or SgsnFunction and GsmCell are managed by the same management node.

Figure 6.2.1.5: CN GERAN NRM Containment/Naming and Association

Each IOC is identified with a Distinguished Name (DN) according to 3GPP TS 32.300 [13] that expresses its containment hierarchy. As an example, the DN of an IOC representing a cell could have a format like:

SubNetwork=Sweden, MeContext =MEC-Gbg-1, ManagedElement =MSC-Gbg-1, MscServerFunction=MSC-1.

Figure 6.2.1.6: CN MBMS NRM Containment/Naming and Association

Figure 6.2.1.7: CN CircuitEndPointSubgroup related NRM Containment/Naming and Association

### 6.2.2	Inheritance

This clause depicts the inheritance relationships that exist between IOCs.

The figures below show the inheritance hierarchy for the CN NRM.

Figure 6.2.2.1: CN NRM Inheritance Hierarchy 1

Figure 6.2.2.2: CN NRM Inheritance Hierarchy 2

Figure 6.2.2.3: CN NRM Inheritance Hierarchy 3

Figure 6.2.2.4: CN NRM Inheritance Hierarchy 4

## 6.3	Information object class definitions

### 6.3.1	MscServerFunction

#### 6.3.1.1	Definition

This IOC represents MSCserver functionality. For more information about the MSC, see 3GPP TS 23.002 [15].

#### 6.3.1.2	Attributes

| Attribute name | Visibility | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| mscServerFunctionId | + | M | M | - |
| userLabel | + | M | M | M |
| mccList | + | M | M | M |
| mncList | + | M | M | M |
| lacList | + | M | M | M |
| sacList | + | M | M | M |
| gcaList | + | O | M | M |
| mscId | + | M | M | M |
| mscServerFunction-GsmCell | + | M | M | - |
| mscServerFunction-ExternalGsmCell | + | M | M | - |
| mscServerFunction-CsMgwFunction | + | M | M | - |

#### 6.3.1.3	Notifications

The common notifications defined in subclause 6.7 are valid for this IOC, without exceptions or additions.

### 6.3.2	HlrFunction

#### 6.3.2.1	Definition

This IOC represents HLR functionality. For more information about the HLR, see 3GPP TS 23.002 [15].

#### 6.3.2.2	Attributes

| Attribute name | Visibility | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| hlrFunctionId | + | M | M | - |
| userLabel | + | M | M | M |

#### 6.3.2.3	Notifications

The common notifications defined in subclause 6.7 are valid for this IOC, without exceptions or additions.

### 6.3.3	VlrFunction

#### 6.3.3.1	Definition

This IOC represents VLR functionality. For more information about the VLR, see 3GPP TS 23.002 [15].

#### 6.3.3.2	Attributes

| Attribute name | Visibility | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| vlrFunctionId | + | M | M | - |
| userLabel | + | M | M | M |

#### 6.3.3.3	Notifications

The common notifications defined in subclause 6.7 are valid for this IOC, without exceptions or additions.

### 6.3.4	AucFunction

#### 6.3.4.1	Definition

This IOC represents AUC functionality. For more information about the AUC, see 3GPP TS 23.002 [15].

#### 6.3.4.2	Attributes

| Attribute name | Visibility | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| aucFunctionId | + | M | M | - |
| userLabel | + | M | M | M |

#### 6.3.4.3	Notifications

The common notifications defined in subclause 6.7 are valid for this IOC, without exceptions or additions.

### 6.3.5	EirFunction

#### 6.3.5.1	Definition

This IOC represents EIR functionality. For more information about the EIR, see 3GPP TS 23.002 [15].

#### 6.3.5.2	Attributes

| Attribute name | Visibility | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| eirFunctionId | + | M | M | - |
| userLabel | + | M | M | M |

#### 6.3.5.3	Notifications

The common notifications defined in subclause 6.7 are valid for this IOC, without exceptions or additions.

### 6.3.6	SmsIwmscFunction

#### 6.3.6.1	Definition

This IOC represents SMS-IWMSC functionality. For more information about the SMS-IWMSC, see 3GPP TS 23.002 [15].

#### 6.3.6.2	Attributes

| Attribute name | Visibility | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| smsIwmscFunctionId | + | M | M | - |
| userLabel | + | M | M | M |

#### 6.3.6.3	Notifications

The common notifications defined in subclause 6.7 are valid for this IOC, without exceptions or additions.

### 6.3.7	SmsGmscFunction

#### 6.3.7.1	Definition

This IOC represents SMS-GMSC functionality. For more information about the SMS-GMSC, see 3GPP TS 23.002 [15].

#### 6.3.7.2	Attributes

| Attribute name | Visibility | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| smsGmscFunctionId | + | M | M | - |
| userLabel | + | M | M | M |

#### 6.3.7.3	Notifications

The common notifications defined in subclause 6.7 are valid for this IOC, without exceptions or additions.

### 6.3.8	GmscFunction

#### 6.3.8.1	Definition

This IOC represents GMSC functionality. For more information about the GMSC, see 3GPP TS 23.002 [15].

#### 6.3.8.2	Attributes

| Attribute name | Visibility | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| gmscFunctionId | + | M | M | - |
| userLabel | + | M | M | M |

#### 6.3.8.3	Notifications

The common notifications defined in subclause 6.7 are valid for this IOC, without exceptions or additions.

### 6.3.9	SgsnFunction

#### 6.3.9.1	Definitions

This IOC represents SGSN functionality. For more information about the SGSN, see 3GPP TS 23.002 [15].

#### 6.3.9.2	Attributes

| Attribute name | Visibility | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| sgsnFunctionId | + | M | M | - |
| userLabel | + | M | M | M |
| mccList | + | M | M | M |
| mncList | + | M | M | M |
| lacList | + | M | M | M |
| racList | + | M | M | M |
| sacList | + | M | M | M |
| sgsnId | + | M | M | M |
| sgsnFunction-GsmCell | + | M | M | - |
| sgsnFunction-ExternalGsmCell | + | M | M | - |
| proceduralStatus (Note) | % | O | - | - |
| Note: This proceduralStatus is not settable or readable via any Interface IRP except conveyed by notifyStateChange notifications. |  |  |  |  |

#### 6.3.9.3	Notifications

The common notifications defined in subclause 6.7 are valid for this IOC, without exceptions or additions.

### 6.3.10	GgsnFunction

#### 6.3.10.1	Definitions

This IOC represents GGSN functionality. For more information about the GGSN, see 3GPP TS 23.002 [15].

#### 6.3.10.2	Attributes

| Attribute name | Visibility | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| ggsnFunctionId | + | M | M | - |
| userLabel | + | M | M | M |
| proceduralStatus (Note) | % | O | - | - |
| Note: This proceduralStatus is not settable or readable via any Interface IRP except conveyed by notifyStateChange notifications. |  |  |  |  |

#### 6.3.10.3	Notifications

The common notifications defined in subclause 6.7 are valid for this IOC, without exceptions or additions.

### 6.3.11	BgFunction

#### 6.3.11.1	Definitions

This IOC represents BG functionality. For more information about the BG, see 3GPP TS 23.002 [15].

#### 6.3.11.2	Attributes

| Attribute name | Visibility | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| bgFunctionId | + | M | M | - |
| userLabel | + | M | M | M |

#### 6.3.11.3	Notifications

The common notifications defined in subclause 6.7 are valid for this IOC, without exceptions or additions.

### 6.3.12	SmlcFunction

#### 6.3.12.1	Definitions

This IOC represents SMLC functionality. For more information about the SMLC, see 3GPP TS 23.002 [15].

#### 6.3.12.2	Attributes

| Attribute name | Visibility | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| smlcFunctionId | + | M | M | - |
| userLabel | + | M | M | M |

#### 6.3.12.3	Notifications

The common notifications defined in subclause 6.7 are valid for this IOC, without exceptions or additions.

### 6.3.13	GmlcFunction

#### 6.3.13.1	Definitions

This IOC represents GMLC functionality. For more information about the GMLC, see 3GPP TS 23.002 [15].

#### 6.3.13.2	Attributes

| Attribute name | Visibility | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| gmlcFunctionId | + | M | M | - |
| userLabel | + | M | M | M |

#### 6.3.13.3	Notifications

The common notifications defined in subclause 6.7 are valid for this IOC, without exceptions or additions.

### 6.3.14	ScfFunction

#### 6.3.14.1	Definitions

This IOC represents SCF functionality (also referred to as gsmSCF). For more information about the SCF, see 3GPP TS 23.002 [15].

#### 6.3.14.2	Attributes

| Attribute name | Visibility | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| scfFunctionId | + | M | M | - |
| userLabel | + | M | M | M |

#### 6.3.14.3	Notifications

The common notifications defined in subclause 6.7 are valid for this IOC, without exceptions or additions.

### 6.3.15	SrfFunction

#### 6.3.15.1	Definitions

This IOC represents SRF functionality (also referred to as gsmSRF). For more information about the SRF, see 3GPP TS 23.002 [15].

#### 6.3.15.2	Attributes

| Attribute name | Visibility | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| srfFunctionId | + | M | M | - |
| userLabel | + | M | M | M |

#### 6.3.15.3	Notifications

The common notifications defined in subclause 6.7 are valid for this IOC, without exceptions or additions.

### 6.3.16	CbcFunction

#### 6.3.16.1	Definitions

This IOC represents CBC functionality. For more information about the CBC, see 3GPP TS 23.002 [15].

#### 6.3.16.2	Attributes

| Attribute name | Visibility | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| cbcFunctionId | + | M | M | - |
| userLabel | + | M | M | M |

#### 6.3.16.3	Notifications

The common notifications defined in subclause 6.7 are valid for this IOC, without exceptions or additions.

### 6.3.17	CgfFunction

#### 6.3.17.1	Definitions

This IOC represents CGF functionality. For more information about the CGF, see 3GPP TS 23.060 [18].

#### 6.3.17.2	Attributes

| Attribute name | Visibility | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| cgfFunctionId | + | M | M | - |
| userLabel | + | M | M | M |

#### 6.3.17.3	Notifications

The common notifications defined in subclause 6.7 are valid for this IOC, without exceptions or additions.

### 6.3.18	GmscServerFunction

#### 6.3.18.1	Definitions

This IOC represents GMSCServer functionality. For more information about GMSCServer, see 3GPP TS 23.002 [15].

#### 6.3.18.2	Attributes

| Attribute name | Visibility | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| gmscServerFunctionId | + | M | M | - |
| userLabel | + | M | M | M |

#### 6.3.18.3	Notifications

The common notifications defined in subclause 6.7 are valid for this IOC, without exceptions or additions.

### 6.3.19	IwfFunction

#### 6.3.19.1	Attributes

This IOC represents IWF functionality. For more information about IWF, see 3GPP TS 23.002 [15].

#### 6.3.19.2	Attributes

| Attribute name | Visibility | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| iwfFunctionId | + | M | M | - |
| userLabel | + | M | M | M |

#### 6.3.19.3	Notifications

The common notifications defined in subclause 6.7 are valid for this IOC, without exceptions or additions.

### 6.3.20	MnpSrfFunction

#### 6.3.20.1	Definitions

This IOC represents MNP-SRF functionality (also known as FNR). For more information about MNP-SRF, see 3GPP TS 23.002 [15].

#### 6.3.20.2	Attributes

| Attribute name | Visibility | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| mnpSrfFunctionId | + | M | M | - |
| userLabel | + | M | M | M |

#### 6.3.20.3	Notifications

The common notifications defined in subclause 6.7 are valid for this IOC, without exceptions or additions.

### 6.3.21	NpdbFunction

#### 6.3.21.1	Definitions

This IOC represents NPDB functionality. For more information about NPDB, see 3GPP TS 23.002 [15].

#### 6.3.21.2	Attributes

| Attribute name | Visibility | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| npdbFunctionId | + | M | M | - |
| userLabel | + | M | M | M |

#### 6.3.21.3	Notifications

The common notifications defined in subclause 6.7 are valid for this IOC, without exceptions or additions.

### 6.3.22	SgwFunction

#### 6.3.22.1	Definitions

This IOC represents SGW functionality. For more information about SGW, see 3GPP TS 23.002 [15].

#### 6.3.22.2	Attributes

| Attribute name | Visibility | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| sgwFunctionId | + | M | M | - |
| userLabel | + | M | M | M |

#### 6.3.22.3	Notifications

The common notifications defined in subclause 6.7 are valid for this IOC, without exceptions or additions.

### 6.3.23	SsfFunction

#### 6.3.23.1	Definitions

This IOC represents SSF functionality. For more information about SSF, see 3GPP TS 23.002 [15].

#### 6.3.23.2	Attributes

| Attribute name | Visibility | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| ssfFunctionId | + | M | M | - |
| userLabel | + | M | M | M |

#### 6.3.23.3	Notifications

The common notifications defined in subclause 6.7 are valid for this IOC, without exceptions or additions.

### 6.3.24	BsFunction

#### 6.3.24.1	Definitions

This IOC represents BS functionality. For more information about BS, see 3GPP TS 23.060 [18].

#### 6.3.24.2	Attributes

| Attribute name | Visibility | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| bsFunctionId | + | M | M | - |
| userLabel | + | M | M | M |

#### 6.3.24.3	Notifications

The common notifications defined in subclause 6.7 are valid for this IOC, without exceptions or additions.

### 6.3.25	IucsLink

#### 6.3.25.1	Definitions

This IOC represents an Iu-cs interface link connecting an MSCserver to the RNC, BSC and HNB GW. For more information about the Iu interface, see 3GPP TS 23.002 [15].

#### 6.3.25.2	Attributes

| Attribute name | Visibility | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| iucslinkId | + | M | M | - |
| userLabel | + | M | M | M |
| connectedRnc | + | O | M | - |
| connectedBss | + | O | M | - |
| connectedHNBGW | + | O | M | - |

#### 6.3.25.3	Attribute constraints

The optional attribute connectedRnc shall be supported when the Iucs interface is between the MSCServer node and an RNC node.

The optional attribute connectedBss shall be supported when the Iucs interface is between the MSCServer node and a BSC node.

The optional attribute connectedHNBGW shall be supported when the Iucs interface is between the MSCServer node and a HNB GW node [25].

The attributes connectedRnc, connectedBss and connectedHNBGW are mutually exclusive.

#### 6.3.25.4	Notifications

The common notifications defined in subclause 6.7 are valid for this IOC, without exceptions or additions.

### 6.3.26	IupsLink

#### 6.3.26.1	Definitions

This IOC represents an Iu-ps interface link connecting a SGSN to the RNC, BSC and HNB GW.  For more information about the Iu interface, see 3GPP TS 23.002 [15].

#### 6.3.26.2	Attributes

| Attribute name | Visibility | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| iupslinkId | + | M | M | - |
| userLabel | + | M | M | M |
| connectedRnc | + | O | M | - |
| connectedBss | + | O | M | - |

#### 6.3.26.3	Attribute constraints

The optional attribute connectedRnc shall be supported when the Iups interface is between the SGSN node and an RNC node.

The optional attribute connectedBss shall be supported when the Iups interface is between the SGSN node and a BSC node.

The optional attribute connectedHNBGW shall be supported when the Iucs interface is between the SGSN node and a HNB GW node [25].

The attributes connectedRnc, connectedBss and connectedHNBGW are mutually exclusive.

#### 6.3.26.4	Notifications

The common notifications defined in subclause 6.7 are valid for this IOC, without exceptions or additions.

### 6.3.27	IubcLink

#### 6.3.27.1	Definitions

This IOC represents an Iu-bc interface link connecting a CBC to the RNC and HNB GW. For more information about the Iu interface, see 3GPP TS 23.002 [15].

#### 6.3.27.2	Attributes

| Attribute name | Visibility | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| iubclinkId | + | M | M | - |
| userLabel | + | M | M | M |
| connectedRnc | + | M | M | - |
| connectedHNBGW | + | O | M | - |

#### 6.3.27.3	Notifications

The common notifications defined in subclause 6.7 are valid for this IOC, without exceptions or additions.

### 6.3.28	ALink

#### 6.3.28.1	Definitions

This IOC represents the A interface link connecting a MSC to the GERAN. For more information about the GERAN, see 3GPP TS 23.002 [15].

#### 6.3.28.2	Attributes

| Attribute name | Visibility | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| aLinkId | + | M | M | - |
| userLabel | + | M | M | M |
| connectedBss | + | M | M | - |

#### 6.3.28.3	Notifications

The common notifications defined in subclause 6.7 are valid for this IOC, without exceptions or additions.

### 6.3.29	GbLink

#### 6.3.29.1	Definitions

This IOC represents the Gb interface link connecting a SGSN to the GERAN. For more information about the GERAN, see 3GPP TS 23.002 [15].

#### 6.3.29.2	Attributes

| Attribute name | Visibility | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| gbLinkId | + | M | M | - |
| userLabel | + | M | M | M |
| connectedBss | + | M | M | - |

#### 6.3.29.3	Notifications

The common notifications defined in subclause 6.7 are valid for this IOC, without exceptions or additions.

### 6.3.30	CsMgwFunction

#### 6.3.30.1	Definitions

This IOC represents CS-MGW functionality. For more information about CS-MGW, see 3GPP TS 23.002 [15].

#### 6.3.30.2	Attributes

| Attribute name | Visibility | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| csMgwFunctionId | + | M | M | - |
| userLabel | + | M | M | M |
| csMgwFunction-MscServerFunction | + | M | M | - |
| csMgwFunction-IucsLink | + | M | M | - |
| csMgwFunction-ALink | + | M | M | - |

#### 6.3.30.3	Notifications

The common notifications defined in subclause 6.7 are valid for this IOC, without exceptions or additions.

### 6.3.31	Void

### 6.3.32	Void

### 6.3.33	Void

### 6.3.34	Void

### 6.3.35	BmScFunction

#### 6.3.35.1	Definitions

This IOC represents BM-SC functionality. For more information about BM-SC see 3GPP TS 23.002 [15].

#### 6.3.35.2	Attributes

| Attribute name | Visibility | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| bmScFunctionId | + | M | M | - |

#### 6.3.35.3	Notifications

The common notifications defined in subclause 6.7 are valid for this IOC, without exceptions or additions.

### 6.3.36	Link_BmSc_Ggsn

#### 6.3.36.1	Definition

This IOC models the Gmb and Gi reference points as defined in TS 23.002 [8].

### 6.3.37	Link_Ggsn_Sgsn

#### 6.3.37.1	Definition

This IOC models the Gn and Gp reference points as defined in TS 23.002 [8].

### 6.3.38	CircuitEndPointSubgroup

#### 6.3.38.1	Definitions

This IOC represents the Circuit End Point Subgroup, relating to definitions in ITU-T M.3100 (see [24]). A Circuit End Point Subgroup is a set of circuit end points that directly interconnect one network element with another (e.g. MSC, BSC). It is derived from Top.

#### 6.3.38.2	Attributes

| Attribute name | Visibility | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| circuitEndPointSubgroupId | + | M | M | - |

#### 6.3.38.3	Notifications

This IOC would not emit notification.

## 6.4	Information relationship definitions

### 6.4.1	AssociatedWith1 (M)

#### 6.4.1.1	Definition

This represents a bi-directional relation between the MscServerFunction and GsmCell.

The role of the relation shall be mapped to a reference attribute of the IOC. The name of the reference attribute shall be the role name.

#### 6.4.1.2	Roles

Table 6.4.1: Roles of the relation AssociatedWith1

| Name | Definition |
| --- | --- |
| mscServerFunction-GsmCell | This role (when present) represents MscServerFunction capability to identify the set of related GsmCell. The mscServerFunction-GsmCell shall carry the set of GsmCell DN(s). |
| gsmCell-MscServerFunction | This role (when present) represents GsmCell capability to identify one related MscServerFunction. When the role is absent, the gsmCell-MscServerFunction shall contain no information. When it is present, it shall contain one MscServerFunction DN. |

#### 6.4.1.3	Constraints

| Name | Definition |
| --- | --- |
| - | - |

### 6.4.2	AssociatedWith2 (M)

#### 6.4.2.1	Definition

This represents a bi-directional relation between the MscServerFunction and ExternalGsmCell.

The role of the relation shall be mapped to a reference attribute of the IOC. The name of the reference attribute shall be the role name.

#### 6.4.2.2	Roles

Table 6.4.2: Roles of the relation AssociatedWith2

| Name | Definition |
| --- | --- |
| mscServerFunction-ExternalGsmCell | This role (when present) represents MscServerFunction capability to identify the set of related ExternalGsmCell. The mscServerFunction-ExternalGsmCell shall carry the set of ExternalGsmCell DN(s). |
| externalGsmCell-MscServerFunction | This role (when present) represents ExternalGsmCell capability to identify one related MscServerFunction. When the role is absent, the externalGsmCell-MscServerFunction shall contain no information. When it is present, it shall contain one MscServerFunction DN. |

#### 6.4.2.3	Constraints

| Name | Definition |
| --- | --- |
| - | - |

### 6.4.3	AssociatedWith3 (M)

#### 6.4.3.1	Definition

This represents a bi-directional relation between the MscServerFunction and CsMgwFunction.

The role of the relation shall be mapped to a reference attribute of the IOC. The name of the reference attribute shall be the role name.

#### 6.4.3.2	Roles

Table 6.4.3: Roles of the relation AssociatedWith3

| Name | Definition |
| --- | --- |
| mscServerFunction-CsMgwFunction | This role (when present) represents MscServerFunction capability to identify the related CsMgwFunction(s). The mscServerFunction-CsMgwFunction shall carry the CsMgwFunction DN(s). |
| csMgwFunction-MscServerFunction | This role (when present) represents CsMgwFunction capability to identify one related MscServerFunction. When the role is absent, the csMgwFunction-MscServerFunction shall contain no information. When it is present, it shall contain one MscServerFunction DN. |

#### 6.4.3.3	Constraints

| Name | Definition |
| --- | --- |
| - | - |

### 6.4.4	AssociatedWith4 (M)

#### 6.4.4.1	Definition

This represents a bi-directional relation between the SgsnFunction and GsmCell.

The role of the relation shall be mapped to a reference attribute of the IOC. The name of the reference attribute shall be the role name.

#### 6.4.4.2	Roles

Table 6.4.4: Roles of the relation AssociatedWith4

| Name | Definition |
| --- | --- |
| sgsnFunction-GsmCell | This role (when present) represents SgsnFunction capability to identify the set of related GsmCells. The sgsnFunction-GsmCell shall carry the set of GsmCell DN(s). |
| gsmCell-SgsnFunction | This role (when present) represents GsmCell capability to identify one related SgsnFunction. When the role is absent, the gsmCell-SgsnFunction shall contain no information. When it is present, it shall contain one SgsnFunction DN. |

#### 6.4.4.3	Constraints

| Name | Definition |
| --- | --- |
| - | - |

### 6.4.5	AssociatedWith5 (M)

#### 6.4.5.1	Definition

This represents a bi-directional relation between the SgsnFunction and ExternalGsmCell.

The role of the relation shall be mapped to a reference attribute of the IOC. The name of the reference attribute shall be the role name.

#### 6.4.5.2	Roles

Table 6.4.5: Roles of the relation AssociatedWith5

| Name | Definition |
| --- | --- |
| sgsnFunction-ExternalGsmCell | This role (when present) represents SgsnFunction capability to identify the set of related ExternalGsmCell. The sgsnFunction-ExternalGsmCell shall carry the set of ExternalGsmCell DN(s). |
| externalGsmCell-SgsnFunction | This role (when present) represents ExternalGsmCell capability to identify one related SgsnFunction. When the role is absent, the externalGsmCell-SgsnFunction shall contain no information. When it is present, it shall contain one SgsnFunction DN. |

#### 6.4.5.3	Constraints

| Name | Definition |
| --- | --- |
| - | - |

### 6.4.6	ConnectedTo1 (M)

#### 6.4.6.1	Definition

This represents a uni-directional relation between the CsMgwFunction and IucsLink.

The role of the relation shall be mapped to a reference attribute of the IOC.

#### 6.4.6.2	Roles

Table 6.4.6: Roles of the relation ConnectedTo1

| Name | Definition |
| --- | --- |
| csMgwFunction-IucsLink | This role (when present) represents CsMgwFunction capability to identify the set of connected IucsLinks. When the role is present, the csMgwFunction-IucsLink shall carry the set of IucsLink DN(s). |

#### 6.4.6.3	Constraints

| Name | Definition |
| --- | --- |
| - | - |

### 6.4.7	ConnectedTo2 (M)

#### 6.4.7.1	Definition

This represents a uni-directional relation between the IucsLink and ExternalRncFunction.

The role of the relation shall be mapped to a reference attribute of the IOC.

#### 6.4.7.2	Roles

Table 6.4.7: Roles of the relation ConnectedTo2

| Name | Definition |
| --- | --- |
| connectedRnc | This role (when present) represents IOC IucsLink capability to identify one connected ExternalRncFunction. When present, it shall contain one ExternalRncFunction DN. |

#### 6.4.7.3	Constraints

| Name | Definition |
| --- | --- |
| - | - |

### 6.4.8	ConnectedTo3 (M)

#### 6.4.8.1	Definition

This represents a uni-directional relation between the IucsLink and RncFunction.

The role of the relation shall be mapped to a reference attribute of the IOC.

#### 6.4.8.2	Roles

Table 6.4.8: Roles of the relation ConnectedTo3

| Name | Definition |
| --- | --- |
| connectedRnc | This role (when present) represents IOC IucsLink capability to identify one connected RncFunction. When present, it shall contain one RncFunction DN. |

#### 6.4.8.3	Constraints

| Name | Definition |
| --- | --- |
| - | - |

### 6.4.9	ConnectedTo4 (M)

#### 6.4.9.1	Definition

This represents a uni-directional relation between the IupsLink and RncFunction.

The role of the relation shall be mapped to a reference attribute of the IOC.

#### 6.4.9.2	Roles

Table 6.4.9: Roles of the relation ConnectedTo4

| Name | Definition |
| --- | --- |
| connectedRnc | This role (when present) represents IOC IupsLink capability to identify one connected RncFunction. When present, it shall contain one RncFunction DN. |

#### 6.4.9.3	Constraints

| Name | Definition |
| --- | --- |
| - | - |

### 6.4.10	ConnectedTo5 (M)

#### 6.4.10.1	Definition

This represents a uni-directional relation between the IupsLink and ExternalRncFunction.

The role of the relation shall be mapped to a reference attribute of the IOC.

#### 6.4.10.2	Roles

Table 6.4.10: Roles of the relation ConnectedTo5

| Name | Definition |
| --- | --- |
| connectedRnc | This role (when present) represents IOC IupsLink capability to identify one connected ExternalRncFunction. When present, it shall contain one ExternalRncFunction DN. |

#### 6.4.10.3	Constraints

| Name | Definition |
| --- | --- |
| - | - |

### 6.4.11	ConnectedTo6 (M)

#### 6.4.11.1	Definition

This represents a uni-directional relation between the IubcLink and RncFunction.

The role of the relation shall be mapped to a reference attribute of the IOC.

#### 6.4.11.2	Roles

Table 6.4.11: Roles of the relation ConnectedTo6

| Name | Definition |
| --- | --- |
| connectedRnc | This role (when present) represents IOC IubcLink capability to identify one connected RncFunction. When present, it shall contain one RncFunction DN. |

#### 6.4.11.3	Constraints

| Name | Definition |
| --- | --- |
| - | - |

### 6.4.12	ConnectedTo7 (M)

#### 6.4.12.1	Definition

This represents a uni-directional relation between the IubcLink and ExternalRncFunction.

The role of the relation shall be mapped to a reference attribute of the IOC.

#### 6.4.12.2	Roles

Table 6.4.12: Roles of the relation ConnectedTo7

| Name | Definition |
| --- | --- |
| connectedRnc | This role (when present) represents IOC IubcLink capability to identify one connected ExternalRncFunction. When present, it shall contain one ExternalRncFunction DN. |

#### 6.4.12.3	Constraints

| Name | Definition |
| --- | --- |
| - | - |

### 6.4.13	ConnectedTo8 (M)

#### 6.4.13.1	Definition

This represents a uni-directional relation between the CsMgwFunction and ALink.

The role of the relation shall be mapped to a reference attribute of the IOC.

#### 6.4.13.2	Roles

Table 6.4.13: Roles of the relation ConnectedTo8

| Name | Definition |
| --- | --- |
| csMgwFunction-ALink | This role (when present) represents CsMgwFunction capability to identify the set of connected ALinks. When the role is present, the csMgwFunction-ALink shall carry the set of ALink DN(s). |

#### 6.4.13.3	Constraints

| Name | Definition |
| --- | --- |
| - | - |

### 6.4.14	ConnectedTo9 (M)

#### 6.4.14.1	Definition

This represents a uni-directional relation between the ALink and ExternalBssFunction.

The role of the relation shall be mapped to a reference attribute of the IOC.

#### 6.4.14.2	Roles

Table 6.4.14: Roles of the relation ConnectedTo9

| Name | Definition |
| --- | --- |
| connectedBss | This role (when present) represents IOC ALink capability to identify one connected ExternalBssFunction. When present, it shall contain one ExternalBssFunction DN. |

#### 6.4.14.3	Constraints

| Name | Definition |
| --- | --- |
| - | - |

### 6.4.15	ConnectedTo10 (M)

#### 6.4.15.1	Definition

This represents a uni-directional relation between the Iucslink and ExternalBssFunction.

The role of the relation shall be mapped to a reference attribute of the IOC.

#### 6.4.15.2	Roles

Table 6.4.15: Roles of the relation ConnectedTo10

| Name | Definition |
| --- | --- |
| connectedBss | This role (when present) represents IOC Iucslink capability to identify one connected ExternalBssFunction. When present, it shall contain one ExternalBssFunction DN. |

#### 6.4.15.3	Constraints

| Name | Definition |
| --- | --- |
| - | - |

### 6.4.16	ConnectedTo11 (M)

#### 6.4.16.1	Definition

This represents a uni-directional relation between the Iucslink and BssFunction.

The role of the relation shall be mapped to a reference attribute of the IOC.

#### 6.4.16.2	Roles

Table 6.4.16: Roles of the relation ConnectedTo11

| Name | Definition |
| --- | --- |
| connectedBss | This role (when present) represents IOC Iucslink capability to identify one connected BssFunction. When present, it shall contain one BssFunction DN. |

#### 6.4.16.3	Constraints

| Name | Definition |
| --- | --- |
| - | - |

### 6.4.17	ConnectedTo12 (M)

#### 6.4.17.1	Definition

This represents a uni-directional relation between the ALink and BssFunction.

The role of the relation shall be mapped to a reference attribute of the IOC.

#### 6.4.17.2	Roles

Table 6.4.17: Roles of the relation ConnectedTo12

| Name | Definition |
| --- | --- |
| connectedBss | This role (when present) represents IOC ALink capability to identify one connected BssFunction. When present, it shall contain one BssFunction DN. |

#### 6.4.17.3	Constraints

| Name | Definition |
| --- | --- |
| - | - |

### 6.4.18	ConnectedTo13 (M)

#### 6.4.18.1	Definition

This represents a uni-directional relation between the GbLink and BssFunction.

The role of the relation shall be mapped to a reference attribute of the IOC.

#### 6.4.18.2	Roles

Table 6.4.18: Roles of the relation ConnectedTo13

| Name | Definition |
| --- | --- |
| connectedBss | This role (when present) represents IOC GbLink capability to identify one connected BssFunction. When present, it shall contain one BssFunction DN. |

#### 6.4.18.3	Constraints

| Name | Definition |
| --- | --- |
| - | - |

### 6.4.19	ConnectedTo14 (M)

#### 6.4.19.1	Definition

This represents a uni-directional relation between the IupsLink and BssFunction.

The role of the relation shall be mapped to a reference attribute of the IOC.

#### 6.4.19.2	Roles

Table 6.4.19: Roles of the relation ConnectedTo14

| Name | Definition |
| --- | --- |
| connectedBss | This role (when present) represents IOC IupsLink capability to identify one connected BssFunction. When present, it shall contain one BssFunction DN. |

#### 6.4.19.3	Constraints

| Name | Definition |
| --- | --- |
| - | - |

### 6.4.20	ConnectedTo15 (M)

#### 6.4.20.1	Definition

This represents a uni-directional relation between the IupsLink and ExternalBssFunction.

The role of the relation shall be mapped to a reference attribute of the IOC.

#### 6.4.20.2	Roles

Table 6.4.20: Roles of the relation ConnectedTo15

| Name | Definition |
| --- | --- |
| connectedBss | This role (when present) represents IOC IupsLink capability to identify one connected ExternalBssFunction. When present, it shall contain one ExternalBssFunction DN. |

#### 6.4.20.3	Constraints

| Name | Definition |
| --- | --- |
| - | - |

### 6.4.21	ConnectedTo16 (M)

#### 6.4.21.1	Definition

This represents a uni-directional relation between the GbLink and ExternalBssFunction.

The role of the relation shall be mapped to a reference attribute of the IOC.

#### 6.4.21.2	Roles

Table 6.4.21: Roles of the relation ConnectedTo16

| Name | Definition |
| --- | --- |
| connectedBss | This role (when present) represents IOC GbLink capability to identify one connected ExternalBssFunction. When present, it shall contain one ExternalBssFunction DN. |

#### 6.4.21.3	Constraints

| Name | Definition |
| --- | --- |
| - | - |

## 6.5	Information attribute definitions

### 6.5.1	Definition and legal values

Table 6.5.1 defines the attributes that are present in several information object classes of the present document.

Table 6.5.1: Attributes

| Attribute Name | Definition | Legal Values |
| --- | --- | --- |
| aucFunctionId | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of the IOC. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| bgFunctionId | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of the IOC. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| bmScFunctionId | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of the IOC. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| bsFunctionId | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of the IOC. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| cbcFunctionId | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of the IOC. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| circuitEndPointSubgroupId | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of the IOC. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| cgfFunctionId | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of the IOC. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| csMgwFunctionId | An attribute whose ‘name+value' can be used as an RDN when naming an instance of the IOC. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| eirFunctionId | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of the IOC. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| gbLinkId | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of the IOC. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| gcaList | List of Group Call Area (Ref. 3GPP TS 23.003 [19]). |  |
| ggsnFunctionId | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of the IOC. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| gmlcFunctionId | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of the IOC. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| gmscFunctionId | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of the IOC. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| gmscServerFunctionId | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of the IOC. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| hlrFunctionId | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of the IOC. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| iubclinkId | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of the IOC. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| iucslinkId | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of the IOC. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| iupslinkId | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of the IOC. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| iwfFunctionId | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of the IOC. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| lacList | List of Location Area Codes (Ref. 3GPP TS 23.003 [19]). |  |
| mccList | List of Mobile Country Codes, MCC (part of the PLMN Id, Ref. 3GPP TS 23.003 [19]). |  |
| mncList | List of Mobile Network Codes, MNC (part of the PLMN Id, Ref. 3GPP TS 23.003 [19]). |  |
| mnpSrfFunctionId | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of the IOC. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| mscId | Unique MSC ID (Ref. 3GPP TS 23.002 [15]). |  |
| mscServerFunctionId | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of the IOC. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| npdbFunctionId | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of the IOC. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| objectClass | An attribute inherited from generic NRM link [16] |  |
| objectInstance | An attribute inherited from generic NRM link [16] |  |
| proceduralStatus | It indicates the procedural status of the object instance. This attribute provides a subset of capabilities of procedural status defined in [20]. There are two cases resulting in a status change to be reported: - Case 1: A notification may be generated to indicate that restart procedure is about to begin or has just begun but has not finished. - the value for this attribute indicates original state == “notInitialized” and new state == “initializing”. - Case 2: A notification shall be generated to indicate that restart procedure has completed successfully - the value for this attribute indicates original state == “initializing” to new state == “” (empty set). | Subset of definitions from [20]: “notInitialized”, “initializing”, “” (empty set) |
| racList | List of Routeing Area Codes covered by MSC (Ref. 3GPP TS 23.003 [19]). |  |
| sacList | List of Service Area Codes covered by MSC (Ref. 3GPP TS 23.003 [19]). |  |
| scfFunctionId | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of the IOC. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| sgsnFunctionId | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of the IOC. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| sgsnId | Unique SGSN ID (Ref. 3GPP TS 23.002 [15]). |  |
| sgwFunctionId | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of the IOC. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| smlcFunctionId | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of the IOC. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| smsGmscFunctionId | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of the IOC. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| smsIwmscFunctionId | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of the IOC. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| srfFunctionId | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of the IOC. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| ssfFunctionId | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of the IOC. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| userLabel | A user-friendly (and user assigned) name of the associated IOC. Inherited from ManagedFunction. |  |
| vlrFunctionId | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of the IOC. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |

### 6.5.2	Constraints

| Name | Definition |
| --- | --- |
| - | - |

## 6.6	Particular information configurations

Not applicable.

## 6.7	Common notifications

This subclause presents a list of notifications that can be referred to by any IOC defined by this IRP specification. 
These notifications are only applicable to IOCs referring to this subclause.

- Notifications

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyAckStateChanged | See Alarm IRP (3GPP TS 32.111-2 [5]) |  |
| notifyAlarmListRebuilt | See Alarm IRP (3GPP TS 32.111-2 [5]) |  |
| notifyChangedAlarm | See Alarm IRP (3GPP TS 32.111-2 [5]) |  |
| notifyClearedAlarm | See Alarm IRP (3GPP TS 32.111-2 [5]) |  |
| notifyComments | See Alarm IRP (3GPP TS 32.111-2 [5]) |  |
| notifyNewAlarm | See Alarm IRP (3GPP TS 32.111-2 [5]) |  |
| notifyPotentialFaultyAlarmList | See Alarm IRP (3GPP TS 32.111-2 [5]) |  |
| notifyAttributeValueChange | See Kernel CM IRP (3GPP TS 32.662 [3]) |  |
| notifyObjectCreation | See Kernel CM IRP (3GPP TS 32.662 [3]) |  |
| notifyObjectDeletion | See Kernel CM IRP (3GPP TS 32.662 [3]) |  |

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Cat | Old | New |
| Dec 2006 | SA_34 | SP-060731 | 0026 | -- | Move IMS part to new IMS NRM (32.732) | C | 6.6.0 | 7.0.0 |
| Jan 2007 | -- | -- | -- | -- | Editorial: Re-inserted deleted Figure 6.2.1.3: Void to keep original numbering | -- | 7.0.0 | 7.0.1 |
| Mar 2007 | SA_35 | SP-070047 | 0027 | -- | Removing links to IMS NRM | F | 7.0.1 | 7.1.0 |
| Jun 2007 | SA_36 | SP-070277 | 0029 | -- | Add the BmScFunction to the CN NRM - Needed for the TraceIRP | B | 7.1.0 | 7.2.0 |
| Dec 2007 | SA_38 | SP-070740 | 0030 | -- | Correct description of common notifications - Align with 32.151 IRP IS Template | F | 7.2.0 | 8.0.0 |
| Jun 2008 | SA_40 | SP-080328 | 0032 | -- | Add definition of IOC CircuitEndPointSubgroup - Align with 32.407 | A | 8.0.0 | 8.1.0 |
| Dec 2009 | - | - | - | - | Update to Rel-9 version | -- | 8.1.0 | 9.0.0 |
| Mar 2010 | SA_47 | SP-100038 | 0033 | -- | Addition of HNBGW relationship attributions in CN NRM | B | 9.0.0 | 9.1.0 |