3GPP TSG-SA WG6 Meeting #38-e	S6-201012

# e-meeting, 20th – 31th July 2020													(revision of S6-xxxxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.282 | CR | 0225 | rev |  | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update information flows to support interconnection for the IP connectivity service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMONASTERY2 |  |  |  |  |  | Date: |  |  | 2020-07-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Railway communication requires support interconnection for the IP connectivity service. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adds missing server to server information flows for the IP connectivity service. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No server to server IP connectivity messages available. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.14.2.1.1, 7.14.2.1.2, 7.14.2.1.3, 7.14.2.1.4, 7.14.2.1.5, 7.14.2.1.6, 7.14.2.1.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 7.14.2.1.1	MCData IPcon point-to-point request

Table 7.14.2.1.1-1 describes the information flow of the MCData IPcon point-to-point request sent from the MCData client to the MCData server, from the MCData server to the MCData server and from the MCData server to the MCData client.

Table 7.14.2.1.1-1: MCData IPcon point-to-point request (MCData client to MCData server)

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The MCData identity of the originator MCData user; |
| Functional alias | O | The associated functional alias of the originator MCData user; |
| MCData ID | O (NOTE 2) | The MCData identity of the target MCData client IP connectivity is requested. |
| Functional alias | O (NOTE 2) | The functional alias of the target MCData client. |
| Requested Priority (NOTE 3) | O | Application priority level requested for this communication. |
| Location Information | O (NOTE 1) | Actual location information of the originating MCData user; |
| Time Limit | O | Proposed time limit of the requested IP connectivity (1min- infinite); |
| Establishment reason | O | IP connectivity establishment reason |
| NOTE 1: This information contains the latest available location information of the requesting MCData user that may be different to the latest available location information in the MC system. NOTE 2: Either the MCData ID or the functional alias of the target MCData user must be present. NOTE 3: The predefined priority of the MC service user is applied by the MCData server if the requested priority is not present or not accepted by the MCData server. |  |  |

Table 7.14.2.1.1-2: MCData IPcon point-to-point request (MCData server to MCData client and MCData server to MCData client)

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The MCData identity of the originator MCData user; |
| MCData ID | M | The MCData identity of the target MCData client IP connectivity is requested. |
| Location Information | O (NOTE 1) | Actual location information of the originating MCData user; |
| Time Limit | O | Proposed time limit of the requested IP connectivity (1min- infinite); |
| Establishment reason | O | IP connectivity establishment reason |
| NOTE 1: This information contains the latest available location information of the requesting MCData user. |  |  |

##### 7.14.2.1.2	MCData IPcon point-to-point response

Table 7.14.2.1.2-1 describes the information content of the MCData IPcon point-to-point response as answer to MCData IPcon point-to-point request from the MCData client to the MCData server, from the MCData server to the MCData server and from the MCData server to the MCData client.

Table 7.14.2.1.2-1: MCData IPcon point-to-point response

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The MCData identity of the targeted MCData user. |
| MCData ID | M | The MCData identity of the requesting MCData user. |
| Time Limit | O | Negotiated time (1 min – infinite) |
| IP connectivity status | M | IP connectivity establishment result |

##### 7.14.2.1.3	MCData remote IPcon point-to-point request

Table 7.14.2.1.3-1 describes the information flow of the MCData remote IPcon point-to-point request sent from the remote MCData client to the MCData server, from the remote MCData server to the MCData server and from the MCData server to the asked MCData client.

Table 7.14.2.1.3-1: MCData remote IPcon point-to-point request

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID remote | M | The MCData identity of the remote MCData client that requests another MCData user to establish an IP connectivity point-to-point session. |
| Functional alias remote | O | The associated functional alias of the remote MCData user. |
| MCData ID asked | M | The MCData identity of the MCData client that is required to establish an IP connectivity point-to-point session. |
| Functional alias asked | O | The functional alias associated with the MCData identity of the MCData client that is required to establish an IP connectivity point-to-point session. |
| MCData ID targeted (NOTE 1) | O | The MCData identity of the MCData client that is the target of the requested IP connectivity point-to-point session. |
| Functional alias targeted (NOTE 1) | O | The functional alias associated with the MC MCData identity of the MCData client that is the target of the requested IP connectivity point-to-point session. |
| Requested Priority (NOTE 2) | O | Application priority level requested for this call. |
| Time Limit | O | Proposed time limit of the requested IP connectivity (1min- infinite). |
| Establishment reason | O | IP connectivity establishment reason |
| NOTE 1: Either the MCData ID or the functional alias of the targeted MCData user must be present. NOTE 2: The predefined priority of the MC service user is applied by the MCData server if the requested priority is not present or not accepted by the MCData server. |  |  |

##### 7.14.2.1.4	MCData remote IPcon point-to-point response

Table 7.14.2.1.4-1 describes the information content of the MCData remote IPcon point-to-point response as answer to MCData remote IPcon point-to-point request from the MCData client to the MCData server, from the MCData server to the MCData server and from the MCData server to the MCData client.

Table 7.14.2.1.4-1: MCData remote IPcon point-to-point response

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID asked | M | The MCData identity of the asked MCData client in the request message; |
| MCData ID targeted | M | The MCData identity of the targeted MCData client in the request message; |
| IP connectivity status | M | The status information about the IP connectivity session to the remote MCData user. |

##### 7.14.2.1.5	MCData remote IPcon point-to-point tear down request

Table 7.14.2.1.5-1 describes the information flow of the MCData remote IPcon point-to-point tear down request sent from the remote MCData client to the MCData server, from the MCData server to the MCData server and from the MCData server to the asked MCData client.

Table 7.14.2.1.5-1: MCData remote IPcon point-to-point tear down request

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID remote | M | The MCData identity of the remote MCData client that requests another MCData user to tear down an IP connectivity point-to-point session. |
| Functional alias remote | O | The associated functional alias of the remote MCData user; |
| MCData ID asked | M | The MCData identity of the MCData client that is asked to tear down an IP connectivity point-to-point session. |
| MCData ID targeted | M | The MCData identity of the MCData client that is the target to be tear down from the IP connectivity point-to-point session. |

##### 7.14.2.1.6	MCData remote IPcon point-to-point tear down response

Table 7.14.2.1.6-1 describes the information content of the MCData remote IPcon point-to-point tear down response as answer to MCData remote IPcon point-to-point tear down request from the MCData client to the MCData server, from the MCData server to the MCData server and from the MCData server to the MCData client.

Table 7.14.2.1.6-1: MCData remote IPcon point-to-point tear down response

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID asked | M | The MCData identity of the asked MCData client in the request message. |
| MCData ID targeted | M | The MCData identity of the targeted MCData client in the request message. |
| Tear down status | M | The status information about the IP connectivity tear down status information |

##### 7.14.2.1.7	MCData remote IPcon point-to-point application priority change request

Table 7.14.2.1.7-1 describes the information flow of the MCData remote IPcon point-to-point application priority change request sent from the remote MCData client to the MCData server, from the MCData server to the MCData server and from the MCData server to the asked MCData client.

Table 7.14.2.1.7-1: MCData remote IPcon point-to-point application priority change request

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID remote | M | The MCData identity of the remote MCData client that requests to change the application priority of an IP connectivity point-to-point session. |
| Functional alias remote | O | The associated functional alias of the remote MCData user; |
| MCData ID | M | The first MCData identity of the MCData client that is involved in the IP connectivity point-to-point session. |
| MCData ID | M | The second MCData identity of the MCData client that is involved in the IP connectivity point-to-point session. |
| Requested Priority | M | Contains the required application priority for the IP data communication between both MCData clients. |

##### 7.14.2.1.8	MCData remote IPcon point-to-point application priority change response

Table 7.14.2.1.8-1 describes the information content of the MCData remote IPcon point-to-point application priority change response as answer to MCData remote IPcon point-to-point application priority change request from the MCData client to the MCData server, from the MCData server to the MCData server and from the MCData server to the MCData client.

Table 7.14.2.1.8-1: MCData remote IPcon point-to-point application priority change response

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The MCData identity of the first MCData client involved in the IP connectivity point-to-point session. |
| MCData ID | M | The MCData identity of the second MCData client involved in the IP connectivity point-to-point session. |
| Requested priority change status | M | The status information about the application priority of the addressed IP connectivity session. |

* * * End of Changes * * * *