3GPP TSG-SA WG6 Meeting #39-e	S6-201340

# e-meeting, 31st August – 8th September 2020								(revision of S6-xxxxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0262 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification to location information cancel subscription |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BDBOS |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enh3MCPTT |  |  |  |  |  | Date: |  |  | 2020-08-11 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The current Location information cancel subscription request in clause 10.9.2.8 as well as Location information cancel subscription response in clause 10.9.2.9 do not differentiate between the subscribed MC service user and the requesting MC service user, so that all MC service users would be able to cancel a specific location information subscription. The current Location information cancel subscription procedure in clause 10.9.3.7 does not confirm the authorisation of Location information cancel subscription request described in clause 10.9.2.8. Adding more information to clause 10.9.2.8 and 10.9.2.9, in order to visualize the differences. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarification to the authorisation in case of a Location information cancel subscription request as well as more details. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | All MC service users would be able to cancel a specific location information subscription. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.9.2.8, 10.9.2.9, 10.9.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 10.9.2.8	Location information cancel subscription request

Table 10.9.2.8-1 describes the information flow from the MC service server to the location management server for location information cancel subscription request. The MC service server is able to request the cancellation of subscriptions associated to the MC service of the MC service server.

Table 10.9.2.8-1: Location information cancel subscription request (MC service server – LMS)

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID list | M | List of identities of the MC service users for whom location information is no longer required |

Table 10.9.2.8-2 describes the information flow from the location management client to the location management server for location information cancel subscription request. The location management client is able to request the cancellation of subscriptions to all known MC services of one or more MC service users.

Table 10.9.2.8-2: Location information cancel subscription request (LMC – LMS)

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID list (NOTE) | M | List of identities (e.g. MCPTT ID, MCData ID, MCVideo ID) of the requesting MC service user |
| MC service ID list | M | List of identities (e.g. MCPTT ID, MCData ID, MCVideo ID) of the MC service users for whom location information is no longer required |
| NOTE: Authorized or subscribed MC service user. |  |  |

* * * Next Change * * * *

#### 10.9.2.9	Location information cancel subscription response

Table 10.9.2.9-1 describes the information flow from the location management server to the MC service server for location information cancel subscription response. The location management server responds to the subscription cancellation request associated to the MC service of the MC service server.

Table 10.9.2.9-1: Location information cancel subscription response (LMS – MC service server)

| Information element | Status | Description |
| --- | --- | --- |
| Subscription status (NOTE) | M | Indicates the subscription result |
| NOTE: Only cancelled subscriptions listed. |  |  |

Table 10.9.2.9-2 describes the information flow from the location management server to the location management client for location information cancel subscription response. The location management server responds to the subscription cancellation request to all requested MC services of the listed MC service users.

Table 10.9.2.9-2: Location information cancel subscription response (LMS – LMC)

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID list | M | List of identities (e.g. MCPTT ID, MCData ID, MCVideo ID) of the requesting MC service user |
| Subscription status (NOTE) | M | Indicates the subscription result |
| NOTE: Only cancelled subscriptions listed. |  |  |

* * * Next Change * * * *

#### 10.9.3.7	Location information cancel subscription procedure

NOTE: This procedure is valid for single MC system operation only.

Figure 10.9.3.7-1 illustrates the high level procedure of location information cancel subscription request. The MC service server or the location management client can request the cancellation of location information subscriptions at any time from the location management server. The location management server is only stopping the forwarding of received location information to the MC service users of the cancellation request.

Figure 10.9.3.7-1: Location information cancel subscription request procedure

1.	MC service server or location management client sends a location information cancel subscription request to the location management server to cancel the subscription for location information of one or more MC service users.

2.	The location management server checks the authorization of this request.

3.	The location management server replies with a location information cancel subscription response indicating the cancel subscription status.

* * * End of Change * * * *