3GPP TSG-SA WG6 Meeting #40-e	S6-202319

e-meeting, 16th – 24th November 2020	(revision of S6-202197)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.286 | CR | 0035 | rev | 1 | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Relay the V2X traffic from Uu to PC5 broadcast |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eV2XAPP |  |  |  |  |  | Date: |  |  | 2020-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | A V2X UE can act as an application layer relay which supports unicast or multicast receiver and will broadcast packets across the PC5 link (V2V) to the UEs it serves. A common data flow configuration is established considering the feasibility for all UE for using Uu and application layer relay. Further V2X traffic is delivered as per the traffic filters established for the common data flow. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the procedures for enabling configuration related to the application layer relay and also the procedure for V2X traffic flows in this scenario. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Enable the V2X traffic message relaying and related configurations.. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.x, 9.x.1, 9.x.2, 9.x.2.1, 9.x.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 9.x	V2X traffic relay management

### 9.x.1	General

The VAE capabilities provide support for delivering the V2X traffic to a group of UEs (e.g., Platoon) or multiple UEs which should receive the same V2X traffic via a common data flow enabled via an application layer relay VAE client (e.g. V2X UE which acts as a Platoon leader). The application layer relay VAE client receives the V2X traffic via Uu and supports delivery of the V2X traffic to the other VAE clients it serves, over PC5.

### 9.x.2	Procedure

### 9.x.2.1	Establish a common data flow configuration

Figure 9.x.2.1-1 illustrates the procedure where the VAE server selects the VAE client to relay the V2X traffic received via Uu to the remote VAE clients via PC5 by establishing a common data flow configuration at the application layer relay VAE client.

Pre-conditions:

1.	The VAE clients 2..N have already connected to the application layer relay VAE client.

2.	The VAE clients 2..N and the application layer relay VAE client may belong to the same group.

Figure 9.x2.1-1: Procedure for establishing the common data flow

1.	The VAE clients in the coverage performs the registration or registration update to the VAE server. The V2X UE ID, UE location, relay supporting, communicating range, numbers of connections or remote UE being supported, etc are included in the registration request towards the VAE server. An application layer relay VAE client provides the above information for the VAE clients it serves and in addition includes the number of VAE clients it serves, the locations of the VAE clients, the V2X services requested by the VAE clients.

2.	The VAE server determines whether MBMS delivery can be used based on e.g. the number of the VAE clients in coverage (which includes the VAE clients served by the application layer VAE client) satisfies the threshold to activate the MBMS. If MBMS is used then the VAE server activates the MBMS bearer or uses the pre-established MBMS bearer, if any. The VAE server determines (e.g. the VAE clients 2..N served by the application layer relay VAE client have requested the same V2X service and have reached a certain threshold) to establish a common data flow for the VAE clients 2..N requesting the same V2X service.

3.	The VAE server sends the common data flow establish request to the application layer relay VAE client(s) to negotiate the common data flow used for delivering the V2X traffic to the VAE clients 2..N. The traffic filter (e.g. target multicast address and port, source IP address and port) which indicates the V2X traffic to be relayed to the VAE clients 2..N is included. In addition, the frequency or re-transmit number for the V2X traffic may also be included.

4.	The application layer relay VAE client stores the traffic filter configuration. The application layer relay VAE client also informs the VAE clients 2..N about the V2X traffic information (e.g. target multicast address and port, source IP address and port), so that the VAE clients 2..N associates the received V2X traffic to the requested V2X service.

5.	The application layer relay VAE client provides the common data flow establish response to the VAE server to confirm the successful or failure of establishing the common data flow.

#### 9.x.2.2	Relay the V2X traffic

Figure 9.x.2.2-1 illustrates the procedure where the application layer relay VAE client relays the V2X traffic to the VAE clients 2..N via the common data flow established in clause 9.x.2.1.

Pre-conditions:

1.	The VAE clients 2..N have already connected to the application layer relay VAE clients.

2.	The application layer relay VAE client and the VAE server have already established the common data flow as described in clause 9.x.2.1.

Figure 9.x2.2-1: Procedure for relaying the V2X traffic

1.	The V2X traffic is ready at the VAE server either from other VAE client 1 or locally.

2.	The VAE server sends the V2X traffic via the common data flow towards the application layer relay VAE client.

3.	The application layer relay VAE client sends the V2X traffic to the VAE clients 2..N via PC5 based on the traffic filter. The frequency or the repeater-transmit number is applied, if available.

* * * End of all the Changes * * * *