3GPP TSG-SA WG6 Meeting #42-bis-e	S6-210792

e-meeting, 12th – 20th April 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0049 | rev | 1 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to GMS APIs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TD-Tech |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSEAL |  |  |  |  |  | Date: |  |  | 2021-04-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The API operation definitions for group creation as specified in clause 10.3.2.1 and location-based group creation as specified in clause 10.3.2.24 is required. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the API operations and the missing information flows |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | These API operations will not be available. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.3.2.34, 10.3.2.x (new), 10.4.1, 10.4.2.x (new), 10.4.2.y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

10.3.2.34 Location-based group creation request

Table 10.3.2.34-1 describes the information flow location-based group creation request from the group management client or VAL server to the group management server.

Table 10.3.2.34-1: location-based group creation request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the group management client performing the request. |
| Location criteria | M | Criteria to combine Users or UEs in a location. |
| VAL service ID list (see NOTE 1) | O | List of VAL services whose service communications are to be enabled on the group. |
| VAL service specific information (NOTE 2) | O | Placeholder for VAL service specific information |
| NOTE 1: This information element shall be included in the message for creating a group configured for multiple VAL services. NOTE 2: The details of this information element are specified in VAL service specific specification and are out of scope of the present document. |  |  |

10.3.2.x	Location-based group creation response

Table 10.3.2.x-1 describes the information flow location-based group creation response from the group management server to the group management client or VAL server.

Table 10.3.2.x-1: Location-based group creation response

| Information element | Status | Description |
| --- | --- | --- |
| VAL group ID | M (NOTE) | VAL group ID of the group |
| Result | M | Indicates the success or failure for the operation |
| NOTE: If the Result information element indicates failure then the value of VAL group ID information element has no meaning. |  |  |

* * * Next Change * * * *

10.4.1	General

Table 10.4.1-1 illustrates the SEAL APIs for group management.

Table 10.4.1-1: List of SEAL APIs for group management

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_GroupManagement | Query_Group_Info | VAL server | Request /Response |
|  | Create_Group_Info | VAL server | Request/Response |
|  | Create_LocationBasedGroup_Info | VAL server | Request/Response |
|  | Update_Group_Info | VAL server | Request /Response |
| SS_GroupManagementEvent | Subscribe_ Group_Info_Modification | VAL server | Subscribe/Notify |
|  | Notify_Group_Info_Modification | VAL server |  |
|  | Notify_Group_Creation | VAL server |  |

* * * Next Change * * * *

10.4.2.x	Create_Group_Info operation

API operation name: Create_Group_Info

Description: Create group information

Known Consumers: VAL server.

Inputs: See subclause 10.3.2.1

Outputs: See subclause 10.3.2.2

See subclause 10.3.3 for the details of usage of this API operation.

10.4.2.y	Create_LocationBasedGroup_Info operation

API operation name: Create_LocationBasedGroup_Info

Description: Create location-based group

Known Consumers: VAL server.

Inputs: See subclause 10.3.2.24

Outputs: See subclause 10.3.2.x

See subclause 10.3.7 for the details of usage of this API operation.