3GPP TSG-SA WG6 Meeting #49BIS-e	S6-221905

e-meeting, 22nd June – 1st July 2022	(revision of S6-220706, 909, 1210, 1728)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0098 | rev | 4 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Sharing location information across VAL servers |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Kyonggi University |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSEAL2 |  |  |  |  |  | Date: |  |  | 2022-06-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | A general vertical use case is illustrated as follows: 1. A IoT policy server (VAL server) or IoT administrator (Location management client) has relationship with several other IoT-App servers (VAL servers). 2. The IoT-App server has the capability to consume location reports via APIs provided by SEAL LM server. 3. The IoT policy server/admin coordinates the API consumption behavior of IoT-App servers. 4. The IoT policy server/admin sets up location sharing policy with the SEAL LM server (identifying which IoT-App servers can be distributed/shared the same location reports) 5. The IoT-App servers subscribe for location reports from SEAL LMS server. The SEAL LM server performs location reporting configuration optimization, considering the location sharing policy from IoT policy server, if provided. 6. A SEAL LM client sends location report to SEAL LM server. 7. The SEAL LM server distributes/shares the location reports as per location sharing policy, if exists. So, this CR proposes an enhanced solution to optimize the location reporting considering the location sharing policy. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduced a location sharing policy information to allow multiple VAL server to share the same location from location management client and a single subscription between the LMS and LMC for multiple VAL services/server. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | New feature for enabling location sharing amongst VAL servers as per the location sharing policy will not be enabled which is a use case for IoT platform. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.2.x (new), 9.3.2.y (new), 9.3.x (new), 9.3.5, 9.3.7, 9.3.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 9.3.2.x	Location sharing policy provisioning request

Table 9.3.2.x-1 describes the information flow for location sharing policy provisioning request from the VAL server or location management client to the location management server.

Table 9.3.2.x-1: Location sharing policy provisioning request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL server/VAL user or VAL UE |
| VAL service ID | O | Identity of the VAL service for which the location location sharing policy applies. |
| Location sharing policy information | O | The information related to location sharing policy |
| > List of VAL UE/user identities | M | List of VAL users or VAL UEs whose location information is requested. |
| > List of VAL service IDs |  | A list of VAL service IDs with whom the location of the VAL UE/users is allowed to be shared. |

#### 9.3.2.y	Location sharing policy provisioning response

Table 9.3.2.y-1 describes the information flow for location sharing policy provisioning response from the location management server to the VAL server or location management client.

Table 9.3.2.y-1: Location sharing policy provisioning response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Success or failure of the location sharing policy provisioning request |

* * * First Change * * * *

### 9.3.x	Location sharing policy provisioning procedure

Figure 9.3.x-1 illustrates the procedure for location sharing policy provisioning by VAL server or location management client on location management server.

Pre-conditions:

1.	The VAL server/location management client can communicate with location management server.

Figure 9.3.x-1: Location sharing policy provisioning procedure

1.	The VAL server or location management client sends a location sharing policy provisioning request to the location management server with the location sharing policy information corresponding to a VAL service.

2.	Location management server checks whether location management client or VAL server is authorized to send a location sharing policy provisioning request. If authorized, it stores the location sharing policy.

3.	The location management server sends a successful or failure response to the VAL server or the location management client.

* * * Next Change * * * *

### 9.3.5	Client-triggered or VAL server-triggered location reporting procedure

Figure 9.3.5-1 illustrates the high level procedure of client-triggered or VAL server-triggered location reporting.

Pre-conditions:

1.	The location sharing policy is available at the location management server if the UE location can be shared among multiple VAL services.

Figure 9.3.5-1: Client-triggered location reporting procedure

1.	Location management client 2 (authorized VAL user or VAL UE) or VAL server sends a location reporting trigger to the location management server to activate a location reporting procedure for obtaining the location information of location management client 1.

NOTE:	Step 1 can be performed when Location management client 2 or VAL server require to update the location reporting trigger.

2.	Location management server checks whether location management client 2 or VAL server is authorized to send a location reporting trigger. Depending on the information specified by the location reporting trigger, location management server initiates an on-demand location reporting procedure or an event-triggered location reporting procedure for the location of location management client 1.

If it is allowed to share the location information among multiple VAL services based on the location sharing policy, the location management server reuses the existing location reporting configuration towards the location management client and and updates the location reporting configuration to incorporate the triggers provided in step 1, if required (e.g. updating the minimum time between consecutive reports with the shorter interval).

3.	Once the location information of the location management client 1 is available in the location management server by the on-demand location reporting procedure, a location information report is sent to the location management client 2 or VAL server. If multiple VAL servers request location from the location management client, the location management server sends the location information available at the location management server to the VAL servers with whom the location information can be shared as per the location sharing policy and as per the triggers for the VAL servers.

* * * Next Change * * * *

### 9.3.7	Location information subscription procedure

Figure 9.3.7-1 illustrates the high level procedure of location information subscription request. The same procedure can be applied for location management client and other entities that would like to subscribe to VAL user or VAL UE location information. This procedure is also used for intiating tracking a UE's location.

Pre-conditions:

1.	The location sharing policy is available at the location management server if the UE location can be shared among multiple service.

Figure 9.3.7-1: Location information subscription request procedure

1.	The VAL server sends a location information subscription request to the location management server to subscribe location information of one or more VAL users or VAL UEs. The request may include an indication for supplementary location information.

2.	The location management server shall check if the VAL server is authorized to initiate the location information subscription request. Further, the location management server may initiate location reporting configuration with the location management client of the UE for immediate reporting.

If it is allowed to share the location information among multiple services based on the location sharing policy, the location management server reuses the existing location reporting configuration towards the location management client and and updates the location reporting configuration to incorporate the triggers provided in step 1, if required (e.g. updating the minimum time between consecutive reports with the shorter interval).

3.	The location management server may optionally subscribe for UE location information from 3GPP core network for the UE. If the indication for supplementary location information is included in step 1, then UE location information is obtained from the 3GPP core network.

4.	The location management server determines the UE location information of the UE as received in steps 3 and 4.

5.	The location management server replies with a location information subscription response indicating the subscription status and if immediate reporting was requested, the location information of the VAL UE(s).

### 9.3.8	Event-trigger location information notification procedure

Figure 9.3.8-1 illustrates the high level procedure of event-trigger usage of location information. The same procedure can be applied for location management client and other entities that would like to subscribe to location information of VAL user or VAL UE. This procedure is also used for obtaining latest UE's location for tracking purpose.

Pre-conditions:

1.	The location sharing policy is available at the location management server if the UE location can be shared among multiple service.

Figure 9.3.8-1: Event-trigger usage of location information procedure

1.	The location management server receives the latest location information of the UE as per the location report procedure described in clause 9.3.3.3.

2.	The location management server may optionally receive the location information of the UE from 3GPP core network. If the indication for supplementary location information is included in the subscription, then UE location information is obtained from the 3GPP core network.

3.	Based on the configurations, e.g., subscription, periodical location information timer, location management server is triggered to report the latest user location information to VAL server. The location management server determines the location information of UE as received in steps 1 and 2, including the supplementary location information (if indicated).

4.	The location management server sends the location information report including the latest location information of one or more VAL users or VAL UEs to the VAL server or to the location management client that has previously configured.

If the VAL server is allowed to receive the location information as per the location sharing policy information, then the location management server utilizes the location information from the location management client which commonly applies for the VAL servers as per the location sharing policy information.

5.	VAL server may further share this location information to a group or to another VAL user or VAL UE.

NOTE:	For other entities, the step 5 can be skipped if not needed.

* * * End of Change * * * *