3GPP TSG-SA6 Meeting #51-e	S6-222860

# Online, , 10th Oct 2022 - 19th Oct 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0135 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SEAL DPM functional architecture |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | eSEAL |  |  |  |  |  | Date: |  |  | 2022-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Data preparation is an essential step in ML model liefcycle and is responsible for (i) understanding the characteristics of data, i.e., collecting information about the data, e.g., type of data, range, etc., (ii) determining if the data suffers from quality issues, e.g., errors or missing values, and dealing with them and (iii) formatting and labelling data, preparing also the data set e.g. for training purposes. This CR proposes to consider adding a new SEAL service for Data Preparation Management (DPM). The DPM layer may include the DPM server (configures DP and performs preparation accordingly), and the DPM client which can be configured to prepare UE data (from apps, SEAL clients, etc). DPM layer may have tasks related to configuration and data preparation for diverse data (e.g. application, network, user data) based on the vertical customer requirements |  |  |  |  |  |  |  |  |
| Summary of change: |  | The CR introduces the DPM layer functional architecture in clause X, and presents the functional model, entities and reference points. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Data preparation management feature is not specified (up to implementation) and is challenging to ensure that application and network data collected at SEAL servers are of sufficient quality. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * * * * FIRST CHANGE * * * * * * *

# X	Data Preparation Management service

## X.1	General

The Data Preparation Management is a SEAL service that offers data management services such as i) understanding the characteristics of data, i.e., collecting information about the data, e.g., type of data, range, etc., ii) determining if the data suffers from quality issues, e.g., errors or missing values, and dealing with them and iii) formatting and labelling data, preparing also the data set e.g. for training purposes.

## X.2	Functional model

### X.2.1	General

The functional model for the data preparation management is based on the generic functional model specified in clause 6.2. It is organized into functional entities to describe a functional architecture which addresses the support for data preparation management aspects for vertical applications.

### X.2.2	Functional model description

Figure X.2.2-1 illustrates the generic functional model for data preparation management.

Figure X.2.2-1: Functional model for data preparation management service

The Data Preparation Management client communicates with the Data Preparation Management server over the DPM-UU reference point. The Data Preparation Management client provides the support for Data Preparation Management functions to the VAL client(s) over DPM-C reference point. The VAL server(s) communicates with the Data Preparation Management server over the DPM-S reference point. The Data Preparation Management server may communicate also with other SEAL services over SEAL-X. The Data Preparation Management server, acting as AF, may communicate with the 5G Core Network functions via NEF (N33) reference point, and with the OAM system via DPM-OAM.

### X.2.3	Functional entities description

#### X.2.3.1 General

The functional entities for Data Preparation Management SEAL service are described in the following subclauses.

#### X.2.3.2 Data Preparation Management server

The Data Preparation Management server performs the configuration of data preparation based on the consumer (e.g. VAL server or SEAL server) requirements, which includes the configuration of the data analysis and recovery required (e.g. algorithms, configuration parameters). The Data Preparation Management server may also include the data preparation tasks (based on the deployment). These tasks fall into the following main categories: i) data collection and analysis to identify irregularities, ii) data recovery and cleaning considering (a) systematic errors involving large data records from different data sources and/or (b) individual data errors due to random or processing errors, iii) data formatting and iv) data labelling and separation into sets for accommodating different training tasks.

#### X.2.3.3 Data Preparation Management client

The Data Preparation Management client receives configuration of the data preparation for data related to the VAL UE and may perform preparation accordingly.

### X.2.4	Reference points description

#### X.2.4.1	General

The reference points for the functional model for Data Preparation Management are described in the following subclauses.

#### X.2.4.2	DPM-UU

The interactions related to Data Preparation Management functions between the Data Preparation Management server and the Data Preparation Management client are supported by DPM-UU reference point. This reference point utilizes Uu reference point as described in 3GPP TS 23.501 [10].

#### X.2.4.3	DPM-C

The interactions related to Data Preparation Management functions between the VAL client(s) and the Data Preparation Management client within a VAL UE are supported by the DPM-C reference point.

#### X.2.4.4	DPM-S

The interactions related to Data Preparation Management functions between the VAL server(s) and the Data Preparation Management server are supported by the DPM-S reference point. This reference point is an instance of CAPIF-2 reference point as specified in 3GPP TS 23.222 [8].

#### X.2.4.5	N33

The reference point N33 supports the interactions between the Data Preparation Management server and the NEF and is specified in 3GPP TS 23.501 [10].

#### X.2.4.6	DPM-OAM

The reference point DPM-OAM supports the interactions between the Data Preparation Management server and the OAM services, e.g. PM MnS.

## X.3	Procedures and information flows for Data Preparation Management

FFS