3GPP TSG-SA WG6 Meeting #52	S6-223351

Toulouse, France 14th – 18th November 2022	(revision of S6-223268)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.379 | CR | 0328 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Ad hoc group call procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MC_AHGC (Approval at SA#98e) |  |  |  |  |  | Date: |  |  | 2022-11-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The Ad hoc group communication has been studied in TR 23.700-76 and for the ad hoc group communication procedures are agreed on the basis of solution#2 with the following conclusions: i. Information flows specific to Ad hoc group calls will be used. ii. Pre-configured group as specified in Solution #2 to be used for achieving the end-to-end security for both the cases where the participants list is supplied by the initiator of the AHGC and participants list is determined by the MC Service server. iii. Pre-configured group to be used for the AHGC is determined by the MC Service server and not by the initiating client. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New procedures for the case where participants list is supplied by the initiator is proposed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ad hoc group call feature will not be available. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.19.3 (new), 10.19.3.2 (new), 10.19.3.2.1 (new), 10.19.3.2.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 10.19.3	Procedures

#### 10.19.3.2	Ad hoc group call involving multiple MC systems

##### 10.19.3.2.1	Procedure for ad hoc group call setup – Participants list provided by the Initiator

Figure 10.19.3.2.1-1 illustrates the procedure for ad hoc group call setup procedure.

Pre-conditions:

1.	The security aspects of sharing the user information between primary and partner MC systems shall be governed as per the service provider agreement between them. In this case, it is considered that the partner MC system does not share their users' information to the primary MC system.

2.	The authorized MC service user/dispatcher belongs to the primary MC system.

3.	The MC service server of the primary MC system is where the authorized MC service user/dispatcher creates the ad hoc group.

4.	Some users of the ad hoc group may belong to partner MC systems.

5.	The pre-configured group identity and pre-configured group configuration to be used for an ad hoc group have been preconfigured in MC service client and other participants of ad hoc group have also received the relevant security related information to allow them to communicate in an ad hoc group communication.

Figure 10.19.3.2.1-1: Ad hoc group call setup

1.	The MC service client of authorized user initiates an ad hoc group call with multiple users from primary and partner MC systems. An ad hoc group call request message with the information of the MC service IDs is routed to the MC service server of the primary MC system.

2.	The MC service server of the primary MC system forms the ad hoc group by using MC service users' information and determines the configuration of the preconfigured ad hoc group for the participants information received in step 1 and assigns a MC service group ID for the newly formed ad hoc group. It identifies the appropriate MC service server responsible for the MC service users of the ad hoc group. The MC service users are automatically affiliated to the ad hoc group.

NOTE 1:	The newly formed ad hoc group information including the MC service group ID and the list of users is held in dynamic data in the MC service server.

3.	The MC service server of the primary MC system sends the ad hoc group call request to the affiliated group members of the ad hoc group belonging to the primary MC system.

4.	The MC service clients receive in the ad hoc group call request the information of the MC service group ID for the ad hoc group and further notify their corresponding MC service user. The affiliated group members of the ad hoc group of the primary MC system may accept or reject the call and respond with the group call response.

5.	The primary MC service server further initiates an ad hoc group call request message to the MC service server of the partner MC system for the ad hoc group's MC service users' belonging to partner MC system.

6.	Upon receiving the ad hoc group call request message from the MC service server of the primary MC system, the MC service server of the partner MC system initiates a call invitation to its MC service users and sends the ad hoc group call request to the MC service users of the partner MC system.

7.	The MC service clients receive in the ad hoc group call request the information of the MC service group ID for the ad hoc group and further notify their corresponding MC service user. The MC service users upon receipt of the invitation may accept or reject the call, and respond with the ad hoc group call response.

8.	The MC service server of the partner MC system provides an ad hoc group call response message to the MC service server of the primary MC system with success or failure result and/or detailed reason information in case of failure.

9.	The MC service server of the primary MC system provides an ad hoc group call response message to the MC service client of the authorized MC service user upon receiving response to the corresponding ad hoc group call request in step 1. The ad hoc group call response will consist of the success or failure result and/or detailed reason information in case of failure. The MC service client 1 receives the MC service group ID assigned for the ad hoc group and the group ID of the pre-configured group to be used for the ad hoc group call via this step.

NOTE 2:	The ad hoc group call response message is triggered depending on the conditions to proceed with the call.

10.	Upon successful ad hoc group call setup, a group call is established amongst the multiple group members from primary and partner MC systems. The media plane and floor control resources are established.

##### 10.19.3.2.2	Procedure for ad hoc group call release by MC service server

This procedure focuses on the case where an MC service server initiates the termination of an ongoing MC service ad hoc group call for all the participants of that group call, since at least one of the termination conditions are met e.g., due to hang time expiry, last participant leaving, second last participant leaving, initiator leaving, or minimum number of affiliated MC service group members are not present.

Procedures in figure 10.19.3.2.2-1 are the signalling control plane procedures for the MC service server initiating termination of an ongoing MC service ad hoc group call.

Figure 10.19.3.2.2-1: Ad hoc group call release

1.	It is assumed that MC service users on MC service client 1, client 2 belonging to primary MC system and client 3 belonging to partner MC system are already part of the ongoing ad hoc group call (e.g. as a result of ad hoc group call setup as specified in clause 10.19.3.2.1).

2.	The MC service server would like to release the ad hoc group call which is ongoing e.g., due to hang time expiry, last participant leaving, second last participant leaving, initiator leaving, or minimum number of affiliated MC service group members are not present.

3.	The MC service server identifies the participants of the ongoing ad hoc group call and generates ad hoc group call release request to release ongoing session.

4.	The MC service server sends a ad hoc group call release request via SIP core towards each participant of the ongoing group call. If the participants belong to partner MC system, then ad hoc group call release request is sent to the partner MC service server, which further sends the ad hoc group call release request to the MC service clients.

5.	The MC service users are notified about the release of the ad hoc group call.

6.	The MC service client(s) receiving ad hoc group call release request, send acknowledgement towards the MC service server by sending a ad hoc group call release response.

7.	The MC service client 1, client 2 and client 3 have successfully released the floor control and media plane resources associated with the ad hoc group call that is terminated. The MC service servers remove the ad hoc group information from the dynamic data held in the MC service servers and thus the ad hoc group ceases to exist.