3GPP TSG-SA WG6 Meeting #53	S6-230667

Athens, Greece 27th February – 3rd March 2023	(revision of S6-230342, S6-230232)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0204 | rev | 2 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | ACR Scenario selection by EEC for T-EES and T-EAS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-01-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Clause 8.15 specifies procedure to select supported ACR scenario list based on AC, EEC, EES and EAS. However, when UE moves to new location, due to service continuity procedures, EEC may connect to new EES (i.e. T-EES) and AC may get service from new EAS (i.e. T-EAS). It is possible that both S-EES and T-EES are from differnet service providers and may not support exactly same ACR scenario list. Similarly, It is possible that both S-EAS and T-EAS are from differnet service providers and may not support exactly same ACR scenario list. In such case, it is required to select supported ACR scenario list again based on T-EES and T-EAS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added a clarification for EEC to either select ACR scenarios based on T-EES and T-EAS supported scenarios or request T-EES to select ACR sccenarios. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | ACR scenarios will not be selected for T-EEs and T-EAS and so T-EES or T-EAS will not be aware of ACR detection and execution. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.8.3.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Related to KI#19, Sol #38 of 3GPP TR 23.700-98 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 8.8.3.5.3	Notify

Figure 8.8.3.5.3-1 illustrates the ACR information notification procedure between the EEC and the EES, which can be used by the EES to notify the EEC of the following:

-	target information, i.e. the details of the selected T-EAS and, if required, the selected T-EES, during ACR as described in clauses 8.8.2.4 and 8.8.2.5;

NOTE:	The T-EAS and T-EES information can be used to determine the PDU session(s) to provide connectivity to the T-EAS and the T-EES. If the ACR does not require change in EES, i.e. T-EES is same as S-EES, then the T-EES information can be skipped.

-	ACR complete events.

Pre-conditions:

1.	The EEC has subscribed with the EES for the ACR information as specified in clause 8.8.3.5.2.

Figure 8.8.3.5.3-1: ACR information notification

1.	An event (e.g. ACR complete, or Target information notification) occurs at the EES that satisfies trigger conditions for providing ACR information to a subscribed EEC.

2.	The EES sends an ACR information notification to the EEC with the ACR information determined in step 1. The ACR information notification may include ACID to indicate the application context relocation of the AC is complete. If the S-EES has received the successful EEC Context Push response from T-EES, along with registration ID and the registration expiration time in the EEC Context Push relocation procedure, then the ACR information notification towards EEC also includes the registration ID and registration expiration time under EEC context relocation status (for successful status). Upon receiving the target information notification to indicate start of the ACR execution, the EEC avoids triggering a second ACR execution for the same ACR identity (ACID, UE ID, S-EAS endpoint and T-EAS endpoint) until the current ACR execution is completed. For ACR scenarios as specified in clause 8.8.2.2, clause 8.8.2.3 and clause 8.8.2.6, upon receiving the ACR complete notification, if the selected ACR scenario list is not available, the EEC may either select ACR scenario considering the supported ACR scenarios of AC, EEC, T-EES and T-EAS or request T-EES to select list of ACR scenarios as specified in clause 8.15.

Editor's note: In order to start detect or decide, till how long the detection entity should wait for current ACR to complete, is FFS

* * * Next Change * * * *

#### 8.8.4.10	ACR information notification

Table 8.8.4.10-1 describes the information elements for ACR information notification from the EES to the EEC.

Table 8.8.4.10-1: ACR information notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Subscription identifier corresponding to the subscription stored in the EES for the request |
| EASID | M | The identifier of the EAS |
| ACID | O | The identifier of the AC corresponding to the Selected target EAS |
| Event ID | M | Either Target information notification or ACR complete |
| Target information (NOTE 1) | O | Details of the selected T-EAS and the T-EES. |
| > T-EAS information | M | Details of the selected T-EAS as described in 'Discovered EAS' IE of Table 8.5.3.3-1. |
| > T-EES information (NOTE 4) | O | Details of the selected T-EES as described in 'EDN configuration information' IE of Table 8.3.3.3.3-1. |
| ACR complete event information (NOTE 2) | O | Details of a completed ACR and its result. |
| Result of ACR (NOTE 2) | M | Indicates whether the ACR is successful or failure |
| > T-EAS endpoint | M | Endpoint address of the T-EAS to which an ACR has been performed. |
| EEC Context Relocation status (NOTE 5) | O | Indicates whether the EEC context relocation was successful or not. |
| > Registration Id (see NOTE 6) | O | Identifier of the registration for the EEC. |
| > Expiration Time (see NOTE 6) | O | Indicates the expiration time of the registration. |
| > Selected ACR scenario list (see NOTE 6) | O | The list of ACR scenarios selected by the T-EES |
| Cause information (see NOTE 3) | O | Indicates the cause information for the failure |
| NOTE 1: This IE shall be included when Event ID indicates 'Target information notification' event NOTE 2: This IE shall be included when Event ID indicates 'ACR complete' event NOTE 3: This IE shall be included when the Result of ACR indicates failure. NOTE 4: This IE shall be included if the selected T-EES is different from the S-EES. Otherwise, it may be skipped. NOTE 5: This IE shall be included when Event ID indicates 'ACR complete' event and EEC context relocation was attempted. NOTE 6: This IE shall be included if S-EES has received it in EEC Context Push response. |  |  |

* * * Next Change * * * *

#### 8.9.3.4	EEC Context Push request

Table 8.9.3.4-1 describes information elements in the EEC Context Push request between two EESs.

Table 8.9.3.4-1: EEC Context Push request

| Information element | Status | Description |
| --- | --- | --- |
| EES ID | M | Unique identifier of the requesting EES. |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| EEC Context | M | EEC Context |
| ACID | O | The identifier of the AC. |
| EAS ID | O | The identifier of the T-EAS |
| ACR scenario list | O | The list of ACR scenarios currently selected for the given AC and S-EAS pair |
| ACR scenario selection request | O | Indicates T-EES to select the ACR scenario. |

#### 8.9.3.5	EEC Context Push response

Table 8.9.3.5-1 describes information elements in the EEC Context Push request between two EES.

Table 8.9.3.5-1: EEC Context Push response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the request was successful. |
| > registration ID (see NOTE 1) | O | Identifier of the registration for the EEC |
| > expiration time (see NOTE 1) | O | Indicates the expiration time of the EEC registration. |
| > Selected ACR scenario list (see NOTE 2) | O | The list of ACR scenarios selected by the T-EES |
| Failure response | O | Indicates that the request failed. |
| > Cause | O | Indicates the cause of request failure, mandatory if the request failed. |
| NOTE 1: This IE shall be included if implicit registration is performed by T-EES. NOTE 2: Only if request includes “ACR scenario selection request” |  |  |

* * * End of Change * * * *