3GPP TSG-SA6 Meeting #55	S6-231682

# Berlin, Germany, 22nd May 2023 - 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0353 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Modification to AC profile |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Add new elements relevant to EEC information and EAS endpoint in AC profile. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Complete the AC profile by adding new elements to include EEC information and EAS endpoint. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The AC may have trouble communicating with EEC without the information of EEC. - The AC may have trouble establishing contact with the EAS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.2. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * *First Change * * * *

8.2.2	AC Profile

An AC Profile includes information about AC used to determine services and service characteristics required.

Table 8.2.2-1: AC Profile

| Information element | Status | Description |
| --- | --- | --- |
| ACID | M | Identity of the AC. |
| AC Type | O | The category or type of AC (e.g. V2X). This is an implementation specific value. |
| Preferred ECSP list | O | When used in a service provisioning request, this IE indicates to the ECS which ECSPs are preferred for the AC. The ECS may use this information in the selection of EESs. |
| AC Schedule | O | The expected operation schedule of the AC (e.g. time windows) |
| Expected AC Geographical Service Area | O | The expected location(s) (e.g. route) of the hosting UE during the AC's operation schedule. This geographic information can express a geographic point, polygon, route, signalling map, or waypoint set. |
| AC Service Continuity Support | O | Indicates if service continuity support is required or not for the application. The IE indicates which ACR scenarios are supported by the AC and which of these are preferred by the AC, also indicates the AC ability of handling bundled EAS ACR. |
| EEC ID | M | Identifies the EEC that can communicate with the AC. |
| Simultaneous EAS connectivity information in service continuity | O | Indicates if simultaneous EAS connectivity is needed and the inactive time guidance for keeping connectivity towards the S-EAS. |
| List of EASs | O | List of EAS that serve the AC along with the service KPIs required by the AC |
| > EASID | M | Identifier of the EAS |
| > EAS Endpoint | M | Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the EAS. This information maybe discovered by EEC and exposed to ACs so that ACs can establish contact with the EAS. |
| > Expected AC Service KPIs | O | KPIs expected in order for ACs to receive currently required services from the EAS, as described in Table 8.2.3-1 |
| > Minimum required AC Service KPIs | O | Minimum KPIs required in order for ACs to receive meaningful services from the EAS, as described in Table 8.2.3-1 |
| List of required EAS bundle information | O | Information related to the EAS bundles which AC requires. |
| > Bundle ID or list of EASID | M | A list of EASIDs or a bundle ID as described in clause 7.2.10. |
| > EAS bundle requirements | O | Requirements associated with the EAS bundle as described in clause 8.2.10. |

* * * End of Changes * * * *