3GPP TSG-SA WG6 Meeting #55	S6-231706

Berlin, Germany 22nd – 26th May 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0356 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | EEC Triggering execution |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA6 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Samsung |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-05-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The EEC Triggering execution procedure is missing in the TS. Also, the following EN on how to decide port ID for triggering during the execution needs to be resolved. “Editor's Note: It is FFS whether and how the ECS and EES determine the parameters (e.g., port ID) necessary for EEC triggering service or if configuration is sufficient.” The ECS and EES can determine the port ID for EEC triggering service based on the local configuration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add EEC Triggering execution procedure Remove EN on how to decide port ID for EEC triggering and add a NOTE on the port ID determination |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification remains uncomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.16.1, 8.16.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is related to KI#1/Sol#3 in 23.700-98 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

8.16.1	General

The functional entities of the Edge Enabler Layer can utilize the Application Triggering services (or Device Triggering) provided by the 3GPP core network via NEF or SCEF (see clause 4.4.5 in 3GPP TS 23.501 [2], clause 4.13.2, 5.2.6.5.2 in TS 23.502 [3], and clause 4.5.1 TS 23.682 [17]) to enable for ECS or EES to perform EEC Triggering, i.e., to trigger EEC to perform the procedures over EDGE-1 or EDGE-4 (e.g., service provisioning or EAS discovery).

Editor's Note:	It is FFS whether and how to describe the detailed EEC operation checking if EEC triggering service via Application Triggering or Device Triggering mechanism is available.

The EEC may indicate to the ECS or EES that the EEC triggering service is needed.

The triggering mechanism is used by the ECS and EES for messaging to EEC by invoking application triggering service as specified for Nnef_Trigger_Delivery service in clause 4.13.2 of TS 23.502 [3] or the device triggering procedure via T8 in clause 5.17 of TS 23.682 [17]. The ECS can use it to trigger the EEC to initiate service provisioning request (as per clause 8.3.3.2.2) when the EDN configuration information in the ECS is updated. The EES can use it to trigger the EEC to initiate EAS discovery request (as per clause 8.5.2.2) when the EAS profile in the EES is updated.

Editor's Note:	It is FFS how to describe the EEC triggering procedure according to the decision on either having it as a separate procedure or updating the existing procedure.

NOTE:	The ECS and EES can determine the port ID for EEC triggering service based on the local configuration.

When ECS or EES performs EEC triggering service by invoking the application triggering service, the ECS or EES includes, in the Trigger Payload, Triggering Entity Information (identifier and endpoint address of the triggering EES or ECS) and optionally Trigger Description, which indicates the procedure (e.g., service provisioning or EAS discovery) to be initiated by the EEC. Upon receiving the Trigger Payload, the EEC selects which procedure needs to be performed according to the received Trigger Description and initiates the selected procedure towards either EES or ECS indicated by the Triggering Entity Information in the Trigger Payload.

Editor's Note:	Optionality of Trigger Description is FFS.

* * * Next Change * * * *

8.16.x	Procedure

8.16.x.1	EEC Triggering execution

Figure 8.16.x.1-1 illustrates the procedure of EEC Triggering from EES or ECS.

Pre-conditions:

1.	The EES and ECS supports to check if it is authorized to use Device Triggering procedure (see TS 23.682 [17]) or Application Triggering service (see TS 23.502 [3]).

Figure 8.16.x.1-1: EEC Triggering execution

1.	The EEC performs the EEC Triggering setup by sending EEC Triggering Service information to the ECS or EES during Service Provisioning or EAS discovery procedures as described in clause 8.3.3 and 8.5.2.

2.	Once the EEC triggering is met in EES or ECS, the EES or ECS performs the invocation of Application Triggering service as described in TS 23.502 [3] or Device Triggering procedure as described in TS 23.682 [17] in order to provide the Trigger Payload to be provided to the EEC.

The ECS or EES includes, in the Trigger Payload, Triggering Entity Information (identifier and endpoint address of the triggering EES or ECS) and optionally Trigger Description, which indicates the procedure (e.g., service provisioning or EAS discovery) to be initiated by the EEC. Upon receiving the Trigger Payload, the EEC selects which procedure needs to be performed according to the received Trigger Description and initiates the selected procedure towards either EES or ECS indicated by the Triggering Entity Information in the Trigger Payload.

NOTE:	The ECS and EES can determine the port ID for EEC triggering service based on the local configuration.

3.	The SMS containing the Trigger Payload provided in step 4 is delivered to the EEC.

4.	Upon receiving the Trigger Payload, the EEC performs the procedure indicated in the Trigger Payload toward the Triggering entity (i.e., EES or ECS)

* * * End of Change * * * *