SA WG6 Meeting #55	S6-231735

Berlin, Germany, May 22 – 26, 2023	(revision of S6-23xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0361 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Provision ECS configuration information together with PLMN ID to 5GC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP |  |  |  |  |  | Date: |  |  | 2023-05-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In R18, the list of PLMNs that this ECS supports are included in ECS configuration information. However, the core network needs to identify which ECS addresses can be used to roaming UEs under a specific PLMN. When the UE is roaming in a PLMN A, 5GC needs to send the ECS address that supports this PLMN A to the roaming UE. If AF does not indicate the supported PLMN ID when providing ECS addresses to 5GC, 5GC will not be able to identify which ECS addresses are available for which supported PLMN. The supported PLMNs are inside of the ECS configuration information, and the 5GC can’t read the information internal the ECS configuration information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | When AF provides ECS configuration information to 5GC, the list of supported PLMN(s) should be provided together. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The 5GC can’t identify which PLMN that this ECS configuration information supports. And the 5GC can’t provide the available ECS configuration information to UE according to the PLMN ID when UE is roaming to other PLMN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change 1 * * *

8.3.2.1	General

ECS configuration information consists of one or more endpoint information (e.g. URI(s), FQDN(s), IP address(es)) of ECS(s), and optionally the corresponding ECS Provider Identifier. ECS configuration information can be

-	pre-configured with the EEC;

-	configured by an edge-aware AC;

-	configured by the user;

-	provisioned by MNO through 5GC procedure if the UE has the capability to deliver the ECS configuration information to the EEC on the UE (see 3GPP TS 23.548 [20], clause 6.5.2);

-	derived from HPLMN identifier for non-roaming scenario or from VPLMN identifier for roaming scenario; or

-	derived from serving SNPN identifier.

NOTE 1:	How the ECS configuration information is configured to the EEC user, or pre-configuration is out of scope of the present specification.

It may be possible to provide the ECS configuration information to the EEC from the 5GC if the UE has the capability to deliver the ECS configuration information to the EEC on the UE.

If the ECS (AF) provides the ECS configuration information to 5GC, in order to support to deliver the ECS configuration information to UE when roaming in other PLMN, the list of supported PLMN(s) should be provided together with ECS configuration information to 5GC.

NOTE 2:	As described in 3GPP TS 23.548 [20], NFs do not need to be aware of internal structure of ECS configuration information, the list of supported PLMN ID should be provided together.

It may be possible to provide the ECS configuration information to the EEC from an edge-ware AC via EDGE-5 reference point within the UE if the AC is configured with the ECS configuration information and can communicate with the EEC. When the ECS configuration information is provided from an AC, the EEC uses the ECS configuration information for the initial service provisioning for the AC if there is no ECS configuration information is provided from the 5GC.

If the ECS configuration information is provided by 5GC and available at the EEC, the EEC shall use the information for the initial provisioning request. Otherwise, the EEC shall use pre-configured ECS address for the initial provisioning if ECS configuration information is preconfigured with the EEC.

NOTE 3:	The ECS configuration information configured by an edge-aware AC is considered to be part of pre-configured ECS configuration information with the EEC for the AC.

Table 8.3.2.1-1 describes the information elements of ECS configuration information for an ECS.

Table 8.3.2.1-1: ECS configuration information per ECS

| Information element | Status | Description |
| --- | --- | --- |
| ECS address | M | Endpoint information of ECS (e.g. URI, FQDN, IP address) |
| ECSP Identifier (see NOTE 1) | O | The identifier of the ECSP (e.g., the MNO or a 3rd party service provider) that provides the ECS. |
| Spatial Validity Conditions | O | Spatial validity condition, as described in 3GPP TS 23.548 [20] |
| List of supported PLMN(s) | O | The List of PLMNs and associated ECSPs for which EDN configuration information can be provided by the ECS. |
| > PLMN ID | O | The identifier of a PLMN for which EDN configuration information can be provided by the ECS. |
| > List of supported ECSP(s) (see NOTE 2) | O | The identifier of the ECSP(s) associated with the PLMN and whose information is available at the ECS |
| >> ECSP ID | M | Identifier of an ECSP |
| NOTE 1: This IE shall be included when the ECS configuration information is provisioned by the MNO through the 5GC procedure. NOTE 2: This IE may not be included if the ECSP does not want to expose its EES deployment information or business relationship-related information. |  |  |