3GPP TSG-SA WG6 Meeting #55	S6-231764

Berlin, Germany 22nd – 26th May 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.554 | CR | 0134 | rev | - | Current version: | 18.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | update of Application Client (de-)registration using MSGin5G Client |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMARCH_Ph2 |  |  |  |  |  | Date: |  |  | 2023-05-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The architecture of Application Client, constrained device and UE is updated an agreed in S6-231645. This CR is proposed to update Application Client registration using MSGin5G Client based on the updated architecture and definitions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update Application Client registration using MSGin5G Client. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Application Client registration using MSGin5G Client is not aligned with the updated architecture. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.11.2, 8.11.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

8.11.2	Application Client registration using MSGin5G Client

The signalling flow for registration of Application Client on the UE-2 with MSGin5G Client on MSGin5G UE-1 to use MSGin5G service is illustrated in figure 8.11.2-1.

Pre-conditions:

1.	The MSGin5G UE-1 is configured with information to recognize and authorize UE-2.

2.	The Application Client on UE-2 has discovered or configured that MSGin5G Client resides on MSGin5G UE-1 can provide the MSGin5G service capability to it..

3.	The UE-2 is using NR-PC5 to communicate with MSGin5G UE-1.

Figure 8.11.2-1: Registration of Application Client on UE-2 with MSGin5G Client on MSGin5G UE-1

1)	An Application Client on the UE-2 registers with MSGin5G Client-1 in MSGin5G UE-1 to use MSGin5G service. The request message includes information elements as specified in Table 8.11.2-1.

Table 8.11.2-1: Information elements for Application Client register request

| Information element | Status | Description |
| --- | --- | --- |
| Layer-2 ID | M | Layer-2 identity of UE-2 |
| Application ID | M | Application ID of the application client on UE-2 |
| Credential information | M | UE-2 credential information |

Editor's note: The security parameters to include in the message between UE-2 and MSGin5G UE-1 are FFS.

2)	Upon receiving the request from the Application Client, the MSGin5G Client authorizes the Application Client on UE-2 to it for MSGin5G service. The MSGin5G Client assigns a Registration ID and stores the mapping of the Registration ID, Application ID and Layer-2 ID of the UE-2.

NOTE 1:	The MSGin5G Client may reject the request for registration to use it for MSGin5G service based on local condition (like available power or connectivity to access network or any other reason outside the scope of 3GPP).

3)	The MSGin5G Client sends response to the Application Client. The response message includes information elements as specified in Table 8.11.2-2.

Table 8.11.2-2: Information elements for Application Client register response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates success or failure of the request |
| Registration ID | O | Identifies registration. This IE is included only if Result IE is set to success. |
| Failure reason | O | Indicates failure reason. This IE is included only if Result IE is set to failure. |

NOTE 2:	If MSGin5G Client decided to reject the request for registration to use it for MSGin5G service or if authorization fails as in step 2, the MSGin5G Client sends a failure response to the Application Client. Otherwise, the MSGin5G Client sends a success response to the Application Client.

NOTE 3:	The MSGin5G Client may provide MSGin5G service capabilities for multiple Application Clients, at the same time.

8.11.3	Application Client de-registration using MSGin5G Client

The signalling flow for deregistration of Application Client on the UE-2 with MSGin5G Client-1 on MSGin5G UE-1 is illustrated in figure 8.11.3-1.

Pre-conditions:

1.	The Application Client on UE-2 issuccessfully registered with MSGin5G Client on MSGin5G UE-1.

Figure 8.11.3-1: Deregistration of Application Client on UE-2 with MSGin5G Client on MSGin5G UE-1

1)	An Application Client on the UE-2 deregisters with MSGin5G Client in MSGin5G UE-1 to discontinue usage of MSGin5G service. The request message includes information elements as specified in Table 8.11.3-1.

Table 8.11.3-1: Information elements for Application Client de-register request

| Information element | Status | Description |
| --- | --- | --- |
| Registration ID | M | Identifies the registration |
| Credential information | M | UE-2 credential information |

2)	Upon receiving the request from the Application Client, the MSGin5G Client removes the mapping of Registration ID, Application ID and Layer-2 ID of the UE-2. The MSGin5G Client sends response to the Application Client on UE-2. The response message includes information elements as specified in Table 8.11.3-2.

Table 8.11.3-2: Information elements for Application Client de-register response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates success or failure of the request |
| Failure reason | O | Indicates failure reason. This IE is included only if the Result IE is set to failure. |