3GPP TSG-SA WG6 Meeting #55	S6-231765

Berlin, Germany 22nd – 26th May 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.554 | CR | 0135 | rev | - | Current version: | 18.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | update of Application Client sending and receiving message using MSGin5G Client |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMARCH_Ph2 |  |  |  |  |  | Date: |  |  | 2023-05-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The architecture of Application Client, constrained device and UE is updated an agreed in S6-231645. This CR is proposed to update Application Client registration using MSGin5G Client based on the updated architecture and definitions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update Application Client registration using MSGin5G Client. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Application Client registration using MSGin5G Client is not aligned with the updated architecture. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.11.2, 8.11.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

8.11.4	Application Client sending a message using MSGin5G Client on another UE

The signalling flow for the Application Client on the UE-2 to send a message using MSGin5G Client on MSGin5G UE-1 is illustrated in figure 8.11.4-1.

Pre-conditions:

1.	The MSGin5G UE-1 is connected to an access network that provides connectivity to the MSGin5G Server.

2.	The Application Client on UE-2 is successfully registered with MSGin5G UE-1 to use MSGin5G service.

Figure 8.11.4-1: Application Client on UE-2 sends message using MSGin5G Client on MSGin5G UE-1

1)	An Application Client on the UE-2 sends a request to send MSGin5G message to the MSGin5G Client. The information elements defined in Table 8.11.4-1 are included in the message.

Table 8.11.4-1: Information elements for Request to send MSGin5G message

| Information element | Status | Description |
| --- | --- | --- |
| Recipient UE Service ID or AS Service ID (see NOTE) | O | The service identity of the recipient. This IE is mandatory to initiate Point-to-Point messaging and Point-to-AS messaging. |
| Group Service ID (see NOTE) | O | The service identifier of the target MSGin5G Group. This IE is mandatory to initiate Group messaging. |
| Broadcast Area ID (see NOTE) | O | The service identifier of the Broadcast Service Area where the message needs to be broadcast. This IE is mandatory in the Broadcast Message. |
| Messaging Topic (see NOTE) | O | Indicates which Messaging Topic this message is related to. This IE is mandatory for a message distribution based on topic. |
| Application ID | O | Identifies the application(s) for which the payload is intended. |
| Payload | M | Payload of the message. MSGin5G Server/Client is unaware of the content. |
| Delivery status required | O | Indicates whether delivery status is required or not. |
| Priority type | O | Application priority level requested for this message as specified in Table 8.3.2-1. |
| NOTE: Only one occurrence shall be present of any of these IEs. |  |  |

Editor's note:	If table 8.11.4-1 should be moved to separate clause to be applicable for communication between any Application Client and any MSGin5G Client is FFS.

2)	Upon receiving the request from the Application Client in UE-2, the MSGin5G Client constructs an MSGin5G message with the related IEs specified in table 8.3.2-1 and sends the MSGin5G message.

a)	if the size of the received message exceeds the maximum allowed packet size, the MSGin5G Client sends the message as specified in clause 8.5;or

b)	If the size of the received message does not exceed the maximum allowed packet size, the MSGin5G Client sends the message as specified in clause 8.7.; or

c)	If the size of the received message does not exceed the maximum allowed packet size, the MSGin5G Client may apply message aggregation as specified in clause 8.4 before sending the message as specified in clause 8.7

NOTE 1:	The MSGin5G Client may also reject the request to send the MSGin5G message based on local condition (like available power or connectivity to access network or any other reason outside the scope of 3GPP).

3)	The MSGin5G Client sends response to send MSGin5G message to Application Client on UE-2. The response message includes information elements as specified in Table 8.11.4-2.

Table 8.11.4-2: Information elements for Response to send MSGin5G message

| Information element | Status | Description |
| --- | --- | --- |
| UE Service ID or AS Service ID or Group Service ID or Broadcast Area ID or Message Topic | M | The service identifier that the message is sent |
| Result | M | Indicates success or failure of the request |
| Failure reason | O | Indicates failure reason. This IE is included only if Result IE is set to failure. |

NOTE 2:	If the MSGin5G Client has decided to reject the request to send the message or the MSGin5G Client received reject response from MSGin5G Server in step 2, the MSGin5G Client sends failure response to the Application Client  and stops performing further steps.

4)	If delivery status is requested while sending the message in step 1, the MSGin5G Client may receive MSGin5G message delivery status report from the MSGin5G Server.

5)	Upon receiving the MSGin5G message delivery status report, the MSGin5G Client sends the message delivery status report to the Application Client on UE-2. The message delivery status report includes information elements as specified in Table 8.11.4-3.

Table 8.11.4-3: Information elements for MSGin5G message delivery status

| Information element | Status | Description |
| --- | --- | --- |
| UE Service ID or AS Service ID | M | The service identifier that sends the message delivery status |
| Delivery status | M | Indicates delivery status |

8.11.5	Application Client receiving message via MSGin5G Client on another UE

The signalling flow for Application Client on the UE-2 to receive a message using MSGin5G Client on MSGin5G UE-1 is illustrated in figure 8.11.5-1.

Pre-conditions:

1.	The MSGin5G UE-1 is connected to an access network that provides connectivity to the MSGin5G Server.

2.	The Application Client on UE-2 is successfully registered with MSGin5G UE-1 to use MSGin5G service.

Figure 8.11.5-1: Application Client on UE-2 receives message using MSGin5G Client on MSGin5G UE-1

1)	The MSGin5G Client receives MSGin5G message as specified in clause 8.3.3 for the Application Client on UE-2. The MSGin5G Client performs reassembly if the received message is part of a segmented message and waits till the whole message is received. The MSGin5G Client may also perform segment recovery procedure as specified in clause 8.5.4 to recover missing segments.

2)	Upon successfully receiving a message for the Application Client on UE-2, the MSGin5G Client sends message received request to Application Client based on Application ID on the received message. The message includes information elements as specified in Table 8.11.5-1.

Table 8.11.5-1: Information elements for Message received request

| Information element | Status | Description |
| --- | --- | --- |
| Recipient UE Service ID/AS Service ID (see NOTE) | O | The service identifier of the originator. This IE is mandatory for Point-to-Point messaging and AS-to-Point messaging. |
| Group Service ID (see NOTE) | O | The service identifier of the MSGin5G Group This IE is mandatory for Group Message. |
| Messaging Topic (see NOTE) | O | Indicates the Topic for which the message is received. This IE is mandatory for a Topic Message. |
| Payload | M | Payload of the message. MSGin5G Server/Client is unaware of the content. |
| Delivery status required | O | Indicates whether delivery status is required or not |
| Priority type | O | Application priority level requested for this message as specified in Table 8.3.2-1. |
| NOTE: Only one occurrence shall be present of any of these IEs. |  |  |

3)	Upon successfully receiving the message, the Application Client on UE-2 sends the message received response to the MSGin5G Client.

4)	If delivery status is requested in the received message, the Application Client on UE-2 sends message delivery status to the MSGin5G Client.

5)	Upon receiving the delivery status, the MSGin5G Client constructs the MSGin5G message delivery status report  as specified in table 8.3.4-1 and sends it to the MSGin5G Server.