3GPP TSG-SA WG6 Meeting #55	S6-231811

Berlin, Germany, 22th – 26th May 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.282 | CR | 0316 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates adhoc group call procedures for allowing a subsequent MCData communication after an adhoc group emergency alert |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MC_AHGC |  |  |  |  |  | Date: |  |  | 2023-05-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Adhoc group emergency calls may be initiated as subsequent or associated communication after an adhoc group emergency alert. More precisely, the current adhoc group procedures do not describe the reuse of an existing adhoc group, which was formed for an adhoc group emergency alert, to establish an adhoc group emergency call as well. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adhoc group call procedures are updated to cover the subsequent MCData adhoc group emergency communication scenario after an adhoc group emergency alert. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 1 requirements not fullfilled. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.17.2.1, 7.17.3.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 7.17.2.1	Ad hoc group data session request (MCData client – MCData server)

Table 7.17.2.1-1 describes the information flow ad hoc group data session request from the MCData client to the MCData server.

Table 7.17.2.1-1 Ad hoc group data session request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending data |
| Functional alias | O | The associated functional alias of the MCData user sending data. |
| MCData ad hoc group ID (see NOTE 1, NOTE 5) | O | The MCData group ID which is generated by the MCData user to be associated with the ad hoc group data communication |
| Encryption supported (see NOTE 2) | O | Indicates whether this ad hoc group data communication supports end-to-end encryption |
| MCData ID list (see NOTE 3, NOTE 5) | O | MCData IDs of the participants being invited for the ad hoc group data communication |
| SDP offer | M | Offered media parameters |
| Imminent peril indicator (see NOTE 4) | O | Indicates that the ad hoc group data communication request is for ad hoc group imminent peril data communication |
| Emergency Indicator (see NOTE 4) | O | Indicates that the ad hoc group data communication request is for ad hoc group emergency data communication |
| Broadcast indicator (see NOTE 4) | O | Indicates that the ad hoc group data communication request is for a broadcast ad hoc group data communication |
| MCData ID list (see NOTE 3) | O | List of participants required to acknowledge the ad hoc group data communication before start of the data transmission |
| Location information | O | Location of the of the MCData user sending data. |
| Criteria for determining the participants (see NOTE 5) | O | Carries the details of criteria or meaningful label identifying the criteria or the combination of both which will be used by the MCData server for determining the participants e.g., it can be a location based criteria to invite particpants in a particular area |
| Requested priority | O | Application priority level requested for this ad hoc group data communication |
| NOTE 1: If this information element is not included, the MCData server assigns an MCData ad hoc group ID to be used for the ad hoc group data communication. This information element is returned to the MCData user who is sending the data to use in the ad hoc group data communication. If the request follows an ad hoc group for emergency alert, then this element must be present. NOTE 2: This information element is present and set to true only if this ad hoc group data communication is encrypted. When the ad hoc group data communication is initiated with participants provided by the initiator this acts as an indicator that subsequent requests follow targeting the individual participants and carrying the relevant key material. If this information element is set to false or not present, then this ad hoc group data communication is unencrypted. NOTE 3: This element is included only when the data communication initiating client sends the list of participants. NOTE 4: If used, only one of these information elements is present. NOTE 5: Only one of these information elements is present. |  |  |

# * * * * Second change * * * *

##### 7.17.3.1.1	Ad hoc group data communication setup

Figure 7.17.3.1.1-1 below illustrates the ad hoc group data communication setup procedure initiated by an authorized user.

Pre-conditions:

1.	The authorized user at MCData client 1 wants to invite MCData users at MCData client 2, MCData client 3 and MCData client 4 for the ad hoc group data communication or, if the ad hoc group call follows an ad hoc group emergency alert, the MCData client 1 is aware of the ad hoc group ID.

2.	Number of participants being invited for the ad hoc group data communication is within the limit for non pre-configured approach.

3.	End-to-End encryption is supported for this ad hoc group data communication.

4.	MCData client 1 is aware of the MCData IDs of the participants.

NOTE 1:	Selection of MCData IDs of the participants can be manual or from the user profile configuration data or by any other means. This is left for the implementation.

Figure 7.17.3.1.1-1: Ad hoc group data communication setup

1.	User at MCData client 1 would like to initiate an ad hoc group data communication. The MCData client 1 initiates the ad hoc group data communication by sending the ad hoc group data session request containing the list of participants or an ad hoc group ID from an ad hoc group emergency alert to the MCData server. Encryption supported information element shall be set to true since end-to-end encryption is supported. An SDP offer containing the MCData client media parameters is included. If the MCData user of MCData client 1 has selected a functional alias, then the ad hoc group data session request contains that functional alias.

If the MCData user at MCData client 1 initiates an MCData emergency ad hoc group data communication or the MCData emergency state is already set for the MCData client 1 (due to a previously triggered MCData emergency alert):

i.	the MCData ad hoc group data session request shall contain an emergency indicator;

ii.	if the MCData emergency state is not set already, MCData client 1 sets its MCData emergency state. The MCData emergency state of MCData client 1 is retained until explicitly cancelled by the user of MCData client 1.

2.	The MCData server accepts the ad hoc group data session request if the ad hoc group data communication is supported and authorized. Otherwise reject the ad hoc group data session request and do not continue with the rest of the steps. If authorised, it validates whether the number of invited participants is within the configured limit before proceeding with the data communication setup.

If functional alias is present, the MCData server checks whether the provided functional alias allowed to be used and has been activated for the user.

If location information was included in the ad hoc group data session request, the MCData server checks the privacy policy of the MCData user to decide if the location information of MCData client 1 can be provided to other users on the ad hoc group data communication (refer to Annex A.3 "Authorisation to provide location information to other MCData users on a data communication when talking").

If an emergency indicator is present in the received MCData ad hoc group data session request, the MCData ad hoc group is considered to be in the in-progress emergency state until this ad hoc group data communication is terminated; and

If an imminent peril indicator is present in the received MCData ad hoc group data session request, the MCData ad hoc group is considered to be in the in-progress imminent peril state until this ad hoc group data communication is terminated.

The MCData server considers the ad hoc group data communication participants as implicitly affiliated to the ad hoc group.

3.	The MCData server shall send the ad hoc group data session request return message to MCData client 1 containing the below:

i.	The MCData ad hoc group ID; and

ii.	Result of whether the ad hoc group data session request is authorized or not

If the ad hoc group data session request is not authorized, MCData client 1 shall not proceed with the rest of the steps.

4a-4c.	The MCData server sends the ad hoc group data session requests towards the MCData clients of the invited users based on step 1. While sending the ad hoc group data session requests, the MCData server shall remove the information elements that are not required to be conveyed to the target MCData clients (e.g. MCData ID list of the users who are required to acknowledge)

5a-5c.	The receiving MCData clients are notified about the incoming ad hoc group data session request.

6a-6c.	The receiving MCData clients accept the ad hoc group data session requests and send ad hoc group data session responses to the MCData server. The response may also contain a functional alias of the responding MCData user, which is verified (valid and activated for the user) by the MCData server.

7.	The MCData server sends the ad hoc group data session response to MCData client 1 through the signalling path to inform about successful data communication establishment.

NOTE 2:	Steps 5 to step 7 can start to occur before all of step 4 is completed since the MCData server do not require to wait for the previous ad hoc group data session request to complete before sending the ad hoc group data session request to another participant.

8.	If the initiating MCData user requires the acknowledgement from the invited MCData users, and the required MCData users do not acknowledge the data session setup within a configured time (the "acknowledged data communication setup timeout"), then the MCData server may proceed with or abandon the data session and then notify the initiating MCData user that the acknowledgements did not include all required members according to ad hoc group data communication policy from the user profile configuration. The MCData server may notify the initiating MCData user of all MCData users who did not acknowledge the ad hoc group data session request within the configured time. This notification may be sent to the initiating MCData user by the MCData server more than once during the data communication when MCData users join or leave the MCData ad hoc group data communication.

9.	MCData client 1, MCData client 2, MCData client 3 and MCData client 4 establish media plane for data communication.

NOTE 2:	Step 9 can occur any time following step 7 if the conditions to proceed with the data communication are met.

# * * * * End of changes * * * *