3GPP TSG-SA WG6 Meeting #55	S6-231905

Berlin, Germany 22nd – 26th May 2023	(revision of S6-231542, S6-231306)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0313 | rev | 2 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Edge Node Sharing solution |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-02-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | KI#22, Solution #44 was studied in TR 23.700-98. In Solution #44 studied procedure to Get list of all registered EAS from partner OP. GSMA OPG has requirement: - Get a list of Applications; for an OP to retrieve the list of published application instances with specific criteria (e.g. edge location, availability zone, etc.) - As indicated in LS S6-223333, Both ECS and EES(s) can be considered as a part of Operator Platform. - As per clause 4.4 of “Operator Platform Telco Edge Requirements”: “Operator’s OP (Home OP) may trigger a discovery request for the applications available on the Partner's resources” - 3GPP SA6 is currently algning the EDGEAPP architecture with GSMA requirements. It is required to enable EES (as an OP) to get list of all applications from EES (of partner OP). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added a NOTE to clarify that EAS discovery procedure can be initaited without discovery filters in such case partner EES can share all EASs based on federation agreement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | GSMA requirement will not be fulfilled in Rel-18. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.18.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | KI#22, Solution #44 Depends on approval of CR: 0318 (TDoc: S6-231627) which was agreed in meeting SA6#54-e |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 8.18.2.3.2	EAS discovery via leading ECSP

Pre-conditions:

1.	ECSP-1 and ECSP-2 have a service level agreement to share edge services.

Figure 8.18.2.3.2-1: EAS discovery for ENS

1.	The EEC sends EAS discovery request to EES (ECSP-B) as specified in clause 8.5 and may include the endpoint of EES (ECSP-A) in the request, if provided by the ECS as specified in clause 8.18.2.2.1.

2.	If the EAS discovery request contains EES information of a partner ECSP (ECSP-A) or 	the required EAS is not available with the EES the EES may determine to use ENS. If the EAS discovery request contains EES information of a partner ECSP, the EES can use it after validating the information.

NOTE 1:	To validate the information of partner ECSPs, EES (ECSP-B) can use the federation agreement between ECSPs.

3.	If the EAS discovery request does not contain EES information of a partner ECSP, the EES may use the EES retrieval procedures as specified in clause 8.8.3.3 to obtain EES information of a partner ECSP. The Retrieve EES request may include an ENS indication so that the ECS (ECSP-B) can skip checking T-EES(s) locally registered in the ECS (ECSP-B).

4.	Once EES information of the partner ECSP is available, the EES (ECSPB) sends the EAS discovery request to the EES (ECSP-A) as described in step 3 of clause 8.8.3.2. The EAS discovery request includes the information of the MNO serving the UE (e.g. MNO name, PLMN ID).The EES (ECSPA) validates the request and returns EAS discovery response including the discovered candidate EAS(s) to the EES (ECSP-B). In addition to step 4 of clause 8.8.3.2, the candidate EAS(s) are discovered based on the serving MNO information received from the EES (ECSP-B) and allowed MNO information registered by the EAS.

NOTE 2:	To validate the EAS discovery request, EES (ECSP-A) can use the federation agreement between ECSPs.

NOTE 3:	Upon receiving EAS discovery request from EEC, the EES (ECSP-B) may send EAS discovery request without any discovery filters. In such case, EES (ECSP-A) provides list of all registered and allowed EASs in the response message.

5.	EES (ECSP-B) provides the discovered information to the EEC as part of the EAS discovery response as specified in clause 8.5.

* * * End of Change * * * *