3GPP TSG-SA WG6 Meeting #55	S6-231916

# Berlin, Germany, 22th May – 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0223 | rev | - | Current version: | 18.4.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SEAL NRM determines time synchronization activation for TSC stream |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSEAL2 |  |  |  |  |  | Date: |  |  | 2023-05-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The SEAL Network Resource management server supports the creation of TSC streams. This CR suggest to add the activation and deactivation of the time synchronization in the TSC stream creation procedure for the streams which require time synchronization. The deactivation of the time synchronization is required for the activated streams during the TSC deletion procedure. N5 is the reference point between an AF/TSN AF and PCF (3GPP TS 23.501), which is why a correction in the text is made. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The ability for the SEAL NRM server to activate time synchronization is added to the creation procedure and deactivation of the same for the deletion procedure. The TSC stream creation request is updated to include elements needed to determine whether time synchronization is needed. Some additional correction is made. The reference point(Nxx) was not specified in the TSC stream deletion procedure. This was corrected to the N5 reference point between the PCF and AF. A few corrections are also made where references are not written correctly in clause 14.3.2.25 and 14.3.7.3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SEAL NRM will not be able to determine and activate/deactivate time synchronization for the streams having time synchronization requirements. The wrong reference point is mentioned and references are not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.3.2.25, 14.3.7.3, 14.3.7.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| () |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

* * * First Change * * * *

#### 14.3.2.25	TSC stream creation request

Table 14.3.2.25-1 describes the information flow TSC stream creation request from the VAL server to the NRM server.

Table 14.3.2.25-1: TSC stream creation request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the VAL server performing the request. |
| VAL Stream ID | M | It identifies the VAL stream. |
| Stream specification | M | Stream specification includes MAC addresses of the source and destination DS-TT ports (e.g. as defined in IEEE 802.1CB [37]). |
| Traffic specification | M | It includes MaxLatency, MaxFrameInterval, MaxFrameSize, MaxIntervalFrames; and it may additionally include EarliestTransmitOffset, LatestTransmitOffset, Jitter etc. (e.g. as described in IEEE 802.1Qcc [35] in clause 46.2). |

* * * Next Change * * * *

#### 14.3.7.3	TSC stream creation procedure

This procedure allows the VAL server to create a TSC stream. The TSC stream creation procedure enables the VAL server to establish TSC connectivity with the required QoS between the UEs connected to the 5GS after the stream discovery procedure.

Pre-conditions:

1.	Each UE has an established Ethernet PDU session for its DS-TT port MAC address.

2.	Connectivity between the DS-TTs has been validated by the TSC stream availability discovery procedure specified in clause 14.3.7.2.

3.	The NRM server maintains mapping from the traffic class to TSC QoS.

Figure 14.3.7.3-1: TSC stream creation procedure

1.	The NRM server receives a TSC stream creation request from a VAL server to create a TSC stream identified by a VAL Stream ID, between DS-TT ports in the stream specification and for the traffic class in the traffic specification.

2.	The NRM server calculates the schedule for the VAL Stream ID based on the information collected earlier from the 5GS via N5. It provides per-stream filtering and policing parameters (e.g. as defined in IEEE 802.1Q [36]) used to derive the TSC QoS information and related flow information. NRM server also provides the forwarding rule (e.g. as defined in IEEE 802.1Q [36]) used to identify the DS-TT MAC address of the corresponding PDU session. Based on the 5GS bridge delay information it determines the TSC QoS information and TSC Assistance information for the stream.

3. 	Based on the Traffic specification (from the TSC stream creation request in step 1), the SEAL NRM server determines whether time synchronization needs to be activated for the TSC stream on the DS-TTs. If the DS-TTs are time synchronized, then the NRM does not activate the time synchronization for the corresponding DS-TT.

4.	As a TSCTSF, the NRM server triggers via N5 the Npcf_PolicyAuthorization_Create service operation as described in 3GPP TS 23.502 [11] for the TSC stream for both UL QoS flow (sender UE to UPF/bridge) and DL QoS flow (UPF/bridge to receiver UE). The Policy Authorization request includes the DS-TT port MAC address, TSC QoS information, TSC Assistance Information (3GPP TS 23.501 [1], cl.5.27.2.3), flow bit rate, priority, Service Data Flow Filter containing flow description including Ethernet Packet Filters. The QoS flow will be assigned for the PDU session for the source MAC address for the UL direction and for the PDU session for the destination MAC address for the DL direction. This information is delivered to the DS-TT by the 5GS.

As part of the Npcf_PolicyAuthorization_Create procedure, if time synchronization is determined to be needed for the TSC stream on the DS-TTs in step 3, the NRM server uses the procedures in clause K.2.2 of 3GPP TS 23.501 [10] to activate the time synchronization. The procedure includes the configuration and initialization of the PTP instance in the DS-TTs, the construction of PMICs to each DS-TT/UE to activate the time synchronization service in the DS-TT and to subscribe for the port management information changes in the DS-TTs.

Editor's note: Using gate control parameters for hold and forward buffering is FFS.

5.	The NRM server sends TSC stream creation response to the VAL server with the result of TSC stream creation for the VAL Stream ID.

* * * Next Change * * * *

#### 14.3.7.4	TSC stream deletion procedure

This procedure allows the VAL server to delete a TSC stream.

Pre-conditions:

1.	The TSC stream is configured in the 5GS and the DS-TTs.

Figure 14.3.7.4-1: TSC stream deletion procedure

1.	The NRM server receives a request from VAL server to delete a TSC stream for with a VAL Stream ID.

2.	The NRM server identifies the MAC addresses of the DS-TTs involved in the stream based on the stored information for the VAL Stream ID. If none of the streams require to keep the time synchronization activated, the NRM server deactivates the time synchronization for the corresponding DS-TTs, otherwise keeps the time synchronization activated.

3.	As a TSCTSF, the NRM server triggers via N5 the Npcf_PolicyAuthorization_Delete service operation defined in 3GPP TS 23.502 [11] for MAC addresses referred to by the VAL Stream ID. The NRM server uses the procedure to delete both UL QoS flow (sender UE to UPF/bridge) and DL QoS flows (UPF/bridge to receiver UE) from the PDU sessions of the UEs referred to by the VAL Stream ID. As a part of the Npcf_PolicyAuthorization_Delete procedure, the NRM server may deactivate the time synchronization for the DS-TTs as described in clause K.2.2 3GPP TS 23.501 [10].

4.	The NRM server sends TSC stream deletion response to the VAL server with the result of TSC stream deletion for the VAL Stream ID.

* * * End of changes * * * *