3GPP TSG-SA6 Meeting #55	S6-231917

# Berlin, Germany, 22nd May 2023 - 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0401 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CAS decided ACR scenario via old S-EES for CESless architecture |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | KPN N.V. |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-05-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Adding a procedure for CESless architecture for the CAS to be able to perform ACR to T-EAS re-using an existing procedure – clause 8.8.2.4 (S-EAS decided ACR scenario) where the CAS acts as an S-EAS, and the T-EAS discovery is sent to the S-EES which holds the EEC context. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new procedure is added for CAS decided ACR scenario via S-EES for CESless architecture in Clause 8.8.2A. The procedure is similar with "S-EAS decided ACR scenario" as specified in clause 8.8.2.4, where the CAS acts like the S-EAS. However, during the ACR execution phase, the CAS needs to know the EES to use for the discovery before continuing with T-EAS discovery. This procedure uses the S-EES endpoint information and the UE Identifier that are shared from the EEC to the CAS via the AC to connect to the old S-EES and discover the T-EAS. This allows also for the EEC context to be pushed to the T-EES if the T-EES is different from the S-EES. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The CAS decided ACR scenario remains undefined when the CAS wants to perform an ACR to a T-EAS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.8.2A.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * Change 1 * * * *

#### 8.8.2A.x 	CAS decided ACR scenario via old S-EES

In this scenario, the CAS detects the need for ACR and makes the decision about whether to perform the ACR and starts the ACR at a proper time.

During the ACR execution phase, the CAS needs to determine an EES in the service area of the EEC before continuing with T-EAS discovery. The EEC sends to the CAS an AF-specific UE ID or Edge UE ID and the endpoint of the S-EES that has been serving the EEC before the ACR from EAS to CAS, and which may still hold the EEC context (i.e. the EEC context is not stale). The CAS sends the EAS discovery request to the S-EES that selects an appropriate T-EES and T-EAS as per clause 8.8.3.2 where the CAS acts as a S-EAS.

Pre-conditions:

- The CAS has a buisness relationship with the ECSP, otherwise, the EES will reject the discovery request during authorization.

- Prior to being connected to the CAS, the EEC was connected to a S-EES that may still hold its context.

- When performing the ACR from the EAS to CAS, the EEC has requested an AF-specific UE ID or Edge UE ID from the S-EES to re-use it when performing the ACR from CAS to EAS as described in clause 8.8.2A.2.

- When performing the ACR from the EAS to CAS, the EEC has subscribed to notifications from the S-EES via the SEAL notification management service as described in clause 9.1.

Figure 8.8.2A.x-1: CAS decided ACR with T-EAS Discovery via S-EES

Phase I: ACR Detection

1.	The CAS detects the need for ACR.

NOTE 1:	How the CAS detects the need for ACR is outside the scope of this specification.

Phase II: ACR Decision

2.	The CAS makes the decision to perform the ACR.

NOTE 2:	How the CAS determines when to start the ACR is outside the scope of this specification.

Phase III: ACR Execution

3.  The EEC provides to the CAS via the AC the Edge UE ID and the endpoint of the S-EES that it was connected to prior to the ACR from Edge to CAS and which may still hold the EEC context. This step may be skipped if the EEC has already provided this information to the CAS during the ACR from Edge to CAS.4. Same as steps 3 to 5 in clause 8.8.2.4 where the CAS acts as an S-EAS.

NOTE 3:	When the CAS performs the EAS discovery operation, the S-EES checks whether the requesting CAS is authorized to perform the discovery operation and may decide to reject or accept the request.

5.  If the S-EES can communicate with the EEC, step 6 from clause 8.8.2.4 is performed to notify the EEC of the selected T-EES. Otherwise, the notification is sent to the EEC via the SEAL notification management service as described in clause 9.1 if the EEC has subscribed to receive notifications from the S-EES.

6. Same as step 7 in clause 8.8.2.4 where the CAS acts as an S-EAS.

Phase IV: Post-ACR Clean up

7. Same as step 8 to step 9 in clause 8.8.2.4 where the CAS acts as an S-EAS.

8. If the S-EES can communicate with the EEC, step 10 from clause 8.8.2.4 is performed. Otherwise, the notification is sent to the EEC via the SEAL notification management service as described in clause 9.1 if the EEC has subscribed to receive notifications from the S-EES.

# * * * End of Change * * * *