3GPP TSG-SA WG6 Meeting #55	S6-231920

Berlin, Germany, 22nd – 26th May 23	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0403 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | AC information exposure update and correction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Convida Wireless LLC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-5-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The AC information exposure subscription has parameters for describing HOW the notifications should be sent (e.g. how many times) but not WHEN. In addition, allowing filtering for EAS bundles is neccesary |  |  |  |  |  |  |  |  |
| Summary of change: |  | The description of the “Event conditions” IE is updated to provide conditions for triggering the notification A new IE “Notification parameters” has been added The filters are updated with EAS bundle information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The AC information exposure subscription has no triggering conditions specified. There is no support for the EAS to be notified based on EAS bundle information |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.6.4.3.2; 8.6.4.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1 minimizes the changes to the “event conditions” description, but still clarifies that the event is the sending of the notification. |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 8.6.4.3.2	AC information subscription request

Table 8.6.4.3.2-1 describes the information flow for an AC information subscription request from the EAS to the EES.

Table 8.6.4.3.2-1: AC information subscription request

| Information element | Status | Description |
| --- | --- | --- |
| EASID | M | The identifier of the EAS |
| Security credentials | M | Security credentials of the EAS |
| Subscription type | M | Choice of "One time" or "Event based" determining the type of processing requested |
| Notification Target Address | M | The Notification Target Address (e.g. URL) where the notifications destined for the EAS should be sent to. |
| Filters | O | List of characteristics for discovery purposes, as detailed in Table 8.6.4.3.2-2. |
| Event conditions | O | Parameters indicating notification event conditions e.g. number of notifications instances, subscription timeout, etc. |
| Trigger parameters | O | Parameters indicating notification triggering conditions e.g. EEC registration, EAS discovery. |

Table 8.6.4.3.2-2: Filters

| Information element | Status | Description |
| --- | --- | --- |
| ACID(s) | O | List of identities of AC(s) to be matched |
| UE ID(s) | O | List of UE identifiers to be matched |
| AC Type(s) | O | List of categories or types of ACs (e.g. V2X) to be matched |
| ECSP ID(s) | O | Identifier(s) for the ECSP associated with the EEC. |
| Operation Schedule | O | EAS operation schedule (e.g. time windows) to be matched |
| Geographical Service Area | O | EAS service area for identifying UEs with matching expected geographical location(s) (e.g. route). This geographic information can express a geographic point, polygon, route, signalling map, or waypoint set. |
| Topological Service Area | O | EAS service area for identifying UEs with matching expected topological location(s). See possible formats in Table 8.2.7-1. |
| Maximum AC Service KPIs (NOTE) | O | Maximum value for identifying the ACs to be matched. Both Minimum required AC Service KPIs and Expected AC Service KPIs from the AC Profiles provided during registration (Table 8.4.2.3.2-1) and/or discovery subscription (Table 8.5.3.4-1) need to be lower than this value. |
| Min AC Service KPIs (NOTE) | O | Minimum value for identifying the ACs to be matched. Both Minimum required AC Service KPIs and Expected AC Service KPIs from the AC Profiles provided during registration (Table 8.4.2.3.2-1) and/or discovery subscription (Table 8.5.3.4-1) need to be greater than this value. |
| UE location(s) | O | List of locations (e.g. routes) of the UE(s) for which the request applies |
| EAS bundle indication | O | Indicator for identifying ACs which include EAS bundle information in the AC Profile and the EAS bundle includes the requesting EAS. If “Bundle ID or list of EASID” or “EAS bundle requirements” filters are not included, the presence of this IE signifies that all ACs which include EAS bundle information in the AC Profile match. |
| > Bundle ID or list of EASID | O | A list of EASIDs or a bundle ID as described in clause 7.2.10 for identifying ACs with EAS bundle information in the AC Profile and with matching bundle EAS identification information. |
| > EAS bundle requirements | O | Bundle requirements (as described in clause 8.2.10) for identifying ACs with EAS bundle information in the AC Profile and with matching bundle requirements. |
| > Partial EAS bundle determination | O | Indicator for identifying ACs which include EAS bundle information in the AC Profile but for which only an EAS subset has been determined by EES. |
| NOTE: Maximum and minimum AC Service KPIs values may be provided for any IEs from Table 8.2.3-1. |  |  |

* * * Next Change * * * *

##### 8.6.4.3.5	AC information subscription update request

Table 8.6.4.3.5-1 describes the information flow for an AC information subscription update request from the EAS to the EES.

Table 8.6.4.3.5-1: AC information subscription update request

| Information element | Status | Description |
| --- | --- | --- |
| Security credentials | M | Security credentials of the EAS |
| Subscription ID | M | Subscription identifier corresponding to the subscription. |
| Filter | O | List of characteristics for discovery purposes, as detailed in Table 8.6.4.3.2-2. |
| Event conditions | O | Parameters indicating notification event conditions e.g. number of notifications instances, subscription timeout, etc. |
| Trigger parameters | O | Parameters indicating notification triggering conditions e.g. EEC registration, EAS discovery. |

* * * END Changes * * * *