3GPP TSG-SA WG6 Meeting #55	S6-232103

Berlin, Germany 22nd – 26th May 2023	(revision of S6-231872)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0388 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Interaction with CR in common EAS discovery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-05-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Interaction with CR is not specified in common EAS discovery. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add CR interaction between the EES and CR, based on sol#30 in TR. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete TS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.Y.2.5 (new), 8.Y.3.7 (new), 8.Y.3.8 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 8.Y.2.5	Common EAS information storage

Figure 8.Y.2.5-1: Common EAS information storage

- The EES sends Common EAS information store request message to the Central Repository. The request message includes EES ID, UE ID, UE location, EAS ID, EAS service area, EAS endpoint and Application Group ID.

- Upon receiving the request, the the Central Repository checks if there is any stored EAS serving the application group for the EAS ID and capable of serving the UE (i.e. UE location is within the stored EAS service area). If no common EAS is identified, the Central Repository stores the received information for subsequent binding requests.

- The Central Repository responds the EES with Common EAS information store response message. The Central Repository may reject the request with common EAS information identified in step 2 or accept the request.

* * * Next Change * * * *

#### 8.Y.3.7	Common EAS information store request

Table 8.Y.3.7-1 describes the information elements for common EAS information store request from the EES to the Central Repository.

Table 8.Y.3.7-1: Common EAS information store request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the EES. |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| EAS ID | M | The identifier of the EAS. |
| EAS endpoint | M | Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the EAS. |
| Application Group ID | M | Identity of the group of UEs associated with the EAS ID. |
| UE ID | O | Identifier of the UE. |
| UE location | M | The location information of the UE. The UE location is described in clause 7.3.2. |
| EAS service area (NOTE) | O | EAS Geographical Service Area and/or EAS Topological Service Area as described in table 8.2.4-1. |
| NOTE: One of these IEs shall be present in the message. |  |  |

#### 8.Y.3.8	Common EAS information store response

Table 8.Y.3.8-1 describes the information elements for common EAS information store response from the Central Repository to the EES.

Table 8.Y.3.8-1: Common EAS information store response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response (NOTE) | O | Indicates that the request was successful. |
| Failure response (NOTE) | O | Indicates that the request has failed. |
| > Cause | O | Indicates the failure cause. Only included when Failure response is included. |
| > Common EAS information | O | This IE includes common EAS endpoint. |
| NOTE: One of these IEs shall be present in the message. |  |  |

* * * END of Change * * * *