3GPP TSG-SA WG6 Meeting #55	S6-232108

Berlin, Germany 22nd – 26th May 2023	(revision of S6-231781)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0372 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enhancements for EAS synchronization |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-05-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | EAS synchronization has been studied in TR 23.700-98 but not yet specified in the normative phase. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new clause for EAS synchronization; add some new elements to EAS profile and EAS discovery filter. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EAS synchronization is not supported in Rel-18. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.4, 8.5.3.2, 8.X (new), 8.X.1(new), 8.X.2(new), 8.X.2.1(new), 8.X.2.2(new), 8.X.2.2.1(new), 8.X.2.2.2(new), 8.X.2.2.3(new), 8.X.2.2.4(new), 8.X.2.3(new), 8.X.3(new), 8.X.3.1(new), 8.X.3.2(new), 8.X.3.3(new), 8.X.3.4(new), 8.X.3.5(new), 8.X.3.6(new), 8.X.3.7(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | KI#13, Solution#53 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

8.5.3.2	EAS discovery request

Table 8.5.3.2-1 describes information elements for the EAS discovery request. Table 8.5.3.2-2 provides further detail about the EAS Discovery Filter information element.

Table 8.5.3.2-1: EAS discovery request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The ID of the requestor (e.g. EECID) |
| UE Identifier | O | The identifier of the UE (i.e. GPSI or identity token) |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| EAS discovery filters | O | Set of characteristics to determine required EASs, as detailed in Table 8.5.3.2-2. |
| UE location | O | The location information of the UE. The UE location is described in clause 7.3.2. |
| Serving MNO information (NOTE 2) | O | The serving MNO information (e.g. MNO name, PLMN ID) which is serving the subscriber. |
| Target DNAI (NOTE 1) | O | Target DNAI information which can be associated with potential T-EAS(s) |
| EEC Service Continuity Support | O | Indicates if the EEC supports service continuity or not. The IE also indicates which ACR scenarios are supported by the EEC or, if this message is sent by the EEC to discover a T-EAS, which ACR scenario(s) are intended to be used for the ACR. |
| EES Service Continuity Support (NOTE 1) | O | The IE indicates if the S-EES supports service continuity or not. The IE also indicates which ACR scenarios are supported by the S-EES or, if the EAS discovery is used for an S-EES executed ACR according to clause 8.8.2.5, which ACR scenario is to be used for the ACR. |
| EAS Service Continuity Support (NOTE 1) | O | The IE indicates if the S-EAS supports service continuity or not. The IE also indicates which ACR scenarios are supported by the S-EAS or, if the EAS discovery is used for an S-EAS decided ACR according to clause 8.8.2.4, which ACR scenario is to be used for the ACR. |
| EAS Instantiation Triggering Suppress | O | Indicates to the EES that EAS instantiation triggering should not be performed for the current request. |
| EAS selection request indicator | O | Indicates the request for EAS selection support from the EES (e.g., for constrained device). |
| NOTE 1: This IE shall not be included when the request originates from the EEC. NOTE 2: This IE shall be included if edge node sharing is used. |  |  |

Table 8.5.3.2-2: EAS discovery filters

| Information element | Status | Description |
| --- | --- | --- |
| List of AC characteristics (NOTE 1) | O | Describes the ACs for which a matching EAS is needed. |
| > AC profile (NOTE 2) | M | AC profile containing parameters used to determine matching EAS. AC profiles are further described in Table 8.2.2-1. |
| List of EAS characteristics (NOTE 1, NOTE 3) | O | Describes the characteristic of required EASs. |
| > EASID | O | Identifier of the required EAS. |
| > EAS synchronization ID | O | Identity of a group of EASs for synchronization. |
| > Bundle ID or list of EASID | O | A list of EASIDs or a bundle ID as described in clause 7.2.10. |
| > EAS provider identifier | O | Identifier of the required EAS provider |
| > EAS type | O | The category or type of required EAS (e.g. V2X) |
| > EAS schedule | O | Required availability schedule of the EAS (e.g. time windows) |
| > EAS Geographical Service Area | O | Location(s) (e.g. geographical area, route) where the EAS service should be available. |
| > EAS Topological Service Area | O | Topological area (e.g. cell ID, TAI) for which the EAS service should be available. See possible formats in Table 8.2.7-1. |
| > Service continuity support | O | Indicates if the service continuity support is required or not. |
| > Service permission level | O | Required level of service permissions e.g. trial, gold-class |
| > Service feature(s) | O | Required service features e.g. single vs. multi-player gaming service |
| NOTE 1: Either "List of AC characteristics" or "List of EAS characteristics" shall be present. NOTE 2: "Preferred ECSP list" IE shall not be present. NOTE 3: The "List of EAS characteristics" IE must include at least one optional IE, if used as an EAS discovery filter. |  |  |

* * * Next Change * * * *

8.5.1	General

Discovery procedures enable entities in an edge deployment to obtain information about EAS and their available services, based on specified criteria of interest.

EAS discovery enables the EEC to obtain information about available EASs of interest (e.g. instantiated EASs registered with the EES and instantiable EASs that may be created when needed). The discovery of the EASs is based on matching EAS discovery filters provided in the request.

When multiple EASs are discovered for a specific AC, the EEC may select one or more EASs to enable AC communication with one of the selected EASs. The selection algorithm is outside the scope of this specification. Once the EAS is selected, the EEC may subscribe for the ACR event notifications at the EES of the selected EAS, as described in clause 8.8.3.5.2. The EDN configuration information received from ECS may be used for establishing a connection to EAS(s).

EAS discovery may be initiated by the EEC when a certain trigger condition at the UE is met. Some examples are as follows:

-	AC related updates available at the EEC (e.g. due to AC installation/re-installation/activation), AC requesting application server access;

-	Lifetime received via EAS discovery response specified in clause 8.5.3 is expired; or

-	EEC detects the need of application context relocation as in clause 8.8.

NOTE:	When the EEC decides to perform EAS discovery is up to EEC implementation.

When the bundle EASs register to different EESs, then the EEC send the EAS discovery request message to the associated EES for the bundle EAS information.

EAS discovery procedure also supports synchronization EAS retrieval which enables an EAS to obtain EAS(s) so that the request EAS can start communication with obtained EAS(s) for data synchronization.

* * * Next Change * * * *

8.5.2.2	Request-response model

Pre-conditions:

For EEC as consumer:

1.	The EEC has received information (e.g. URI, IP address) related to the EES;

2.	The EEC has received appropriate security credentials authorizing it to communicate with the EES as specified in clause 8.11; and

3.	The EES is configured with ECSP's policy for EAS discovery.

NOTE 1:	Details of ECSP's policy are out of scope.

For EAS as consumer to obtain synchronization EAS information:

1.	The requesting EAS is aware of EAS synchronization ID.

Figure 8.5.2.2-1: EAS Discovery procedure

1.	The EEC sends an EAS discovery request to the EES. The EAS discovery request includes the requestor identifier [EECID] along with the security credentials and may include EAS discovery filters and may also include UE location to retrieve information about particular EAS(s) or a category of EASs, e.g. gaming applications, or Edge Applications Server(s) available in certain service areas, e.g. available on a UE's predicted or expected route. The request may include an EAS selection request indicator or UE type.

The EAS may send EAS discovery request with EAS ID and EAS synchronization ID, which indicates the request to obtain synchronization EAS with the requested EAS ID.

2.	Upon receiving the request from the EEC, the EES checks if the EEC is authorized to discover the requested EAS(s). The authorization check may apply to an individual EAS, a category of EASs or to the EDN, i.e. to all the EASs. If UE's location information is not already available, the EES obtains the UE location by utilizing the capabilities of the 3GPP core network as specified in clause 8.10.3. If EAS discovery filters are provided by the EEC, the EES identifies the EAS(s) based on the provided EAS discovery filters and the UE location.

When the bundle EAS information is provided, then;

-	If bundle EAS information includes EAS bundle identifier, the EES identifies all or part of the EAS(s) associated the same EAS bundle identifier.

-	If bundle EAS information includes a list of EASIDs, the EES identifies the EASs which are all or part of the EAS bundle.

If the EEC indicates that service continuity support is required, the EES shall take the indication which ACR scenarios are supported by the AC and the EEC and which of these are preferred by the AC into consideration. The EES may select one EAS and determine whether to perform application traffic influence for this AC based on AC's service KPI or EAS’s service KPI in desired response time, when the EAS does not perform traffic influence in advance.

When EAS discovery filters are not provided, then:

-	if available, the EES identifies the EAS(s) based on the UE-specific service information at the EES and the UE location;

-	EES identifies the EAS(s) by applying the ECSP policy (e.g. based only on the UE location);

NOTE 2:	Details of the UE-specific service information and how it is available at the EES is out of scope.

NOTE 3:	Both steps are evaluated prior to sending a response.

Upon receiving the request from the EEC, if the EEC does not indicate EAS Instantiation Triggering Suppress in the EAS Discovery request, the EES may trigger the EAS management system to instantiate the EAS that matches with EAS discovery filter IEs (e.g. ACID) as in clause 8.12.

Otherwise, upon receiving the request from the EEC, if the EEC indicates EAS Instantiation Triggering Suppress in the EAS Discovery request and the EES supports such capability, the EES may determine Instantiable EAS Information for EAS(s) that are instantiable but not yet instantiated and match the EAS discovery filter IEs. Instantiable EAS Information is provided in the EAS Discovery response and includes the EASID(s) and, for each EASID, the status indicating wether the EAS is instantiated or instantiable but not yet instantiated.

If the EEC provides in the EAS discovery request the EAS selection request indicator, the EES selects EAS satisfying the EAS discovery filter or based on other information (e.g. ECSP policy) as described above (if no EAS discovery filter received), and then provides the selected EAS information to the EEC in the discovered EAS list of EAS discovery response.

NOTE 4:	Without EAS selection request indication, the EES handling is as per R17 procedure.

If EAS synchronization ID is received and the ECS-ER is available, the EES checks with the ECS-ER with the received EAS ID and EAS synchronization ID and obtains a list of EAS(s) with the EAS synchronization ID for the desired application service identified by the EAS ID as described in clause 8.Y.

3.	If the processing of the request was successful, the EES sends an EAS discovery response to the EEC, which includes information about the discovered EASs and Instantiable EAS Information. For discovered EASs, this includes endpoint information. Depending on the EAS discovery filters received in the EAS discovery request, the response may include additional information regarding matched capabilities, e.g. service permissions levels, KPIs, AC locations(s) that the EASs can support, ACR scenarios supported by the EAS, etc. The EAS discovery response may contain a list of EASs and Instantiable EAS Information. This list may be based on EAS discovery filters containing a Geographical or Topological Service Area, e.g. a route, included in the EAS discovery request by the EEC.

If the EES is unable to determine the EAS information using the inputs in the EAS discovery request, UE-specific service information at the EES or the ECSP policy, the EES shall reject the EAS discovery request and respond with an appropriate failure cause.

If the EEC is not registered with the EES, and ECSP policy requires the EEC to perform EEC registration prior to EAS discovery, the EES shall include an appropriate failure cause in the EAS discovery response indicating that EEC registration is required.

If the UE location and predicted/expected UE locations, provided in the EAS discovery request, are outside the Geographical or Topological Service Area of an EAS, then the EES shall not include that EAS in the discovery response. The discovery response may include EAS(s) that cannot serve the UE at its current location if a predicted/expected UE location was provided in the EAS discovery request.

For responding EAS requesting synchronization EAS, only EAS endpoint and EAS ID are included in EAS profile of Discovered EAS list.

Upon receiving the EAS discovery response, if the EEC selects an EAS which is instantiated (i.e., an EAS profile was provided), the EEC uses the endpoint information for routing of the outgoing application data traffic to EAS(s), as needed, and may provide necessary notifications to the AC(s). The EEC may use the border or overlap between EAS Geographical Service Areas for service continuity purposes. The EEC may cache the EAS information (e.g. EAS endpoint) for subsequent use and avoid the need to repeat step 1. If the Lifetime IE is included in the response, the EEC may cache the EAS information only for the duration specified by the Lifetime IE.

Upon receiving the EAS discovery response, if the EEC selects an EAS which is not instantiated (i.e. an EAS profile is not provided), the EEC sends the EAS information provisioning request indicating the selected EASID as in clause 8.15.

NOTE 5:	Within the duration specified by the Lifetime IE, the cached EAS Profile can be updated (e.g. according to notifications from the EES for changes of EAS information due to EAS status change) or the cached EAS Profile can be invalidated due to new EAS information discovery (e.g. due to UE mobility). The EEC can update or invalidate the cached EAS information (e.g. on PDU Session Release or Modification Command).

NOTE 6:	The AC can cache the EAS information (e.g. EAS endpoint) for subsequent use. In the case of the cached information needing to be updated or invalidated, the mechanisms for the EEC to notify the AC is up to implementation and is not specified in the current release of the present document.

NOTE 7:	The EEC can use the EAS information provided by the discovery procedure to perform service continuity planning, for example when ultra-low latency ACR is required.

If the EAS discovery request fails, the EEC may resend the EAS discovery request, taking into account the received failure cause. If the failure cause indicated that EEC registration is required, the EEC shall perform an EEC registration before resending the EAS discovery request.

NOTE 8:	As long as a proper EAS (e.g. considering expected AC service KPIs included in EAS discovery request) is discovered and selected by the EES, EEC of a constraint UE can stop sending EAS discovery to rest candidate EES(s), and provide the selected EAS information to AC.

* * * Next Change * * * *

8.X	EAS synchronization

8.X.1	General

It is possible for the ASP to provide EAS (with same service) in different EDNs. For example, a gaming service provider may have deployed game servers in different EDNs to serve its users. An EAS may synchronize the particular communication session among other EAS(s).

8.X.2	Procedure

8.X.2.1	General

Following procedures are specified for EAS synchronization:

-	Registration and other related operations for EAS synchronization ID; and

-	Discovering EAS for synchronization.

8.X.2.2	Registration

8.X.2.2.1	General

EAS may dynamically choose with which EAS(s) it synchronizes the communication session based on service-specific requirements. Such dynamic synchronization group information, typically indicated by EAS synchronization ID, is registered to EES and managed separately from EAS profile, which usually contains static information.

8.X.2.2.2	EAS synchronization ID registration

Figure 8.X.2.2.2-1 illustrates the procedure for EAS synchronization ID registration.

Pre-conditions:

1.	EAS can communicate with EES.

Figure 8.X.2.2.2-1: EAS synchronization ID registration

1.	The EAS sends an EAS synchronization ID registration request to the EES. The request includes the EASID, EAS's security credentials and EAS synchronization ID.

2.	The EES checks EAS's security credentials and validates the request. When the ECS with repository function is available, the EES interacts with the ECS-ER to store the EAS synchronization ID.

3.	If the request is successfully validated, the EES registers the information provided in the request and responds back to the EAS with EAS synchronization ID registration response.

8.X.2.2.3	EAS synchronization ID registration update

Figure 8.X.2.2.3-1 illustrates the procedure for EAS synchronization ID registration update.

Pre-conditions:

1.	EAS can communicate with EES.

Figure 8.X.2.2.3-1: EAS synchronization ID registration update

1.	The EAS sends a EAS synchronization ID registration update request to the EES. The request includes the EASID, EAS's security credentials and updated EAS synchronization ID.

2.	The EES checks EAS's security credentials and validates the request. When the ECS with repository function is available, the EES interacts with the ECS-ER to store the EAS synchronization ID.

3.	If the request is successfully validated, the EES registers the updated information provided in the request and responds back to the EAS with EAS synchronization ID registration update response.

8.X.2.2.4	EAS synchronization ID de-registration

Figure 8.X.2.2.4-1 illustrates the procedure for EAS synchronization ID de-registration.

Pre-conditions:

1.	EAS can communicate with EES.

Figure 8.X.2.2.4-1: EAS synchronization ID de-registration

1.	The EAS sends an EAS synchronization ID de-registration request to the EES. The request includes the EASID and EAS's security credentials.

2.	The EES checks EAS's security credentials and validates the request. When the ECS with repository function is available, the EES interacts with the ECS-ER to delete the EAS synchronization ID.

3.	If the request is successfully validated, the EES de-registers the EAS synchronization ID of the EAS and responds back to the EAS with EAS synchronization ID de-registration response.

8.X.3	Information flows

8.X.3.1	General

The following information flows are specified for EAS synchronization:

-	EAS synchronization ID registration request and response;

-	EAS synchronization ID registration update request and response; and

-	EAS synchronization ID de-registration request and response.

8.X.3.2	EAS synchronization ID registration request

Table 8.X.3.2-1 describes information elements for the EAS synchronization ID registration request.

Table 8.X.3.2-1: EAS synchronization ID registration request

| Information element | Status | Description |
| --- | --- | --- |
| EASID | M | Identifier of the EAS |
| Security credentials | M | Security credentials of the EAS sending the registration request. |
| EAS synchronization ID | M | Identifier of the EAS group for synchronization. |
| Proposed expiration time | O | Proposed expiration time for the registration |

8.X.3.3	EAS synchronization ID registration response

Table 8.X.3.3-1 describes information elements for the EAS synchronization ID registration response.

Table 8.X.3.3-1: EAS synchronization ID registration response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the registration request was successful. |
| > Registration ID | M | Identifier of the registration. |
| > Expiration time | O | Indicates the expiration time of the registration. To maintain an active registration status, a registration update is required before the expiration time. If the Expiration time IE is not included, it indicates that the registration never expires. |
| Failure response | O | Indicates that the registration request failed. |
| > Cause | O | Indicates the cause of registration request failure |

8.X.3.4	EAS synchronization ID registration update request

Table 8.X.3.4-1 describes information elements for the EAS synchronization ID registration update request.

Table 8.X.3.4-1: EAS synchronization ID registration request

| Information element | Status | Description |
| --- | --- | --- |
| Registration ID | M | Identifier of the registration. |
| Security credentials | M | Security credentials of the EAS |
| Updated EAS synchronization ID | M | Updated identifier of the EAS group for synchronization. |
| Proposed expiration time (NOTE) | O | Proposed expiration time for the registration |
| NOTE: At least one of the IEs is included. |  |  |

8.X.3.5	EAS synchronization ID registration response

Table 8.X.3.5-1 describes information elements for the EAS synchronization ID registration response.

Table 8.X.3.5-1: EAS synchronization ID registration response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the registration request was successful. |
| > Expiration time | O | Indicates the expiration time of the registration. To maintain an active registration status, a registration update is required before the expiration time. If the Expiration time IE is not included, it indicates that the registration never expires. |
| Failure response | O | Indicates that the registration request failed. |
| > Cause | O | Indicates the cause of registration request failure |

8.X.3.6	EAS synchronization ID de-registration request

Table 8.X.3.6-1 describes information elements for the EAS synchronization ID de-registration request.

Table 8.X.3.6-1: EAS synchronization ID de-registration request

| Information element | Status | Description |
| --- | --- | --- |
| Registration ID | M | Identifier of the registration. |
| Security credentials | M | Security credentials of the EAS |

8.X.3.7	EAS synchronization ID de-registration response

Table 8.X.3.7-1 describes information elements for the EAS synchronization ID de-registration response.

Table 8.X.3.7-1: EAS synchronization ID de-registration response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the de-registration request was successful. |
| Failure response | O | Indicates that the de-registration request failed. |
| > Cause | O | Indicates the cause of de-registration request failure |

* * * Next Change * * * *

8.Y	Interaction with ECS with Repository function

8.Y.1	General

This clause describes procedures and services provided by the ECS with Repository function (ECS-ER), which is applicable when the ECS-ER is available.

Editor's Note:	The ECS with Repository function and its relationship with ECS-ER is FFS in the whole TS.

8.Y.2	Procedure

8.Y.2.1	General

Clause 8.Y.2.2, clause 8.Y.2.3 and clause 8.Y.2.4 illustrate the synchronization EAS information retrieval procedures.

8.Y.2.2	Get synchronization EAS

Figure 8.Y.2.2-1: Obtain synchronization EAS

- The EES sends synchronization EAS information get request message to the ECS-ER. The request message includes EAS ID and EAS synchronization ID.

- Upon receiving the request, the ECS-ER checks if there are any stored synchronization EAS(s) for the EAS ID.

- If the processing of the request was successful, the ECS-ER responds the EES with synchronization EAS information identified in step 2.

8.Y.3	Information flows

8.Y.3.1	General

The information flows are specified for common EAS information service.

8.Y.3.2	Synchronization EAS information get request

Table 8.Y.3.2-1 describes the information elements for synchronization EAS information get request from the EES to the ECS-ER.

Table 8.Y.3.2-1: Synchronization EAS information get request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the EES. |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| EAS ID | M | The identifier of the EAS. |
| EAS synchronization ID | M | Identity of the group of EAS for synchronization. |

8.Y.3.3	Synchronization EAS information get response

Table 8.Y.3.3-1 describes the information elements for synchronization EAS information get response from the ECS-ER to the EES.

Table 8.Y.3.3-1: Synchronization EAS information get response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response (NOTE) | O | Indicates that the request was successful. |
| > synchronization EAS information | M | This IE includes a list of synchronization EAS endpoints. |
| Failure response (NOTE) | O | Indicates that the request has failed. |
| > Cause | O | Indicates the failure cause. Only included when Failure response is included. |
| NOTE: One of these IEs shall be present in the message. |  |  |