3GPP TSG-SA WG6 Meeting #55	S6-232134

Berlin, Germany 22nd – 26th May 2023	(revision of S6-232028)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0328 | rev | 5 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | More procedures with CES |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-05-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In case CES is deployed, it can be used to support service exposure to CAS and facilitate EEC context relocation. The CAS discovery can use traditional DNS in case of no ENS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the architecture figure with reference point change. Update reference point explanation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No procedure support for ACR btw. CAS and EAS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Existing clauses: 6.15.16, 6.5.17, 6.5.18, 6.5.19, 8.8.2B.1 New clauses: 8.8.2B.2, 8.8.2B.2.1, 8.8.2B.2.2, 8.8.2B.2.3, 8.8.2B.2.4, 8.8.2B.2.5, 8.8.2B.3, 8.8.2B.3.1, 8.8.2B.3.2, 8.8.2B.3.3, 8.8.2B.3.4, 8.8.2B.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev3: added ACR btw. CASs and more procedures btw. edge & cloud with CES. Rev4: removed ACR btw. CASs. |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 6.5.16	ECI-3

ECI-3 enables interaction between CES and ECS.

ECI-3 supports:

a)	T-EES retrieval procedure.

### 6.5.17	ECI-4

ECI-4 enables interaction between CES and EES.

ECI-4 supports:

a)	EEC context relocation procedure; and

b)	ACR parameter information procedure.

### 6.5.18	CLOUD-1

CLOUD-1 enables interaction between CAS and CES.

CLOUD-1 supports:

a)	providing access to network capability information (e.g. location information);

b)	requesting the setup of a data session between AC and CAS with a specific QoS and receiving QoS related information; and

c)	service continuity procedures (e.g. ACR status).

### 6.5.19	CLOUD-2

CLOUD-2 enables interaction between CES and CES.

Editor's note:	What functionalities of EDGE-9 are reused for CLOUD-2 is FFS.

* * * Next Change * * * *

#### 8.8.2B.1	General

Since the EAS may have service area restriction, once the UE is moving out of the current edge coverage, to keep service continuity, the application client needs to communicate with the CAS.

CAS(s) are registered in the CES via CLOUD-1 reference point which enables the CES to provide appropriate CAS to S-EAS or S-EES.

DNS can be used for CAS discovery by the UE.

The EES of the source EDN may interact with the CES via ECI-4 reference point and application context is transferred from source EAS to the CAS. Later, if the UE is moving to an area with edge coverage, the CES may interact with the EES in the target EDN via ECI-4 reference point and application context is transferred from the CAS to target EAS.

* * * Next Change * * * *

#### 8.8.2B.2	ACR from edge to cloud

##### 8.8.2B.2.1	General

The following clauses describe ACR scenarios from edge to cloud.

##### 8.8.2B.2.2	Initiation by EEC using regular EAS Discovery

The scenario described in clause 8.8.2A.2 applies with the following differences:

-	After step 7, the S-EES relocates context to the CES. The relocation procedure re-uses the procedure described in clause 8.9.2.3.

-	During post-ACR clean up, the CAS triggers ACR status update towards the CES in order to finish ACR.

##### 8.8.2B.2.3	EEC executed ACR via S-EES

The scenario described in clause 8.8.2A.3 applies with the following differences:

-	After step 4, the S-EES relocates context to the CES. The relocation procedure re-uses the procedure described in clause 8.9.2.3.

-	During post-ACR clean up, the CAS triggers ACR status update towards the CES in order to finish ACR.

##### 8.8.2B.2.4	S-EAS decided ACR

The scenario described in clause 8.8.2.4 applies with the following differences:

-	The CES replaces the T-EES and the CAS replaces the T-EAS.

##### 8.8.2B.2.5	S-EES executed ACR

The scenario described in clause 8.8.2.5 applies with the following differences:

-	The CES replaces the T-EES and the CAS replaces the T-EAS.

#### 8.8.2B.3	ACR from cloud to edge

##### 8.8.2B.3.1	General

The following clauses describe ACR scenarios from cloud to edge.

##### 8.8.2B.3.2	Initiation by EEC using regular EAS Discovery

The scenario described in clause 8.8.2.2 applies with the following differences:

-	The CES replaces the S-EES and the CAS replaces the S-EAS.

-	The EEC subscribes to ECS with service provisioning subscription for ACR detection and service provisioning in step 3 is skipped.

##### 8.8.2B.3.3	EEC executed ACR via CES

The scenario described in clause 8.8.2.3 applies with the following differences:

-	The CES replaces the S-EES and the CAS replaces the S-EAS.

-	The EEC subscribes to ECS with service provisioning subscription for ACR detection and service provisioning in step 3 is skipped.

##### 8.8.2B.3.4	CAS decided ACR

The scenario described in clause 8.8.2.4 applies with the following differences:

-	The CES replaces the S-EES and the CAS replaces the S-EAS.

-	The CAS subscribes to CES with ACR management events with "ACR monitoring" for ACR detection and step 3 T-EAS discovery is skipped.

##### 8.8.2B.3.5	CES executed ACR

The scenario described in clause 8.8.2.5 applies with the following differences:

-	The CES replaces the S-EES and the CAS replaces the S-EAS.

-	The ACR is only detected by the CES. The CAS subscribes to CES with ACR management events with "ACR facilitation" for ACR detection	and step 5a T-EAS discovery is skipped.

NOTE:	Detection by other entities is not specified in this release.

* * * END of Change * * * *