3GPP TSG-SA WG6 Meeting #55	S6-232167

Berlin, Germany 22nd – 26th May 2023	(revision of S6-232158, S6-231695)

Source:	Apple

Title:	Pseudo-CR on EDGEAPP and ETSI MEC platform and AS aligned deployment

Spec:	3GPP TR 23.958 v0.4.0

Agenda item:	8.12

Document for:	Approval

Contact:	Walter Featherstone, w_featherstone@apple.com

1. Introduction

Adopting a similar approach that was proposed in S6-231267 that depicted a combined Edge platform, this paper proposes a single EDGEAPP and ETSI MEC platform and application server aligned deployment.

2. Reason for Change

EDGEAPP and ETSI MEC alignment is one of the objectives in R18, where such alignments are expected to be captured in the External TR.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR <TS/TR number and version>.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

[3]		GSMA PRD OPG.02 - “Operator Platform Telco Edge Requirements Version 3.0”, https://www.gsma.com/futurenetworks/wp-content/uploads/2022/10/Operator-Platform-Telco-Edge-Requirements.-v.3-October22.pdf.

[4]	ETSI GS MEC 003 (V3.1.1), "Multi-access Edge Computing (MEC); Framework and Reference Architecture".

[5]	ETSI GS MEC 011 (V3.1.1), “Multi-access Edge Computing (MEC); Edge Platform Application Enablement”.

[6]	ETSI GS MEC 040 (V3.1.1), “Multi-access Edge Computing (MEC); Federation enablement APIs”.

[7]	ETSI GS MEC 010-2 (V2.2.1), "Multi-access Edge Computing (MEC); MEC Management; Part 2: Application lifecycle, rules and requirements management".

* * * Second Change * * * *

# 7	Deployment Considerations

This section captures different deployment possibilities of aligned EDGEAPP and ETSI MEC in an operator network and any other deployment considerations w.r.t EDGEAPP, ETSI MEC and GSMA OP as per TS 23.558.

## 7.1	General

These sections capture deployment of EDGEAPP and ETSI MEC architecture as an aligned edge systemin an operator network, their architectural requirements etc. as per TS 23.558.

## 7.2	Deployment  with ETSI MEC

Application Server

(EAS and/or

MEC app instance)

Edge platform

(EES + MEP)

EDGE-3

/ Mp1

EDGE-9 / Mp3

Application Server

(EAS and/or

MEC app instance)

Edge platform

(EES + MEP)

EDGE-3

/ Mp1

CAPIF-1(e) & CAPIF-2(e)

CAPIF-1(e) & CAPIF-2(e)

CAPIF-6(e)

Figure 7.2-1 shows the EDGEAPP and ETSI ISG aligned Application Server and platform deployment. The depicted Edge platform consists of functionalities provided by both EES and MEC platform. The depicted Application Server implements the functionalities of an Edge Application Server or MEC application instance or both.

Figure 7.2-1: EDGEAPP and ETSI MEC aligned Application Server and platform deployment.

EDGE-3 is provided to Application Servers presenting themselves to the Edge platform as Edge Application Servers. Mp1 is provided for Application Servers presenting themselves to the Edge platform as MEC application instances.

An Edge platform adopting the CAPIF framework could provide the capabilities offered by EDGE-3 and Mp1 through utilisation of CAPIF-1 (/CAPIF-1e) and CAPIF-2 (/CAPIF-2e) to provide a unified service. For instance, a mapping of the MEC service management API to the 3GPP CAPIF API is presented in Annex B of ETSI GS MEC 011 [5].

EDGE-9 and Mp3 provide services required for interconnectivity between Edge platforms. EDGE-9 provides the interconnectivity required by Edge Enabler Servers, whilst Mp3 provides the interconnectivity required by MEC platforms.

An Edge platform adopting the CAPIF framework could provide the capabilities offered by EDGE-9 and Mp3 through utilisation of CAPIF-6 (/CAPIF-6e) to provide a unified service.

NOTE: APIs supported over Mp3 have not currently been specified by ETSI MEC.

Management aspects relating to application server and platform management are captured in 3GPP TS 28.538 [2] (EDGEAPP entity specific) and ETSI GS MEC 010-2 [7] (MEC entity specific), where the commonality is that both specify ETSI NFV MANO for performing lifecycle management functions. Management related interfaces are not depicted in Figure 7.2-1.

* * * End of Changes * * * *