3GPP TSG-SA WG6 Meeting #55	S6-232171

Berlin, Germany 22nd – 26th May 2023	(revision of S6-232150)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0321 | rev | 6 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | EAS bundle types and bundle requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-05-16 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Following ENs: Editor's Note: Whether EAS bundle requirement is needed in EAS profile or EAS discovery filters is FFS. Editor's Note: Whether EAS bundle requirement is needed in EES profile is FFS. EAS bundle requirements are needed in the EAS profile and EES profile for indicating coordinated T-EAS discovery and coordinated ACR requirements during S-EAS and S-EES initiated ACR scenarios. EAS bundle type is required to distinguish between different types of EAS bundles. See detailed discussion in S6-231843. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added EAS bundle type as part of EAS bundle information. Added ‘EAS Bundle requirements’ IE in the EAS profile and the EES profile. Related ENs are also removed. The CR also corrects a mention of EES in place of ECS in the EES registration procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete functionality |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.10, 8.2.2, 8.2.4, 8.2.6, 8.4.4.2.2, 8.5.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 7.2.10	EAS bundle information

The EAS bundle information includes EAS bundle type, a list of EASIDs or a bundle ID and EAS bundle requirements. EAS bundle ID establishes an association between the EASs. When included in the EAS profile, EAS bundle ID denotes the bundle to which the EAS belongs. When included in the AC profile EAS bundle ID is used to perform different Edge Enabler Layer operations, such as EAS discovery. Edge Enabler Layer handles the EASs belonging to the same bundle as required by related EAS bundle requirements as described in clause 8.2.x.

NOTE 1 :	Both, EAS bundle ID and EAS bundle requirements, are provided by the ASP.

NOTE 2:	Bundle ID is necessary when the affinity between bundled EASs is strong (e.g., co-deployment and co-migration is essential), and the related ASPs which provide the AC and bundled EASs established the bundle. List of EASIDs is required when the affinity between the bundled EASs is weak (e.g., co-deployment and co-migration is only "nice to have").

NOTE 3:	Following types of EAS bundles are considered in this release:

-	Direct bundle, where AC interacts with multiple EASs of the EAS bundle directly with no coordination between the EASs; and

-	Proxy bundle, where the AC interacts with one EAS of the EAS bundle which in turn coordinates with other EASs of the EAS bundle to provide services to the AC.

* * * Next Change * * * *

### 8.2.2	AC Profile

An AC Profile includes information about AC used to determine services and service characteristics required.

Table 8.2.2-1: AC Profile

| Information element | Status | Description |
| --- | --- | --- |
| ACID | M | Identity of the AC. |
| AC Type | O | The category or type of AC (e.g. V2X). This is an implementation specific value. |
| Preferred ECSP list | O | When used in a service provisioning request, this IE indicates to the ECS which ECSPs are preferred for the AC. The ECS may use this information in the selection of EESs. |
| AC Schedule | O | The expected operation schedule of the AC (e.g. time windows) |
| Expected AC Geographical Service Area | O | The expected location(s) (e.g. route) of the hosting UE during the AC's operation schedule. This geographic information can express a geographic point, polygon, route, signalling map, or waypoint set. |
| AC Service Continuity Support | O | Indicates if service continuity support is required or not for the application. The IE indicates which ACR scenarios are supported by the AC and which of these are preferred by the AC, also indicates the AC ability of handling bundled EAS ACR. |
| Simultaneous EAS connectivity information in service continuity | O | Indicates if simultaneous EAS connectivity is needed and the inactive time guidance for keeping connectivity towards the S-EAS. |
| List of EASs | O | List of EAS that serve the AC along with the service KPIs required by the AC |
| > EASID | M | Identifier of the EAS |
| > Expected AC Service KPIs | O | KPIs expected in order for ACs to receive currently required services from the EAS, as described in Table 8.2.3-1 |
| > Minimum required AC Service KPIs | O | Minimum KPIs required in order for ACs to receive meaningful services from the EAS, as described in Table 8.2.3-1 |
| List of required EAS bundle information | O | Information related to the EAS bundles which AC requires. |
| > Bundle ID or list of EASID | M | A list of EASIDs or a bundle ID as described in clause 7.2.10. |
| > Bundle type | M | Type of the EAS bundle as described in clause 7.2.10 |
| > EAS bundle requirements | O | Requirements associated with the EAS bundle as described in clause 8.2.10. |

Editor's Note: It is FFS whether and how EAS endpoint information may be included in the AC Profile.

* * * Next Change * * * *

### 8.2.4	EAS Profile

Table 8.2.4-1: EAS Profile

| Information element | Status | Description |
| --- | --- | --- |
| EASID | M | The identifier of the EAS |
| EAS Endpoint | M | Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the EAS. This information maybe discovered by EEC and exposed to ACs so that ACs can establish contact with the EAS. |
| List of EAS bundle information | O | List of EAS bundles to which the EAS belongs and related bundling requirements. |
| > Bundle ID | M | Bundle ID as described in clause 7.2.10. |
| > Bundle type | M | Type of the EAS bundle as described in clause 7.2.10 |
| > EAS bundle requirements | O | Requirements associated with the EAS bundle as described in clause 8.2.10. |
| ACID(s) | O | Identifies the AC(s) that can be served by the EAS |
| EAS Provider Identifier | O | The identifier of the ASP that provides the EAS. |
| Allowed MNO information | O | Information of the allowed operator (e.g. MNO name, PLMN ID) from which its subscriber can consume the EAS services |
| EAS Type | O | The category or type of EAS (e.g. V2X) |
| EAS description | O | Human-readable description of the EAS |
| EAS Schedule | O | The availability schedule of the EAS (e.g. time windows) |
| EAS Geographical Service Area | O | The geographical service area that the EAS serves. ACs in UEs that are located outside that area shall not be served. |
| EAS Topological Service Area | O | The EAS serves UEs that are connected to the Core Network from one of the cells included in this service area. ACs in UEs that are located outside this area shall not be served. See possible formats in Table 8.2.7-1. |
| EAS Service KPIs | O | Service characteristics provided by EAS, detailed in Table 8.2.5-1 |
| EAS service permission level | O | Level of service permissions e.g. trial, gold-class supported by the EAS |
| EAS Feature(s) | O | Service features e.g. single vs. multi-player gaming service supported by the EAS |
| EAS Service continuity support | O | Indicates if the EAS supports service continuity or not. This IE indicates which ACR scenarios are supported by the EAS, also indicates the EAS ability of handling bundled EAS ACR. |
| EAS Transport layer service continuity support | O | This IE indicates the EAS service continuity support for seamless transport layer (e.g. TCP/TLS/QUIC) relocation |
| General context holding time duration (NOTE) | O | The time duration that the EAS holds the context before the AC connects to the EAS in case of ACR for service continuity planning. It is an indication of the time the EAS holds the application context for a UE to move to its service area after receiving an ACR notification from the EES following an ACR request from the EEC. |
| List of EAS DNAI(s) | O | DNAI(s) associated with the EAS. This IE is used as Potential Locations of Applications in clause 5.6.7 of 3GPP TS 23.501 [2]. It is a subset of the DNAI(s) associated with the EDN where the EAS resides. |
| List of N6 Traffic Routing requirements | O | The N6 traffic routing information and/or routing profile ID corresponding to each EAS DNAI. |
| EAS Availability Reporting Period | O | The availability reporting period (i.e. heartbeat period) that indicates to the EES how often it needs to check the EAS's availability after a successful registration. |
| EAS Status | O | The status of the EAS (e.g. enabled, disabled, etc.) |
| NOTE: Since the EASID of the EAS identifies the type of the application (e.g. SA6Video, SA6Game etc) as described in clause 7.2.4, "General context holding time duration" determined by EAS can depend on the EASID (type of the application). |  |  |

NOTE:	The EAS Transport layer service continuity support can be used in EAS discovery, e.g. as described in 3GPP TS 23.433 [26] for SEALDD server acting as EAS, which can further support the EAS IP replacement function.

* * * Next Change * * * *

### 8.2.6	EES Profile

The EES profile includes information about the EES and the services it provides.

Table 8.2.6-1: EES Profile

| Information element | Status | Description |
| --- | --- | --- |
| EESID | M | The identifier of the EES |
| EES Endpoint | M | Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the EES. This information is provided to the EEC to connect to the EES. |
| EASIDs | M | List of EASIDs registered or expected to be registered with the EES. |
| > List of EAS bundle information | O | List of EAS bundles to which the EAS belongs and related bundling requirements. |
| >> Bundle ID | M | A bundle ID as described in clause 7.2.10. |
| >> Bundle type | M | Type of the EAS bundle as described in clause 7.2.10 |
| >> EAS bundle requirements | O | Requirements associated with the EAS bundle as described in clause 8.2.10. |
| Instantiable EAS information | O | The EAS instantiation status per EASID (e.g. instantiated, instantiable but not be instantiated yet). |
| > Instantiation criteria (see NOTE) | O | The criteria upon which EAS can be instantiated (e.g. based on specific date and time). |
| EEC registration configuration | M | Indicates whether the EEC is required to register on the EES to use edge services or not. |
| ECSP ID | O | The identifier of the ECSP that provides the EES. |
| EES Topological Service Area | O | The EES serves UEs that are connected to the Core Network from one of the cells included in this service area. EECs in UEs that are located outside this area shall not be served. See possible formats in Table 8.2.7-1. |
| EES Geographical Service Area | O | The area being served by the EES in Geographical values (as specified in clause 7.3.3.3) |
| List of EES DNAI(s) | O | DNAI(s) associated with the EES. This IE is used as Potential Locations of Applications in clause 5.6.7 of 3GPP TS 23.501 [2]. It is a subset of the DNAI(s) associated with the EDN, where the EES resides. |
| EES Service continuity support | O | Indicates if the EES supports service continuity or not. This IE indicates which ACR scenarios are supported by the EES, also indicates the EES ability of handling bundled EAS ACR. |
| NOTE: "Instantiation criteria" IE shall be present only when the value of "Instantiable EAS information" IE is "instantiable but not be instantiated yet". |  |  |

NOTE:	The list of EES DNAI(s) can include the DNAI(s) of the EAS(s) registered with the EES.

* * * Next Change * * * *

##### 8.4.4.2.2	EES registration

Pre-conditions:

1.	The EES has the address (e.g. URI) of the ECS;

2.	Both, the EES and ECS, have the necessary credentials to enable communications.

Figure 8.4.4.2.2-1: EES Registration procedure

1.	The EES sends the EES registration request to the ECS. The request from the EES includes the EES profile and EES security credentials. The request may include a proposed expiration time for the registration.

2.	Upon receiving the request from the EES, the ECS verifies the security credentials of the EES and stores the EES registration information obtained in step 1. If the EES profile includes bundle ID, the ECS stores the information and associates the EES with other EESs providing the same EAS bundle information.

3.	The ECS sends an EES registration response indicating success or failure of the registration operation. The ECS may provide an updated expiration time to indicate to the EES when the registration will automatically expire. To maintain the registration, the EES shall send a registration update request prior to the expiration time. If a registration update request is not received prior to the expiration time, the ECS shall treat the EES as implicitly de-registered.

* * * Next Change * * * *

#### 8.5.3.2	EAS discovery request

Table 8.5.3.2-1 describes information elements for the EAS discovery request. Table 8.5.3.2-2 provides further detail about the EAS Discovery Filter information element.

Table 8.5.3.2-1: EAS discovery request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The ID of the requestor (e.g. EECID) |
| UE Identifier | O | The identifier of the UE (i.e. GPSI or identity token) |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| EAS discovery filters | O | Set of characteristics to determine required EASs, as detailed in Table 8.5.3.2-2. |
| UE location | O | The location information of the UE. The UE location is described in clause 7.3.2. |
| Serving MNO information (NOTE 2) | O | The serving MNO information (e.g. MNO name, PLMN ID) which is serving the subscriber. |
| Target DNAI (NOTE 1) | O | Target DNAI information which can be associated with potential T-EAS(s) |
| EEC Service Continuity Support | O | Indicates if the EEC supports service continuity or not. The IE also indicates which ACR scenarios are supported by the EEC or, if this message is sent by the EEC to discover a T-EAS, which ACR scenario(s) are intended to be used for the ACR. |
| EES Service Continuity Support (NOTE 1) | O | The IE indicates if the S-EES supports service continuity or not. The IE also indicates which ACR scenarios are supported by the S-EES or, if the EAS discovery is used for an S-EES executed ACR according to clause 8.8.2.5, which ACR scenario is to be used for the ACR. |
| EAS Service Continuity Support (NOTE 1) | O | The IE indicates if the S-EAS supports service continuity or not. The IE also indicates which ACR scenarios are supported by the S-EAS or, if the EAS discovery is used for an S-EAS decided ACR according to clause 8.8.2.4, which ACR scenario is to be used for the ACR. |
| EAS Instantiation Triggering Suppress | O | Indicates to the EES that EAS instantiation triggering should not be performed for the current request. |
| EAS selection request indicator | O | Indicates the request for EAS selection support from the EES (e.g., for constrained device). |
| NOTE 1: This IE shall not be included when the request originates from the EEC. NOTE 2: This IE shall be included if edge node sharing is used. |  |  |

Table 8.5.3.2-2: EAS discovery filters

| Information element | Status | Description |
| --- | --- | --- |
| List of AC characteristics (NOTE 1) | O | Describes the ACs for which a matching EAS is needed. |
| > AC profile (NOTE 2) | M | AC profile containing parameters used to determine matching EAS. AC profiles are further described in Table 8.2.2-1. |
| List of EAS characteristics (NOTE 1, NOTE 3) | O | Describes the characteristic of required EASs. |
| > EASID | O | Identifier of the required EAS. |
| > Bundle ID or list of EASID | O | A list of EASIDs or a bundle ID as described in clause 7.2.10. |
| > Bundle type (NOTE 4) | O | Type of the EAS bundle as described in clause 7.2.10 |
| > EAS bundle requirements (NOTE 4) | O | Requirements associated with the EAS bundle as described in clause 8.2.10. |
| > EAS provider identifier | O | Identifier of the required EAS provider |
| > EAS type | O | The category or type of required EAS (e.g. V2X) |
| > EAS schedule | O | Required availability schedule of the EAS (e.g. time windows) |
| > EAS Geographical Service Area | O | Location(s) (e.g. geographical area, route) where the EAS service should be available. |
| > EAS Topological Service Area | O | Topological area (e.g. cell ID, TAI) for which the EAS service should be available. See possible formats in Table 8.2.7-1. |
| > Service continuity support | O | Indicates if the service continuity support is required or not. |
| > Service permission level | O | Required level of service permissions e.g. trial, gold-class |
| > Service feature(s) | O | Required service features e.g. single vs. multi-player gaming service |
| NOTE 1: Either "List of AC characteristics" or "List of EAS characteristics" shall be present. NOTE 2: "Preferred ECSP list" IE shall not be present. NOTE 3: The "List of EAS characteristics" IE must include at least one optional IE, if used as an EAS discovery filter. NOTE4: When EAS discovery request is sent by the EEC, this IE shall not be included. |  |  |