3GPP TSG-SA WG6 Meeting #57	S6-232950

Xiamen, China 9th – 13th October 2023	(revision of S6-23xxxx)

Source:	Lenovo

Title:	Key Issue on improving ML model/analytics correctness

Spec:	3GPP TR 23.700-82 v0.0.0

Agenda item:	8.3

Document for:	Approval

Contact:	Emmanouil Pateromichelakis <epateromiche@lenovo.com>

1. Introduction

This paper proposes a new key issue on improving ML model or application layer analytics correctness.

2. Reason for Change

Correctness of predictions is usually associated to accuracy, which represents the most prominent KPI to rate ML models. However, the accuracy can be corrupted by a wrong calculation and/or unfair feedback collection. It is thus of utmost importance to ensure that the accuracy is meaningful, consistent, and comparable.

Incorrect predictions can be due to the fact that the accuracy of an ML model during inference may be lower than the accuracy of the same ML model during training. This is likely to happen if the training data set differs significantly in terms of distribution and features from the input data that the ML model is fed with during inference. Knowing how the ML model is meant to be used would help to train the ML model efficiently and effectively. In such cases, a possible drift may be due to an issue of the data source, and such issue may not be in the control of MNO to examine whether the data source itself (which can be trusted or untrusted) provides correct inference data.

The following aspects need to be studied:

-	How and what information is required to compute and represent the correctness of ADAE analytics or the the application-layer ML model in a reliable way?

-	Whether and how the AF/Enabler layer can improve correctness of ADAE Analytics or the application-layer ML model?

-	How to detect possible drift due to data source abnormal / unreliable behavior, (i.e., from a data source that is not in the control of the operator) at the application enabler layer side?

-	How to dynamically re-act to the detection of a drift caused by an abnormal data source to ensure minimum impact to the ADAE analytics service accuracy?

3. Proposal

It is proposed to agree the new key issue for 3GPP TR 23.700-82 v 0.0.0.

* * * * First Change * * * *

# 5	Key issues

## 5.x	Key issue #x: Key issue on improving ML model/analytics correctness

Incorrect predictions can be due to the fact that the accuracy of an ML model during inference may be lower than the accuracy of the same ML model during training. This is likely to happen if the training data set differs significantly in terms of distribution and features from the input data that the ML model is fed with during inference. Knowing how the ML model is meant to be used would help to train the ML model efficiently and effectively. In such cases, a possible drift may be due to an issue of the data source, and such issue may not be in the control of MNO to examine whether the data source itself (which can be trusted or untrusted) provides correct inference data.

The following aspects need to be studied:

-	How and what information is required to compute and represent the correctness of ADAE analytics or the the application-layer ML model in a reliable way?

-	Whether and how the AF/Enabler layer can improve correctness of ADAE Analytics or the application-layer ML model?

-	How to detect possible drift due to data source abnormal / unreliable behavior, (i.e., from a data source that is not in the control of the operator) at the application enabler layer side?

-	How to dynamically re-act to the detection of a drift caused by an abnormal data source to ensure minimum impact to the ADAE analytics service accuracy?

* * * * End of Change * * * *