3GPP TSG-SA WG6 Meeting #57	S6-233006

Xiamen, China 9th – 13th October 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0237 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update subscription service operation for SS_LocationInfoEvent |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEAL_Ph3 |  |  |  |  |  | Date: |  |  | 2023-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Currently, subscriptions for Location Information events can be created and deleted. The issue with this approach is that when a VAL server wants to update a subscription (e.g. to change the time between consecutive reports or the Location QoS), it has to delete the old subscription and create a new one. Depending on the order these operations are invoked, the VAL server may either receive duplicated notifications (create-new followed by delete-old) or may miss notifications (delete-old followed by create-new). Also, the CRUD (Create-Read-Update-Delete) paradigm used in the RESTful mapping in stage 3 does require to allow an update operation. Additionally, the unsubscribe procedure should target a specific Subscription Identifier (a VAL server could have several subscriptions). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Enable a Location Information subscription update procedure triggered by a VAL server using a Subscription ID (obtained during the subscription creation) which by the way is alredady refereced in some other information flows (see 9.3.2.7-1), but not yet introduced. Additionally, the unsubscribe request targets a specific subscription ID (in case the requestor has several active subscriptions). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment with stage 3 specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.2.6, 9.3.7.a (New), 9.3.2.X (New), 9.3.2.Y (New), 9.4.1, 9.4.3.1, 9.4.3.5 (New), 9.3.2.32 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * First Change * * * *

#### 9.3.2.6	Location information subscription response

Table 9.3.2.6-1 describes the information flow from the location management server to the VAL server or location management client for location information subscription response.

Table 9.3.2.6-1: Location information subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL server/VAL user or VAL UE |
| Subscription status | M | It indicates the subscription result |
| Subscription ID | O (NOTE) | Identifies the subscription |
| NOTE: This IE shall be present if subscription status is set to success |  |  |

* * * * Next Change * * * *

#### 9.3.2.7a	Location Information subscription update procedure

Figure 9.3.2.7a-1 illustrates the high level procedure for allowing a VAL server update an existing subscription about Location information at the location management server.

Pre-conditions:

-	the VAL server has an active Location information subscription (identified by Subscription ID) at the location management server.

Figure 9.3.2.7a-1: VAL service area identifier subscription update procedure

1.	When the VAL server decides to update an active Location information subscription, it sends a Location information subscription update request to the location management server including the information specified in table 9.3.2.X-1.

2.	If the VAL server is authorized to update the subscription, the location management server updates the Location information subscription.

3.	The location management server sends a Location information subscription update response to the VAL server including the information as specified in table 9.3.2.Y-1.

* * * * Next Change * * * *

#### 9.3.2.X	Location information subscription update request

Table 9.3.2.X-1 describes the information flow from the VAL server to the location management server for Location information subscription update request.

Table 9.3.2.X-1: Location information subscription update request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL server |
| Subscription ID | M | Identifies the subscription |
| Time between consecutive reports | O (NOTE 3) | It indicates the interval time between consecutive reports |
| Supplementary location information indication | O (NOTE 3) | Indicates that supplementary location information is required. |
| Location QoS | O (NOTE 2, NOTE 3). | Definition of the location Quality of Service for which the location information is requested (see NOTE 1). |
| NOTE 1: The definition of location QoS has been defined in clause 4.1b of TS 23.273 [50] and the clause 6.1.6.2.13 of TS 29.572 [51]. NOTE 2: The element is only applicable for the information flow from the VAL server to the location management server. NOTE 3: Only the IEs to be updated should be included. |  |  |

* * * * Next Change * * * *

#### 9.3.2.Y	Location information subscription update response

Table 9.3.2.Y-1 describes the information flow from the location management server to the VAL server for location area monitoring subscription modify response.

Table 9.3.2.Y-1: Location information subscription update response

| Information element | Status | Description |
| --- | --- | --- |
| Subscription update status | M | It indicates the result of the subscription update operation |

* * * * Next Change * * * *

## 9.4	SEAL APIs for location management

### 9.4.1	General

Table 9.4.1-1 illustrates the SEAL APIs for location management.

Table 9.4.1-1: List of SEAL APIs for location management

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_LocationReporting | Create_Trigger_Location_Reporting Update_Trigger_Location_Reporting Cancel_Trigger_Location_Reporting | VAL server | Request /Response |
| SS_LocationInfoEvent | Subscribe_Location_Info | VAL server | Subscribe/Notify |
|  | Unsubscribe_Location_Info | VAL server |  |
|  | Update_Location_Info_Subscription | VAL server |  |
|  | Notifiy_Location_Info | VAL server |  |
| SS_LocationInfoRetrieval | Obtain_Location_Info | VAL server | Request /Response |
| SS_LocationAreaInfoRetrieval | Obtain_UEs_Info | VAL server | Request/Response |
| SS_LocationMonitoring | Subscribe_Location_Monitoring | VAL server | Subscribe/Notify |
|  | Unsubscribe_Location_Monitoring | VAL server |  |
|  | Notify_Location_Monitoring_Events | VAL server |  |
|  | Subscribe_Location_Area_Monitoring | VAL server | Subscribe/Notify |
|  | Notify_Location_Area_Monitoring_Events |  |  |
|  | Update_Location_Area_Monitoring_Subscribe |  |  |
|  | Unsubscribe_Location_Area_Monitoring |  |  |
| SS_VALServiceAreaConfiguration | Configure_VAL_Service_Area | VAL server | Request/Response |
|  | Obtain_VAL_Service_Area | VAL server |  |
|  | Update_VAL_Service_Area | VAL server |  |
|  | Delete_VAL_Service_Area | VAL server |  |
|  | Subscribe_VAL_Service_Area_Change_Event | SEAL server | Subscribe/Notify |
|  | Notify_VAL_Service_Area_Change_Event | SEAL server |  |
|  | Unsubscribe_VAL_Service_Area_Change_Event | SEAL server |  |

* * * * Next Change * * * *

#### 9.4.3.1	General

API description: This API enables the VAL server to subscribe, receive and unsubscribe the UEs location information from the location management server over LM-S, as well as modify an active subscription.

* * * * Next Change * * * *

#### 9.4.3.5	Update_Location_Info_Subscribe

API operation name: Update_Location_Info_Subscription

Description: Updating an active subscription about the UEs location information.

Known Consumers: VAL server.

Inputs: See subclause 9.3.2.X

Outputs: See subclause 9.3.2.Y

See subclause 9.3.2.7a for the details of usage of this API operation.

* * * * Next Change * * * *

#### 9.3.2.32	Location information unsubscribe request

Table 9.3.2.32-1 describes the information flow from the VAL server or location management client to the location management server for location information unsubscribe request.

Table 9.3.2.32-1: Location information unsubscribe request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M | Identity of the VAL server or VAL UE’s requesting ID. |
| Subscription ID | M | Identifies the subscription to be deleted |

* * * * End of Changes * * * *