3GPP TSG-SA WG6 Meeting #57	S6-233069

Xiamen, China 9th – 13th October 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0245 | rev | 0 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NRM device triggering |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Convida Wireless LLC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI-19 |  |  |  |  |  | Date: |  |  | 2023-10-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TR 23.700-97 proposed in clause 5.4 a solution for Device triggering. Other TR solutions, e.g. BDT management, have been adopted in SEAL as NRM functionalities Introducing Device Triggering in NRM allows for a generic mechanism which can be used by any VAL servers and completes the normative work for this solution. Compared to the TR 23.700-97 solution, this proposal adds value-add functionality at the NRM for device triggerring, e.g. the ability to configure SEAL with time windows or area information used to determine validity conditions for sending the device trigger. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added procedure for device triggering in NRM. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Device triggering in SEAL on behalf of VAL applications is not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New: 14.3.x; 14.3.2.y1; 14.3.2.y2; 14.4.2.z |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR … CR … |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 14.3.x	Device triggering

#### 14.3.x.1	General

A service  may initiate a device trigger to a UE to cause it to establish a connection to the SEAL layer, to connect to a VAL server, to provide updated information, etc. The NRM Server may initiate the device trigger itself (implicit) or it may be initiated by a request that it receives from VAL Servers (explicit).

#### 14.3.x.2	Device Triggering via NRM procedure

Figure 14.3.x.2-1: Device Triggering via NRM Procedure

1). The device triggering procedure may be initiated based on an optional interaction between a VAL Server and the NRM Server/ For example, the VAL Server may send an explicit API request, or the VAL Server may send another message based on which the NRM Server determines to start the device triggering procedure.

2). The NRM Server determines the valid conditions to initiate the device triggering. The determination is based on request parameters as follows:

- If “Other UE availability information use” IE is present, the NRM Server includes UE monitoring requests and/ or other local policies to determine whether the conditions are valid.

- If “Time Window” IE is present, the NRM Server includes its value to limit the conditions considered as valid based on step a).

- If “Area information” IE is present, the NRM Server includes its value to limit the conditions considered as valid based on step b)

When valid conditions to initiate the device triggering have been met, the NRM Server determines whether aggregation of the device trigger with other device trigger requests is possible, e.g. if another server made a trigger request for the same UE which meets the same conditions. If aggregation is possible, the NRM server includes all valid payloads in the same device triggering request, to be initiated at the earliest time the aggregated conditions are met.

If the NRM Server also determines that other group device triggering requests received include the target of this request (e.g. the VAL group ID of another request has the VAL ID of this request as a member) and meet the validity conditions of this request, then it considers the group device triggering request to map to this request and determines to perform only the group device triggering request. A group device trigger request is considered mapped to another request only if the Trigger Purpose IE of the two requests is the same.

3).	The NRM Server performs the device triggering procedure described in 3GPP TS 23.682 [7] clause 5.2. The procedure requires that the UE Identifier, port number(s) and protocol information are available at the NRM Server. The trigger may be sent to ensure that a target UE in PSM mode is reachable when communications resume.

As part of the procedure, the NRM Server receives a Device Triggering delivery status report from SCEF/NEF indicating the success of the delivery.

4).	If a request was received in step 1, the NRM Server responds to the request.

Based on the trigger purpose derived from the payload, the targeted NRM Client or VAL Client performs the corresponding actions (e.g., connect to the VAL Server).

* * * Next Change * * * *

#### 14.3.2.y1	Device triggering request

Table 14.3.y1-1 describes the information flow for the Device triggering request from the VAL server to the NRM Server.

Table 14.3.2.y1-1: Device triggering request

| Information element | Status | Description |
| --- | --- | --- |
| Identifier | M | VAL UE ID or VAL group ID to be triggered. |
| Port number | M | Device triggering port number |
| Triggering purpose | M | Indicates the device triggering purpose which is included in the payload. The default is a reserved value e.g. NULL. |
| Other UE availability information use | O | Indicates whether other UE availability information may be used to determine when the device triggering procedure is performed. Values include “availability monitoring only”, “policy-based”, “availability monitoring and policy”. If this IE is not present it is assumed that only information provided in this request may be used for determination of the triggering timing. |
| Time window | O | Desired time window for the trigger |
| Area information | O | Desired geographical area for trigger. |
| Aggregation indication | O | Indicates whether aggregation with other device triggering requests is requested, forbidden or don’t care. If aggregation is allowed, the payload of the resulting device triggering request includes the aggregated Triggering purpose of all aggregated triggers. If aggregation is allowed, the IE may also include an integer as the maximum number of requests which may be aggregated, otherwise the maximum is based on payload constraints and local policies. |
| Grouping indication | O | Indicates whether mapping the requested device trigger to a VAL group trigger is allowed or not allowed. If allowed, the IE may include one or more equivalent VAL group IDs. If allowed, mapping is valid only for triggers with the same Triggering purpose, including a default (e.g. NULL) triggering purpose. Mapping an individual device trigger to a group trigger implies that if the NRM Server may consider the device triggering request as performed when a corresponding group trigger is performed within valid conditions. This avoids duplication of triggers. |

#### 14.3.2.y2	Device triggering response

Table 14.3.2.y2-1 describes the information flow for Device triggering response from the NRM server to the VAL server.

Table 14.3.2.y2-1: Device triggering response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result indicates success or failure of the operation. |

* * * Next Change * * * *

#### 14.4.2.z	Request_Device_Triggering

API operation name: Request_Device_Triggering

Description: Requesting device triggering.

Known Consumers: VAL server.

Inputs: See subclause 14.3.2.y1

Outputs: See subclause 14.3.2.y2

* * * End of Changes * * * *