3GPP TSG-SA WG6 Meeting #57	S6-233166

Xiamen, China, 9th – 13th October 2023	(revision of S6-232905)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.282 | CR | 0326 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Changing the criteria during an ongoing MCData ad hoc group data session (multiple MC systems) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | FRMCS_Ph5 |  |  |  |  |  | Date: |  |  | 2023-10-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Current standard allows to update the participants list of an ongoing adhoc group data session by changing the criteria within one MC system. The support when the adhoc group participants are hosted by multiple MC systems is not supported. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Message flows are extended and the procedure to change the criteria during an ongoing adhoc group data session in multiple MC system is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The list of ad hoc group data session participants cannot be modified during an ongoing group call by changing the criteria when the participants are in multiple MC systems. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (new) 7.17.2.20a, (new) 7.17.2.21a, (new) 7.17.3.2.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 7.17.2.20a	Modify ad hoc group data session criteria request (MCData server – MCData server)

Table 7.17.2.20a-1 describes the information flow Modify ad hoc group data session criteria request from the MCData server to the MCData server.

Table 7.17.2.20a-1: Modify ad hoc group data session criteria request

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The MCData ID of the MCData user who is requesting for modification of ad hoc group data session participants |
| Functional alias | O | The associated functional alias of the MCData user who is requesting for modification of ad hoc group data session participants |
| MCData ad hoc group ID | M | The MCData group ID of ad hoc group data data session whose participants list needs to be modified |
| Criteria to determine the list of participants | O | Carries the details of criteria or meaningful label identifying the criteria or the combination of both which will be used by the MCData server for determining the participants e.g., it can be a location-based criteria to invite participants in a particular area |

#### 7.17.2.21a	Modify ad hoc group data session criteria response (MCData server – MCData server)

Table 7.17.2.21a-1 describes the information flow Modify ad hoc group data session criteria response from the MCData server to the MCData server.

Table 7.17.2.21a-1 Modify Ad hoc group data session criteria response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The MCData ID of the MCData user who is requesting for modification of ad hoc group data session participants |
| Functional alias | O | The associated functional alias of the MCData user who is requesting for modification of ad hoc group data session participants |
| MCData ad hoc group ID | M | The MCData group ID of ad hoc group data data session whose participants list needs to be modified |
| Result | M | Result of the modify ad hoc group data data session participants request (success or failure) |

# * * * * Second change * * * *

##### 7.17.3.2.x	Modification of ad hoc group data session criteria by an authorized user

Figure 7.17.3.2.x-1 illustrates the modification of ad hoc group data session criteria procedure by the initiator of the ad hoc group data session between multiple MC systems.

Preconditions:

-	The MCData user at the MCData client 1 is authorized to modify the criteria.

-	The MCData server A and MCData server B are aware of the criteria sent by MCData client 1 related to the ongoing ad hoc group data session.

-	MC system A and MC system B are interconnected.

Figure 7.17.3.2.x-1: Modifying the criteria for determining the participants during an ongoing ad hoc group data session between multiple MC systems

1.	An ad hoc group data session has been established based on criteria sent by an authorized MCData client upon initiating the ad hoc group data session.

2.	The MCData user at the MCData client 1 is authorized and wishes to modify the criteria for determining the list of ad hoc group data session participants.

3a/b.	The MCData server A receives the modify ad hoc group data session criteria request and forwards the request to MCData server B at MC system B.

NOTE 1:	If MCData server A identifies that the criteria requires to add participants from another MC system then MCData server A sends an ad hoc group data session request to the MCData server in that MC system.

4a/b.	The MCData server A receives the ad hoc group data session criteria modify response and forwards the response to MCData client 1 in MC system A

5.	MCData server B detects that MCData client 3 meets the criteria for the ongoing ad hoc group data session initiated at MC system A.

5a.	MCData server B sends an ad hoc group data session add user notification message towards MCData server A.

5b.	MCData server A sends an ad hoc group data session request towards MCData client 3.

5c.	The MCData user 3 is notified of entering an ongoing ad hoc group data session.

5d.	MCData client 3 sends an ad hoc group data session response towards MCData server A.

5e.	MCData server B sends an ad hoc group data session remove user notification message towards MCData server A.

5f.	MCData server A sends an ad hoc group data session leave request towards MCData client 4.

5g.	The MCData user 4 is notified of leaving an ongoing ad hoc group data session.

5h.	MCData client 4 sends an ad hoc group data session leave response towards MCData server A.

6.	The MCData server A adds MCData client 3 to the ad hoc group data session and removes MCData client 4 from the ad hoc group data session.

NOTE 2:	If the ad hoc group data session is associated with an ad hoc group emergency alert and the change of criteria caused the modification of the ad hoc group data session participant list then the ongoing ad hoc group emergency alert is modified accordingly.

# * * * * End of changes * * * *