3GPP TSG-SA WG6 Meeting #57	draft S6-233351

Xiamen, China 9th – 13th October 2023	(revision of S6-232868)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.554 | CR | 0169 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | delivery mechanism for the recipient is not registered |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMARCH_Ph3 |  |  |  |  |  | Date: |  |  | 2023-09-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR is proposed to add a new delivery mechanism for the recipient is not registered to fulfill the requirement [R-5.1.2-005] specified in TS TS22.262, i.e. [R-5.1.2-005] The MSGin5G Service shall support storage of a message if a UE is unavailable (disconnected or power off) for future delivery once the UE becomes available. This delivery mechanism is different from the store and forward mechanism specified in clause 8.3.6 of 3GPP TS 23.554 since it applies in the terminating side and not triggered by the requirement from sender. . Some additional clarification on this new procedure: •S/F existing in 3GPP TS 23.554 is a feature required by the sender. The sender tells the MSGin5G Server that the message is needed to be stored if it can not be delivered immediately. This feature exists in some messaging mechanism, e.g. MQTT, the sender can request the Server to discard the message if the recipient is unavailable, or try to deliver the message more than one time. •Deferred delivery is depended on the service policy of the MSGin5G service provider, it will be applied for all messages regardless of the requirement from sender. This feature exists in some other messaging mechanism, e.g. SMS. The SMSC will store the SMS if the recipient is not attached until the time expired. Considering the coexistence of S/F and Deferred delivery, also considering the backward compatibility, the Deferred delivery will only be applied when S/F is not applied. |  |  |  |  |  |  |  |  |
| Summary of change: |  | add a new delivery mechanism for the recipient is not registered to fulfill the requirement [R-5.1.2-005] specified in TS TS22.262. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirement [R-5.1.2-005] specified in TS TS22.262 is not be fulfilled. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.x (new), 8.3.2. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

8.3.x	MSGin5G deferred message

This procedure applies when an MSGin5G message is received at the MSGin5G Server for delivery and the message cannot be delivered to the recipient since the recipient UE is unavailable (disconnected or power off), i.e. not registered; otherwise, the procedure detailed in clause 8.3.2 applies.

This procedure applies to the MSGin5G Server that is the home of the terminating MSGin5G Service ID based on the configuration from MSGin5G service provider. If the deferred message service is enabled, this procedure applies to all MSGin5G messages received at the MSGin5G Server for delivery.

Figure 8.3.x-1 shows the procedure for providing deferred message services.

Pre-conditions:

1.	MSGin5G message termination handling, the MSGin5G Servers deliver messages to an MSGin5G Service endpoint based on the terminating MSGin5G Service ID. For each MSGin5G Server in the path of delivery, it forwards the messages to the next MSGin5G Server until it reaches to the MSGin5G Server that is the home of the MSGin5G Service ID of the recipient.

2.	The MSGin5G message reaches to the MSGin5G Server that is the home of the terminating MSGin5G Service ID.

3.	The UE with the terminating MSGin5G Service ID is not registered.

Figure 8.3.x-1: MSGin5G deferred message procedure

1.	MSGin5G Server tries to deliver the MSGin5G message to the UE but either:

a)	no UE with the requested MSGin5G Service ID has registered; or

b)	the UE with the requested MSGin5G Service ID registered but is unavailable (e.g. disconnected or power off) for message delivery.

2.	The MSGin5G Server checks whether the value of the Store and forward flag IE (see Table 8.3.2-1) in the MSGin5G message indicates that store and forward services are requested by the sender. If the  store and forward services are requested by the sender, the procedure specified in clause 8.3.6 applies.

3.	If the store and forward services are not requested by the sender, the MSGin5G Server checks whether the deferred message service is enabled by the MSGin5G service provider. If the deferred message service is enabled, the MSGin5G Server stores the MSGin5G message and continues the rest steps in this clause, otherwise the MSGin5G Server discards the MSGin5G message and skipped all the rest steps except step 8b.

4.	Before either the Message expiration time is expired or the maximum deferred time is expired, the MSGin5G Server may trigger the Recipient UE based on the MSGin5G device triggering procedure in clause 8.9.3.

NOTE:	the the maximum deferred time is configured on the MSGin5G Server by the MSGin5G Service provider

5.	The MSGin5G Server may send a message response as defined in table 8.3.2-3 which includes deferred status information in the Delivery Status IE, e.g., that the delivery had been deferred.

6a.	The recipient UE, i.e. the UE with the requested MSGin5G Service ID, becomes available  for message delivery due to either:

a)	the UE with the requested MSGin5G Service ID has registered; or

b)	the UE with the requested MSGin5G Service ID registered and becomes available (e.g. connected or power on).

7a.	 the MSGin5G Server attempts delivery of the request using the procedure specified in clause 8.3.3.

8a.	The MSGin5G Server may send a message response as defined in table 8.3.2-3 which status information in the Delivery Status IE, e.g., that the message was discarded.

6b.	The recipient UE has not became available due to its registration before the Message expiration time is expired or the maximum deferred time is expired.

8b.	The MSGin5G Server discards the MSGin5G message and may send a message response as defined in table 8.3.2-3 which status information in the Delivery Status IE, e.g., that the message was discarded.

If the recipient UE has became available due to its registration before the Message expiration time is expired or the maximum deferred time is expired, step 6a, 7a and 8a apply, otherwise step 6b and 8b apply.

* * * Next Change * * * *

8.3.2	MSGin5G messages origination procedure

Figure 8.3.2-1 shows the procedure for an MSGin5G UE that initiates an MSGin5G message request.

Figure 8.3.2-1: New MSGin5G message request from UE

Figure 8.3.2-2 shows the procedure for an Application Server that initiates an API request specified in clause 9.1.1.1 for sending an MSGin5G message to UE.

Figure 8.3.2-2: Application Server initiates a request for sending an MSGin5G message

Figure 8.3.2-3 shows the procedure for a Legacy 3GPP Message Gateway or a non-3GPP Message Gateway that sends a new MSGin5G message request to the MSGin5G Server on behalf of a Legacy 3GPP UE or Non-3GPP UE.

Figure 8.3.2-3: New MSGin5G message request sending from Message Gateway

The following procedure applies to the above figures 8.3.2-1, 8.3.2-2 and 8.3.2-3 with the exception that step 1 only applies to figure 8.3.2-1.

1.	The Application Client in the UE sends a request to the MSGin5G Client for invoking the MSGin5G Client to send a new MSGin5G message to a recipient or to multiple recipients.

Editor's note:	Whether the APIs provided by the MSGin5G Client to the Application Client is to be specified in another clause of the TS is FFS.

2.	As shown in figure 8.3.2-1 or 8.3.2-3, the MSGin5G Client or Message Gateway sends the MSGin5G message request to the MSGin5G Server and includes the IEs as listed in table 8.3.2-1 in the request; or as shown in figure 8.3.2-2, the Application Server sends an API request to the MSGin5G Server for sending an MSGin5G message, the API request includes the IEs as listed in table 8.3.2-1.

NOTE:	If the value of the Store and forward flag IE in the MSGin5G message request indicates that store and forward services are requested by the sender, the procedure in 8.3.6 applies instead.

Table 8.3.2-1: Request to MSGin5G Server for sending MSGin5G message

| Information element | Status | Description |
| --- | --- | --- |
| Originating UE Service ID/AS Service ID | M | The service identity of the sending MSGin5G Client, Legacy 3GPP UE, Non-3GPP UE or the sending Application Server. |
| Recipient UE Service ID/AS Service ID (see NOTE 1, NOTE 2) | O | The service identity of the receiving MSGin5G Client, Legacy 3GPP UE, Non-3GPP UE or the receiving Application Server. This IE is mandatory for Point-to-Point messaging, AS-to-Point messaging, AOMT messaging and MOAT messaging and is not present in other message scenarios. |
| Group Service ID (see NOTE 1) | O | The service identifier of the target MSGin5G Group. This IE is mandatory for a Group Message and is not present in other message scenarios. |
| Broadcast Area ID (see NOTE 1) | O | The service identifier of the Broadcast Service Area where the message needs to be broadcast. This IE is mandatory in the Broadcast Message and is not present in other message scenarios. |
| Messaging Topic (see NOTE 1) | O | Indicates which Messaging Topic this message is related to. This IE is mandatory for a message distribution based on topic and is not present in other message scenarios. |
| Application ID | O | Identifies the application(s) for which the payload is intended. This list of Application IDs IE is required when the message is sent to one or multiple Application Clients served by same MSGin5G Client. This list of Application IDs IE may be included when the message is sent to an Application Server or to an Application Client. MSGin5G Server is unaware of the content. |
| Message ID | M | Unique identifier of this message. |
| Delivery status required | O | Indicates if delivery acknowledgement from the recipient is requested. |
| Payload | O | Payload of the message. MSGin5G Server/Client is unaware of the content. If the request is sent from MSGin5G Client or Message Gateway to the MSGin5G Server, the maximum size of this IE is a configurable value that shall not exceed 2048 octets. |
| Priority type (see NOTE 3) | O | Application priority level requested for this message. The application priority levels include High, Normal and Low. The default Priority type of an MSGin5G message is Normal. |
| Message is segmented | O | Indicates this message is part of a segmented message. |
| Segmentation set identifier | O | All segmented messages associated within the same set of segmented messages (i.e. associated with the same MSGin5G message) are assigned the same unique identifier. Mandatory IE to be present in every segmented message. |
| Total number of message segments | O | Indicates the total number of segments for the message. The Total Segments needs to be included only in the first segment of the message. |
| Message segment number | O | An incrementing message segment number that indicates segmented message number of each segmented message within a set of segmented messages |
| Last segment flag | O | An indicator of whether this segmented message is the last segment in the set of segmented messages or not. The Last Segment Flag needs to be included only in the last segment of the message. Message segment number of the segment with "Last Segment Flag" set can be considered as total segments. |
| Store and forward flag | O | An indicator of whether store and forward services are requested for this message. If the value indicates that store and forward services are requested by the sender, the store and forward procedure in clause 8.3.6 applies. If this parameter is not presented in this message, there is no specific Store and forward requirement for this message. The MSGin5G Server may apply the local policy on the message delivery, e.g. whether MSGin5G deferred message specified in in clause 8.3.x applies. |
| Store and forward parameters | O | Parameters used by MSGin5G Server for providing store and forward services, as detailed in table 8.3.2-2. This IE shall be included only if the value of the Store and forward flag IE indicates that store and forward services are requested. The MSGin5G store and forward procedure is detailed in clause 8.3.6. |
| NOTE 1: Only one of these IEs shall be included to represent the type of message request. The MSGin5G Client may construct the related IEs based on the information received from Application Client, e.g. adds the MSGin5G service domain. NOTE 2: When the originator is an Application Server, (i.e. Originating AS Service ID is present), this IE shall be a UE Service ID. NOTE 3: The MSGin5G message with high priority should not be aggregated. The other usages of the priority level of the message is implementation specific and is out of scope of this document. |  |  |

Table 8.3.2-2: Store and forward parameters

| Value | Status | Description |
| --- | --- | --- |
| Message expiration time | O | Indicates message expiration time used for providing store and forward services if the destination is not available for communications, The MSGin5GServer attempts delivery at or before the message expiration time, or when the recipient becomes available. |
| Application specific store and forward information | O | Application specific information about store and forward handling, e.g. a delivery time/date. |

3.	The MSGin5G Server verifies that the sender is authorized to send the message and checks the integrity of the message.

If the received MSGin5G message request is for a Group Message, the MSGin5G Server shall generate an individual message to all group members (excluding the message originator) with additional Recipient UE Service ID to each individual message as shown in table 8.3.3-1.

If the received MSGin5G message is a Message Topic message, the MSGin5G Server shall generate an individual message to all subscribers subscribing this message topic (excluding the message originator) with additional Recipient UE Service ID to each individual message as shown in table 8.3.3-1.

4.	The MSGin5G Server may send a Message response to the originating entity if the message is rejected or stored and includes the IEs as listed in table 8.3.2-3 in the response.

Table 8.3.2-3: Message Response

| Information element | Status | Description |
| --- | --- | --- |
| Originating UE Service ID/AS Service ID | M | The identity of the MSGin5G Client, Legacy 3GPP UE, Non-3GPP UE or the identity of the Application Server that initiated the previous Request. |
| Message ID | M | Identifier of the initiating Request. |
| Delivery Status | O | Indicates if delivery is a failure, or if the message is stored for deferred delivery. |
| Failure Cause | O | The reason for failure |