3GPP TSG-SA WG6 Meeting #57	S6-233414

Xiamen, China 9th – 13th October 2023	(revision of S6-233363)

Title:	LS on inputs for mapping GSMA OPG architecture roles with 3GPP defined Edge architectures

Response to:

Release:	Rel-18

Work Item:	EDGEAPP_EXT

Source:	3GPP TSG SA WG6

To:	SA5

Cc:

Contact person:	Sapan Shah (sapan.shah@samsung.com)

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

3GPP SA6 is currently studying to align 3GPP defined EDGEAPP architecture with ETSI MEC and GSMA Operator Platform architecture. Clause 3.2 of Operator Platform Telco Edge Requirements [1] defines roles along with their key functions. As the part of the study, SA6 is working to map GSMA OP architecture defined roles with 3GPP defined EDGEAPP architecture.

3GPP SA6 believes that some of the scenarios as defined for GSMA OP architecture roles may be mapped to 3GPP SA5 defined Edge Computing Management (ECM) in TS 28.538.

Therefore, SA6 would like to request SA5 to provide their inputs in below table if according to SA5 any scenario is mapped to the functionalities of the SA5 defined architecture entity. (See example below)

Below provides introduction to the roles and scenarios:

The three main roles in the GSMA OP architecture and their mapping with EDGEAPP architecture entity is as follows:

1)	Capabilities Exposure Role: It enables an Application Provider to operate their applications. Table 1 shows the scenarios enabled by Capabilities exposure role as indicated in Operator Platform Telco Edge Requirements and their mapping with 3GPP EDGEAPP architecture.

Table 1: Capabilities exposure role

| Interface | Scenario | Description | Inputs from SA5 for Mapping to 3GPP EDGEAPP |
| --- | --- | --- | --- |
| NBI | Edge Cloud Infrastructure Endpoint Exposure | The Application Provider uses an authenticated and authorized endpoint to carry out scenarios involving application instances on edge clouds; | Example: this scenario maps to functionalities of EES (as per clause 6.3.2 and clause 8.5 of TS 23.558) |
|  | Application Onboarding | The Application Provider uses the NBI to provide application images and metadata to the OP Federation Broker/Manager Role |  |
|  | Application Metadata/Manifest Submission | The Application Provider uses the NBI and the metadata model to submit application metadata to the OP and follows defined procedures to extend the metadata model specification |  |
|  | Application CI/CD Management DevOps | The Application Provider integrates the CI/CD framework used to create an application with the OP via NBI APIs (which implies an integration between a CI/CD framework and Application Onboarding and Lifecycle Management) |  |
|  | Application Lifecycle Management | The Application Provider observes and changes the operational state of application instances, including the geographical/network extent of the OP on which application instances may run; |  |
|  | Application Resource Consumption Monitoring | The Application Provider observes resource consumption of application instances, using the resource data model |  |
|  | Edge Cloud Resource Catalogue exposure | The Application Provider inventories edge cloud resources nominally available to application instances |  |
|  | Network Capabilities exposure | The Application Provider inventories network capabilities, like Network Analytics, nominally available to application instances |  |

NOTE: Above SA6 marked example is interpretation of SA6, and if SA5 think differently, please indicate.

2)	Service Resource Manager Role: The Service Resource Manager role in the OP is responsible for managing Cloud and Network resources from the Edge Cloud(s) via the SBI and UNI interfaces. Table 2 shows typical scenarios enabled by the Service Resource Manager role towards the different interfaces as indicated in Operator Platform Telco Edge Requirements and their mapping with 3GPP EDGEAPP architecture.

Table 2: Service Resource Manager Role

| Interface | Scenario | Mapping to 3GPP EDGEAPP |
| --- | --- | --- |
| SBI | Inventory, Allocation and Monitoring of Compute resources from Edge Cloud Infrastructure via the Southbound Interface – Cloud Resources (SBI-CR); |  |
|  | Orchestration of Application instances on the Edge Cloud Infrastructure via the SBI-CR interface |  |
|  | Cloud resource reservation managed by the OP |  |
|  | Configuring UE traffic management policies to accomplish the application's requirements, or the UE's IP address shall be maintained |  |
|  | Exposure of usage and monitoring information to operator's charging engine via the Southbound Interface – Charging functions (SBI-CHF) to enable operators to charge for the OP's services. |  |
| Southbound Interface – Network Resources (SBI-NR) | Fetch Cloudlet locations based on the mobile network data-plane breakout location |  |
|  | Subscribe and receive notifications on UE Mobility events from the network to assist applications |  |
|  | Configure traffic steering in the Mobile Network towards Applications orchestrated in Edge Clouds |  |
|  | Receive statistics/analytics, e.g. to influence Application placement or mobility decisions |  |
|  | Receive information related to the network capabilities, such as QoS, policy, network information, etc |  |
| UNI | Application Instantiation/Termination, e.g. based on triggers from the UNI |  |
|  | Application Endpoint exposure towards User Clients (UC) via the UNI |  |
|  | Application Placement decisions, e.g. based on measurements/triggers from the UNI |  |

3)	Federation Broker and Federation Manager Roles: The Federation Broker and Manager Roles in the OP are responsible for interfacing with other OPs via the East-West Bound Interface. Table 3 shows typical scenarios enabled by the Federation Broker and Federation Manager Roles as indicated in Operator Platform Telco Edge Requirements and their mapping with 3GPP EDGEAPP architecture.

Table 3: Federation Broker and Federation Manager Roles

| Interface | Scenario | Mapping to 3GPP EDGEAPP |
| --- | --- | --- |
| EWBI | Federation Interconnection Management |  |
|  | Edge Cloud Resource Exposure and Monitoring towards partner OPs |  |
|  | Network and Analytics Capabilities Exposure towards partner OPs |  |
|  | Application Images and Application metadata transfer towards partner OPs |  |
|  | Application Instantiation/Termination towards partner OPs |  |
|  | Application Monitoring towards partner OPs |  |
|  | Edge Cloud Resource Catalogue exposure |  |
|  | Service Availability in visited networks |  |

# 2	Actions

To SA5

ACTION: SA6 kindly ask SA5 to provide their inputs to fill above table for the scenarios which are applicable to SA5.

# 3	Dates of next TSG SA WG 6 meetings

SA6#58                 13th November – 17th November 2023 	Chicago, US

SA6#59                 26th February – 1st March 2024 	Athens, Greece