3GPP TSG-SA WG6 Meeting #58	S6-233663

Chicago, USA, 13th - 17th November 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.283 | CR | 0072 | rev |  | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Multi-taker floor control messages |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | FRMCS_Ph5 |  |  |  |  |  | Date: |  |  | 2023-10-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In current 23.283, the multi-talker floor control feature is not supported. Such multi-talker is required for interworking with GSM-R. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the multi-talker floor control messages. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The multi-talker floor control for interworking with GSM-R is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (new) 10.5.2.aa, (new) 10.5.2.bb |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /******************** First Changes ********************/

10.5.2.aa	IWF Multi-talker floor release

Table 10.5.2.aa-1 describes the information elements of floor release for multi-talker control, from the IWF to the MCPTT floor control server and from the floor control server to the IWF, which is used to indicate the media transfer is completed and floor is released. This information flow is sent in unicast from the floor control server.

Table 10.5.2.aa-1: Multi-talker floor release

| MCPTT ID (see NOTE) | M | Identity of participant releasing the floor |
| --- | --- | --- |
| List of functional aliases (see NOTE) | O | Functional alias(es) of participant releasing the floor |
| Source identifier | O | Identifies the communication, e.g. by identifying the media flow within a media multiplex, present only if media multiplexing |
| NOTE: One or more functional aliases may be associated with the MCPTT ID. |  |  |

10.5.2.bb	IWF Multi-talker floor taken

Table 10.5.2.bb-1 describes the information elements of floor taken for multi-talker control, from the IWF to the MCPTT floor control server and from the floor control server to the IWF, which is used to indicate when the floor is simultaneously granted to multiple MCPTT users. The multi-talker floor taken is sent in unicast or broadcast.

Table 10.5.2.bb-1: IWF Multi-talker floor taken

| Information element | Status | Description |
| --- | --- | --- |
| List of MCPTT IDs (see NOTE) | M | Identity (identities) of the granted participant(s) |
| List of functional aliases (see NOTE) | O | Functional alias(es) of the granted participant(s) |
| List of source identifiers (see NOTE) | O | Identifies the communication, e.g. by identifying the media flow within a media multiplex, present only if media multiplexing |
| Acknowledgement required | O | Indicates if acknowledgement from the floor participant is required |
| NOTE: One or more functional aliases and one source identifier may be associated with an MCPTT ID. |  |  |