3GPP TSG-SA WG6 Meeting #58	S6-233671

Chicago, US, 13th – 17th November, 2023

Source:	Ericsson

Title:	Pseudo-CR on Introducing information flow for FL client selection assistance procedure

Spec:	3GPP TR 23.700-82 v0.1.0

Agenda item:	8.3

Document for:	Approval

1. Introduction

This pCR proposes additional information to the existing solution described in clause 8.2.2 TR 23.700-82 v0.1.0 FS_AIMLAPP.

Comment to rapporter:

The reference to table X in Table 8.2.3.1-1 (this pCR) is impemented in pCR S6-233670.

2. Reason for Change

The name of the procedure is changed to introudce the readability and reflect the intention of the procedure defined in clause 8.2.2. The current solution was limited to the procedure description and does not provide information flow. The current procedure was model as Request-Response and this may lead to timeout problems because the AI/ML service producer can take time to provide the selection of FL clients and it may worsen with the large number of FL clients. It is very likely to have large number of FL clients due to the distributed nature of FL. Therefore, it is proposed for asynchronous design and providing response in the notification.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-82 v0.1.0.

* * * First Change * * * *

## 8.2	Solution #2:FL client selection assistance procedure

### 8.2.1	General

The following clauses specify procedures, information flows and APIs for Key issue 3 to provide the assistance to selection of FL clients.

Assumptions:

1.	The proposed solution is based on the AIML Enabler service producer/consumer architecture. The service producer can be a standalone new entity like an AIML Enabler server or collocated with a SEAL entity like an ADAE server or part of the ADAE server. The AIML Enabler service consumer could be VAL servers, VAL UE, or any other entities that consume the AIML Enabler services.

2.	The proposed solution assumes distributed clients in ML. E.g., Federated Learning.

Pre-conditions:

1.	The FL client selection policies are available at the AI/ML service producer.

Editor's Note: This solution may need to be updated for FL case based on KI updates.

### 8.2.2	Procedures

Figure 8.2.2-1: FL client selection assistance procedure

1.	The AIML Enabler service consumer (e.g., VAL server) sends the FL client selection assistance request to the AIML enabler service producer. The request contains VAL service ID and application ID to identify the FL service, FL client selection filtering criteria as per the selection policies  for the selection of FL client(s).

2.	The AI/ML service producer acknowledges the request and provides the FL client selection assistance response message.

3.	The AIML enabler service producer processes the request as per the available selection policies to filter the FL clients and provides the FL client selection assistance notification response containing list of FL client identities satisfying the FL selection policies to the AI/ML service consumer..

Editor's Note 1: The solution is subject to change based on the architecture and is FFS.

* * * Next Change * * * *

### 8.2.3	Information flows

#### 8.2.3.1	FL client selection assistance request

Table 8.2.3.1-1 describes the information flow from the AI/ML service consumer to the AIML service producer for FL client selection assistance request.

Table 8.2.3.1-1 FL client selection assistance request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M | Identity of the Requestor, |
| VAL service ID | M | Identity of the VAL service |
| FL client identities | O | Identifier of FL client, e.g. UE ID, UE address |
| Application Identity | O | Identifier of application |
| FL client selection requirements | M | Indicates the selection filter criterias for FL clients as per the selection policies mentioned in Table X. |

#### 8.2.3.2	FL client selection assistance response

Table 8.2.3.2-1 describes the information flow from the AIML service producer to the AI/ML service consumer to send FL client selection assistance response.

Table 8.2.3.2-1 FL client selection assistance response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates success of failure |

#### 8.2.3.3	FL client selection assistance notification

Table 8.2.3.3-1 describes the information flow from the AIML service producer to the AI/ML service consumer to notify the FL client selection.

Table 8.2.3.3-1 FL client selection assistance response

| Information element | Status | Description |
| --- | --- | --- |
| >Selected FL client identities | M | Identifier of FL client, e.g. UE ID, UE address that satisfies the FL client selection filtering criteria |

* * * End of Changes * * * *