3GPP TSG-SA WG6 Meeting #58	S6-233804

Chicago, USA 13th – 17th November 2023	(revision of S6-233487)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0467 | rev | 1 | Current version: | 19.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of new features for the MC gateway UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BDBOS |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMC |  |  |  |  |  | Date: |  |  | 2023-11-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | SA6 has defined the architecture of an MC gateway UE to host an MC service client on behalf of non-3GPP devices which cannot host an MC service client. But it is still undefined how and where to proceed and hand over media and signalling. Public safety users on non-3GPP direct mode devices (ETSI EN 300-396 series) will be able to use the MC gateway UE to connect to the MC system once the required capabilities are supported by the MC gateway UE as proposed in this CR. For non-3GPP devices which cannot host MC service client no IP connectivity behind an MC gateway UE is necessary. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarification for supporting a functionality to provide MC service to non-3GPP direct mode users on devices which cannot host an MC service client. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Important features for public safety are not defined in the current version of the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.3, 11.4.1, 11.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 11.4.1	General

The MC gateway UE supports IP connectivity behind the MC gateway UE as well as non-IP connectivity. MC clients hosted by non-3GPP devices using non-3GPP connectivity with the MC gateway UE requires the support of an IP network behind the MC gateway UE, where a range of IP addresses is reachable over a single MC gateway UE. That enables the forwarding of signalling information and media plane between non-3GPP devices which host MC clients and the MC server by the MC gateway UE.

* * * Second Change * * * *

### 11.2.1	Functional model

For the application-level signaling necessary for the association between the MC client on the non-3GPP device and the MC gateway UE reference point Gateway-local (GW-local) is used. The MC gateway UE uses the Gateway-Core (GW-Core) reference point towards the MC service server to announce the association between MC gateway client and MC gateway UE server. Once the use of the MC gateway UE has been authenticated and authorized by the corresponding MC service server, the MC service user is authorized MC gateway user.

The MC gateway UE server enables the following functions:

-	Authentication of the MC gateway clients including the MC gateway UE function of the corresponding MC service server

-	The following functions are available when the connection authorization of the MC gateway client is successful:

-	Relay of signalling between MC client and MC service server/CSC server

-	Relay of signaling between the signaling user agent residing on the non-3GPP device and the SIP core

-	Relay of signaling between the HTTP client residing on the non-3GPP device and the HTTP proxy

-	Media plane forwarding between the MC service server and the MC clients

The MC gateway UE function as part of the corresponding MC service server enables the use of an MC gateway UE with a corresponding MC gateway UE function that authorizes the association between the MC gateway client/MC client and the corresponding MC gateway UE to enable forwarding of the media plane to MC clients via the MC gateway UE.

Figure 11.2.1-1: Functional model of MC gateway UE signalling plane - MC client hosted by non-3GPP device

Figure 11.2.1-2: Functional model of MC gateway UE signalling plane - MC client hosted by the MC gateway UE

Figure 11.2.1-2 depicts the case where the Non-3GPP device cannot host MC clients. The MC traffic and signalling is terminated by the respective clients on behalf of the Non-3GPP device in the MC gateway UE and is exchanged with the Non-3GPP Controler. The Non-3GPP Controler authorizes the user at the Non-3GPP device to the MC gateway UE, performs protocol conversion and couples Non-3GPP devices with the corresponding MC service clients.

* * * End of Changes * * * *