3GPP TSG-SA WG6 Meeting #58	S6-233849

Chicago, USA, 13th – 17th November 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.542 | CR | 0047 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PINAPP security clarifications |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | PINAPP |  |  |  |  |  | Date: |  |  | 11/13/2023 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR is in relation to LS Reply S6-233767 received from SA3, providing guidance to SA6 for securing PINAPP reference points procedures. “PIN authorization procedure” is therefore defined in 3GPP TS 23.542. The PIN authorization procedure is mentionned in several existing procedures of 3GPP TS 23.542 clause 8 and the PIN authorization procedure is described as providing security tokens needed in procedures of clause 8. |  |  |  |  |  |  |  |  |
| Summary of change: |  | PIN authorization procedure is added to TS 23.542 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | How the PIN and AS obtain a security token needed to perform procedures defined in 3GPP TS 23.542 clause 8 remain undefined and the procedures cannot be used. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.X (new), 8.X.1 (new), 8.X.2 (new), 8.X.2.1 (new) 8.X.2.2 (new), 8.X.3 (new), 8.X.3.1 (new), 8.X.3.2 (new), 8.X.3.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for the 5G system".

[3]	3GPP TS 22.101: "Service Principles".

[4]	3GPP TS 23.501: "System architecture for the 5G System (5GS)"

[5]	3GPP TS 23.502: "Procedures for the 5G System (5GS)"

[6]	3GPP TS 37.355: "LTE Positioning Protocol (LPP)"

[7]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs; Stage 2".

* * * First Change * * * *

## 8.X	PIN Authorization

### 8.X.1	General

The PIN authorization procedure is used by a PINE or an AS to acquire the securety token needed to perform procedures with other functional entities (e.g., PIN Server, PEMC, PEGC) of the PIN.

A PINE can perform the PIN authorization procedure after discovering the PIN server as described in clause 8.3, and prior to performing procedures that require a security token as defined in clause 8.

An AS can be pre-configured with the PIN server endpoint (e.g., URI, FQDN, IP address), and performs the PIN authorization procedure prior to performing procedures that require a security token as defined in clause 8.

For deployments where the PIN server has CAPIF core function capabilities, and PINE/AS have CAPIF API invoker capabiliites as defined in 3GPP TS 23.222 [7], PINE and AS may obtain security tokens using CAPIF mechanisms.

Editor's note:	Whether further details related to CAPIF mechanisms are needed is FFS.

### 8.X.2	Procedure

#### 8.X.2.1	General

PIN authorization procedure is either performed directly with the PIN server or indirectly via the PEGC for PINE(s) that require support of the PEGC to access the PIN server; the PEGC can deliver the PIN authorization request of the PINE to the PIN server when needed.

#### 8.X.2.2	PIN authorization with PIN server

Figure 8.X.2.2-1 illustrates PIN authorization procedure based on request/response model.

Pre-conditions:

1.	The PINE or AS has been pre-configured or has discovered the address (e.g., IP address, FQDN, URI) of the PIN server;

2-	The PINE or AS has been pre-configured with the PIN identifier;

3-	The PIN server has been configured with the PIN profile;

Figure 8.X.2.2.1-1: PIN authorization with PIN server

1.	The PINE or AS sends a PIN authorization request to the PIN server. The request includes the PIN identifier, and the requestor identifiers, name, description, and address.

2.	Upon receiving the request, the PIN server validates if the requestor is allowed to access the requested PIN using the information provided in the request and the PIN configuration information from the PIN profile.

3.	The PIN server sends a PIN authorization response to the requestor. If the PIN server successfully determines that the requestor is allowed to access the PIN, the PIN server includes a security token for the requestor to use within the PIN. If the PIN server fails to authorize the requestor, the PIN servers indicates failure in the response and include a failure reason.

### 8.X.3	Information flows

#### 8.X.3.1	General

The following information flows are specified for PIN authorization:

-	PIN authorization request and response;

#### 8.X.3.2	PIN authorization request

Table 8.X.3.2-1 describes information elements in the PIN authorization request sent from the PINE or AS to the PIN server.

Table 8.X.3.2-1: PIN authorization request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifiers | M | The identifiers of the requestor (i.e., GPSI or MAC address of the PINE, and PINE identifier or AS identifier). |
| PIN identifier | M | Identifier of the PIN. |
| Name | M | The PINE vendor name or the AS service provider name. |
| Description | M | Description of the PINE or AS. |
| Requestor IP Address | M | The IP address of PINE or AS. |

#### 8.X.3.3	PIN authorization response

Table 8.X.3.3-1 describes information elements in the PIN authorization response sent to the PINE.

Table 8.X.3.3-1: PIN authorization response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (see NOTE) | Indicates that the PIN authorization request was successful. |
| > Security token | M | The security token to be used by the requestor in PIN procedures. |
| Failure response | O (see NOTE) | Indicates that the PIN authorization request failed. |
| > Cause | M | Provides the cause for PIN authorization request failure. |
| NOTE: At least one of the IE shall be present. |  |  |

* * * End of Change(s) * * * *