3GPP TSG-SA WG6 Meeting #58	S6-233861

Chicago, USA, 13th – 17th November 2023	(revision of S6-233681)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0026 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding the consumer for data transmission quality query procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEALDD |  |  |  |  |  | Date: |  |  | 2023-10-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For the agreed CR (S6-233199) in last meeting, based on the input data of transmission quality measurement result from SEALDD server, the ADAES can provide the transmission quality analytics. The data transmission query service exposed by SEALDD server, can be used by ADAES (as a consumer) to obatin the transmission quality measurement result, and further supports the transmission quality analytics. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This paper is proposed to add the consumer for SEALDD data transmission query procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear about how to use SEALDD data transmission query procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.7.2.2, 9.7.3.4, 9.7.3.5, 9.7.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

9.7.2.2	Data transmission quality query

Figure 9.7.2.2-1 illustrate the procedure for SEALDD enabled data transmission quality query. This procedure is used to obtain the historical transmission quality result already measured as described in clause 9.7.2.1.

Pre-conditions:

1.	The SEALDD server performs the data transmission quality measurement procedure, as described in clause 9.7.2.1.

Figure 9.7.2.2-1: SEALDD enabled data transmission quality query procedure

1. The consumers (e.g. VAL server, SEALDD server, NSCE server, ADAE server) can send a SEALDD transmission quality query request to the SEALDD server to obtain the transmission quality measurement result. The request includes the identifiers of the application traffic (e.g. VAL service ID, VAL server ID), VAL UE ID or VAL UE group ID.

2. The SEALDD server responds with the transmission quality measurement result (e.g. packet delay, bitrate, packet loss rate).

* * * Next Change * * * *

9.7.3.4	SEALDD enabled data transmission quality query request

Table 9.7.3.4-1 describes the information flow from the other consumers (e.g. SEALDD server, NSCE server, ADAE server) to the SEALDD server for querying the data transmission quality measurement result.

Table 9.7.3.4-1: SEALDD transmission quality query request

| Information element | Status | Description |
| --- | --- | --- |
| Application traffic identifiers | M | Identify of the application traffic (e.g. VAL server ID, VAL service ID) |
| VAL UE ID(s) | O | Identifier of VAL UE(s) need to be queried, e.g. single VAL UE, multiple VAL UEs, or VAL UE group |

9.7.3.5	SEALDD enabled data transmission quality query response

Table 9.7.3.5-1 describes the information flow from the SEALDD server to the other consumers (e.g. SEALDD server, NSCE server, ADAE server) for returning the data transmission quality reports.

Table 9.7.3.5-1: SEALDD transmission quality query response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Success or failure. |
| Transmission quality measurement reports list | M | The generated transmission quality results in SEALDD server, as specified in Table 9.7.3.3-2. |

* * * Next Change * * * *

9.7.4	APIs

9.7.4.1	General

Table 9.7.4.1-1 illustrates the APIs exposed by SEALDD server for data transmission quality measurement.

Table 9.7.4.1-1: List of SEALDD server APIs for transmission quality measurement

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Sdd_TransmissionQualityMeasurement_subscription | Request | Request/Response | VAL server |
| Sdd_TransmissionQualityMeasurement_notification | Notify | Subscribe/notify | VAL server |
| Sdd_TransmissionQualityQuery | Request | Request/Response | Other consumers (e.g. SEALDD server, NSCE server, ADAE server) |