3GPP TSG-SA WG6 Meeting #60	S6-241143

Changsha, China 9th – 13th April 2024	(revision of S6-241xxx)

Source:	Huawei, HiSilicon

Title:	pCR on Solution of third-party Call Service

Spec:	3GPP TR 23.700-92 v0.4.0

Agenda item:	8.2

Document for:	Approval

Contact:	wanghan28@huawei.com

1. Introduction

This pCR proposes solution of third Party Call Service which solves the Key Issue#1.

2. Reason for Change

Contribution with new solution.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-92 v0.4.0.

* * * First Change * * * *

## 8.X	Solution #X: third-party Call Service

### 8.X.1	Description

This solution resolves Key Issue #1 about third-party Call service via eMMtel enabler Server.

This solution uses the architecture of eMMTel Enabler as specified clause 8.1as the basis and has no requirement to enhance the architecture.

IMS capability exposure is being discussed in SA2, IMS Core network exposures create/update/release an IMS session with data channel capability via N33/DC4/MDC2/MDC3. In this procedure, the eMMTel Enabler Server provides third-party Call service from service granularity which use SA2’s atomic interface to provide high-level call service APIs to hide the complexity of underlying the IMS core capabilities, the eMMTel Enabler Server handles all the necessary API requests/responses related to the session creation on behalf of the application without further interaction with the application until the IMS session with Data Channel media is established successfully or failed. the eMMtel Enabler Server acts as a middle layer which can provide some other value-added services (e.g. provides the virtual number service) between third-party DC AS and IMS Core network as specified in 3GPP TR 23.700-77 [x] clause 6.6.

### 8.X.2	Procedures

#### 8.X.2.1	Service Enabler Description

Figure 8.X.2.1-1: Reference Architecture for the Nemes Services

The eMMTel Enabler Server provides the following services to the application:

- Control operations of third-party Call with data channel media, such as initiating, modifying, terminating.

- Subscribe and unsubscribe to third-party Call events of the specific subscriber, and

- May include some other value-added services, e.g. virtual number.

8.X.2.2	Establishing a third-party Call with data channel media

Figure 8.X.2.2-1 below illustrates how to establish a third-party call which support data channel media between two users. the eMMtel Enabler Server acts as a middle layer between the third-party call application and IMS Core network, according to the session creation API invoked by the eMMTel Enabler Server, the IMS network initiates an IMS session with data channel media.

Figure 8.x.2.2-1: Establishing a Call with data channel media

1.	The third-party call application invokes the session creation API of the eMMTel Enabler Server to create a Call with data channel media. The request includes information elements as specified in Table 8.x.2.2-1.

Table 8.x.2.2-1: Information elements in call request

| Information element | Status | Description |
| --- | --- | --- |
| Originating ID | M | The identifier of the caller. |
| Terminating ID | M | The identifier of the callee. |
| Media information | M | The audio, video, DC media related information of the call originator |
| Application Profile requested | O | The DC application profile expected to be used in this call |

2.	The eMMTel Enabler Server verifies that the sender is authorized to establish a call with data channel media and may execute other value-add service.

3.	The eMMTel Enabler Server initiate a call with Data Channel to the UE A and UE B via IMS Core.

4.	The eMMTel Enabler Server sends a corresponding call session establishment response, e.g the call is established successfully or not, to the sender.

Table 8.x.2.1-2: Information elements in call response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of the session. |
| Reason | O | Indicates the reason for failure. |

5.	UE A and UE B begin to talk with each other with interaction over data channel.

8.X.2.3	Terminating a third-party Call with data channel media

Figure 8.x.2.3-1 below illustrate how to terminate a third-party Call with data channel media between two users. The terminating procedure in Figure 8.x.2.3-1 is triggered by the third-party call application invoking a release session API provided by the eMMTel Enabler Server.

Figure 8.x.2.3-1: Terminating a Call with data channel media

1.	The 3rd Party call application sends a release request to the eMMTel Enabler Server.

2.	The eMMTel Enabler Server verifies that the sender is authorized to release a call.

3.	The eMMTel Enabler Server release the call to the UE A and UE B via IMS Core.

3.	The eMMTel Enabler Server return a release response to the third-party call application.

8.X.2.4	Modifying a third-party Call to add data channel media

Figure 8.x.2.4-1 below illustrates how to modify an established Call to add data channel media between two users. It’s assumed that a Call with audio and video media has established between two users.

Figure 8.x.2.4-1: Modifying a Call to add data channel media

1.	The 3rd Party call application invokes the session modification API of the eMMTel Enabler Server to add data channel media into the specific session.

2.	The eMMTel Enabler Server verifies that the sender is authorized to modify a call with data channel media.

3.	The eMMTel Enabler Server modify the call with Data Channel to the UE A and UE B via IMS Core.

4.	The eMMTel Enabler Server sends a corresponding call session modification response to the sender.

5.	UE A and UE B begin to talk with each other with interaction over data channel.

### 8.X.3	Impacts on Existing Nodes and Functionality

In this solution, the procedures use the basic capability exposured by IMS core which is under studied by SA2.

* * * End of Changes * * * *