3GPP TSG-SA WG6 Meeting #60	S6-241163

Changsha, Hunan, China 15th – 19th April 2024	(revision of S6-24xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.281 | CR | 0218 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MC gateway authentication and authorization (MCVideo) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCGWUE |  |  |  |  |  | Date: |  |  | 2024-04-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | SA3 has discussed the connection authorization procedures specified in clause 11.5.1.2.3 and clause 11.5.1.3.3 of TS 23.280 on and provided the following feedback (see LS reply in S6-240551). This CR reflects the decssions made in SA3. The connection authorization procedure is now only used between the MC gateway client and the MC gateway UE for controlling the selection of an MC gateway UE by the MC gateway client and for allowing the MC gateway UE and to reject an MC gateway client, if needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The MC server is no longer involved in the connection authorization and disconnection mechanisms, only the MC gateway client and the MC gateway UE are involved. The MC gateway UE checks whether the provided MC GW service ID allows to have access. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear requirements/guidance for stage 3 specifications and misalignment with TS 33.180. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex A.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * Start of changes * * * *

# A.5	MCVideo service configuration data

The general aspects of MC service configuration are specified in 3GPP TS 23.280 [6]. The MCVideo service configuration data is stored in the MCVideo server.

Tables A.5-1 and A.5-2 describe the configuration data required to support the use of on-network MCVideo service. Tables A.5-1 and A.5-3 describe the configuration data required to support the use of off-network MCVideo service. Data in table A.5-1 and table A.5-3 can be configured offline using the CSC-11 reference point.

Table A.5-1: MCVideo service configuration data (on and off network)

| Reference | Parameter description | MCVideo UE | MCVideo Server | Configuration management server |
| --- | --- | --- | --- | --- |
| [R-5.2.2-001] of 3GPP TS 22.280 [2] | Levels of group hierarchy for group-broadcast groups (Bc1) | Y | Y | Y |
| [R-5.2.3-001] of 3GPP TS 22.280 [2] | Levels of user hierarchy for user-broadcast groups (Bc2) | Y | Y | Y |
| [R-5.7-002] of 3GPP TS 22.280 [2] | Minimum length (Nc3) of an alphanumeric identifier (i.e. alias) assigned by an MCVideo administrator. | Y | N | Y |

Table A.5-2: MCVideo service configuration data (on network)

| Reference | Parameter description | MCVideo UE | MCVideo Server | Configuration management server |
| --- | --- | --- | --- | --- |
| [R-5.12-001] of 3GPP TS 22.280 [2] | Protect confidentiality of signalling (see NOTE 1) | Y | Y | Y |
| [R-5.12-001] of 3GPP TS 22.280 [2] | Protect integrity of signalling (see NOTE 1) | Y | Y | Y |
| [R-5.12-001] of 3GPP TS 22.280 [2] | Use signalling protection between MCVideo servers (see NOTE 1) | N | Y | Y |
| [R-5.12-001] of 3GPP TS 22.280 [2] | Use transmission control protection between MCVideo servers (see NOTE 1) | N | Y | Y |
| [R-5.10-001a] of 3GPP TS 22.280 [2] | Maximum number of successful simultaneous service authorizations of clients from a user | N | Y | Y |
|  | List of functional alias identities |  |  |  |
| [R-5.9a-005] of 3GPP TS 22.280 [2] | > Functional alias | N | Y | Y |
| [R-5.9a-005] of 3GPP TS 22.280 [2] | >> Limit number of simultaneous activations | N | Y | Y |
| [R-5.9a-005] of 3GPP TS 22.280 [2] | >> This functional alias can be taken over | N | Y | Y |
|  | >> List of users who can activate this functional alias |  |  |  |
| [R-5.9a-005] of 3GPP TS 22.280 [2] | >>> MCVideo ID | N | Y | Y |
| [R-5.9a-016] of 3GPP TS 22.280 [2] | >> Communication priority (see NOTE 2) | N | Y | Y |
| Subclause 5.15 of 3GPP TS 22.280 [2] | List of permitted GW MC service ID(s) |  |  |  |
|  | > GW MC service ID | Y | N | Y |
|  | Ad hoc group call configurations |  |  |  |
| [R-6.15.5.3-005] of 3GPP TS 22.280 [17] | > Support of ad hoc group call (enabled/disabled) (see NOTE 3) | Y | Y | Y |
| [R-6.15.5.3-002] of 3GPP TS 22.280 [17] | > Maximum number of participants allowed to participate in an ad hoc group call | Y | Y | Y |
| [R-6.15.5.3-004] of 3GPP TS 22.280 [17] | > Hang timer for ad hoc group call | N | Y | Y |
|  | > Maximum duration for ad hoc group call | Y | Y | Y |
|  | > List of preferred video codecs for ad hoc group call | Y | Y | Y |
| NOTE 1: Security mechanisms are specified in 3GPP TS 33.180 [14]. NOTE 2: The usage of this parameter by the MCVideo server is up to implementation. NOTE 3: If the support for ad hoc group call is disabled by the MC system then all other configurations related to ad hoc group call are not applicable. |  |  |  |  |

Table A.5-3: MCVideo service configuration data (off network)

| Reference | Parameter description | MCVideo UE | MCVideo Server | Configuration management server |
| --- | --- | --- | --- | --- |
| [R-7.6-001], [R-7.6-002], [R-7.6-003] of 3GPP TS 22.280 [2] | Default ProSe Per-Packet priority (as specified in 3GPP TS 23.303 [7]) values |  |  |  |
|  | > MCVideo private call signalling | Y | N | Y |
|  | > MCVideo private call media | Y | N | Y |
|  | > MCVideo Emergency private call signalling | Y | N | Y |
|  | > MCVideo Emergency private call media | Y | N | Y |
| Subclause 7.5.3.3 | Periodicity interval (in seconds) of Capability announcement (see NOTE) | Y | N | Y |
| NOTE: If the value of the periodicity interval is set to zero, the capability announcement is not sent. |  |  |  |  |

# * * * * End of changes * * * *