3GPP TSG-SA WG6 Meeting #60	S6-241179

Changsha, China 15th – 19th April 2024	(revision of S6-241xxx)

Source:	Lenovo

Title:	New solution on SL positioning / ranging enablement

Spec:	3GPP TR 23.700-72 v0.4.0

Agenda item:	8.1

Document for:	Approval

Contact:	Emmanouil Pateromichelakis <epateromiche@lenovo.com>

1. Introduction

This paper proposes a new solution on ranging / SL positioning enablement.

2. Reason for Change

This paper proposes a new solution on ranging / SL positioning enablement.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-72 v0.4.0.

* * * First Change * * * *

## 8.x	Solution #x: Support for SL positioning/ranging enablement

### 8.x.1	Solution description

This solution provides a mechanism for configuring and provisioning via application layer information on the entities to be used and their responsibilities for a ranging / SL positioning service. This solution assumes that the VAL UEs of different capabilities and potential roles (reference, target, client) have instantiated a location enablement client (LMC). In this solution, an SL positioning enabler at LMS is authorized / delegated by SL positioning server to support the provisioning/delivery of ranging services in an area.

Figure 8.x.1-1 shows a high-level prcoedure with the details.

Pre-conditions:

1.	One or more client UEs send a “discovery” request to the VAL server acting as SL positioning server in case the client UEs location needs to be discovered.

- VAL server (SL Positioning / Ranging Server) sends a subscription request to LMS to provision and manage ranging operation and configurations. This includes monitoring, discovering of UEs assisting the operation, and delivering the configuration for a target service area, or for a target set of UEs to server as reference and target. This request can also include a list of client UEs that their location needs to be discovered (based on the client UE request).

- LMS sends a subscription response to requestor VAL Server.

- LMS finds all the LMCs in the target service area, and categorizes them by their capabilities as target UE, Reference UEs, or even Located UEs (if Reference UE location is known or for out of coverage scenarios). The identification of UEs in a target area can happen via existing SEAL LMS procedures.

- LMS after getting all UE information at the target area, calculates the relative distances among them, and also may request/receive HDMaps (e.g., from other VAL servers) for the target area. This will help identifying LoS or NLoS situations.

- LMS evaluates the need of reference UE, and if reference UE is needed, LMS selects the appropriate UE to server as reference (also considering the capabilities) and configures reference UE with information, thresholds etc.., one time or period ranging, ranging for distance or direction measurement or both.

- LMS configures LMCs with all the configuration information from step 5 and in particular the expected responsibility in relation ranging/SL positioning application service. In this step, Client UE gets provisioned also with ranging UEs, so as to be aware of the entity when he needs to request from LMS or from reference UE to get ranging parameter.

- LMC interact with the VAL clients and other LMCs (not reachable by LMS) to convey the configuration.

Ranging Service gets initiated from the App of the client UE:

- Client UE via LMC requests from LMS to get ranging information between a reference and a target UE.

- LMS interacts with reference UE to derive ranging parameters at LMC or SL Positioning Server UE app, after receiving location reports from the involved entities via LM-C. LMCs report the ranging information to LMS.

- LMS calculates the ranging parameters based on the received reports.

- LMS informs the Client UE (SL positioning app).

### 8.x.2	Solution evaluation

Editor's note:	This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

* * * Next Change * * * *

## 6.0	Mapping of solutions to key issues

Table 6.0-1 Mapping of solutions to key issues

|  | KI # 1 | KI # 2 | KI # 3 | KI # 4 | KI # 5 | KI # x |
| --- | --- | --- | --- | --- | --- | --- |
| Sol #1 |  |  | X |  |  |  |
| Sol #2 | X |  |  |  |  |  |
| Sol #3 |  | X |  |  |  |  |
| Sol #4 | X |  |  |  |  |  |
| Sol #5 |  |  |  | X |  |  |
| Sol #6 |  |  | X |  |  |  |
| Sol #X |  |  |  |  | X |  |