3GPP TSG-SA WG6 Meeting #60	S6-241206

Changsha, China 15th – 19th April 2024	(revision of S6-241xxx)

Source:	Lenovo

Title:	Solution on FL member group management

Spec:	3GPP TR 23.700-82 v0.3.0

Agenda item:	8.3

Document for:	Approval

Contact:	Emmanouil Pateromichelakis <epateromiche@lenovo.com>

1. Introduction

This paper proposes a new solution for KI #3 on FL member management.

2. Reason for Change

This paper proposes a new solution for KI #3 on FL member management.

KI #3 on FL discusses the issue on:

4. 	Whether and how to support the management of FL groups (e.g., how to create and manage a group of FL members) during FL operations (e.g., training)?

This solution addresses KI #3 to support the management of the FL groups.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-82 v0.3.0.

* * * First Change * * * *

## Mapping of solutions to key issues

Table 8.1-1: Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI #5 | KI #6 | KI #7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sol #1 |  |  |  |  |  |  |  |
| Sol #2 |  |  |  |  |  |  |  |
| Sol #3 |  | X |  |  |  |  |  |
| Sol #4 |  | X |  |  |  |  |  |
| Sol #5 |  | X |  |  |  |  |  |
| Sol #6 |  |  | X |  |  |  |  |
| Sol #7 |  |  | X |  |  |  |  |
| Sol #8 |  |  | X |  |  |  |  |
| Sol #9 |  |  | X |  |  |  |  |
| Sol #10 |  |  |  |  |  |  | X |
| Sol #11 | X |  |  |  |  |  |  |
| Sol #12 | X |  |  |  |  | X |  |
| Sol #13 | X |  | X |  |  |  |  |
| Sol #14 |  |  |  |  |  |  | X |
| Sol #15 |  | X |  |  |  |  |  |
| Sol #16 |  |  | X |  |  |  |  |
| Sol #17 |  |  | X |  |  |  |  |
| Sol #18 |  |  |  | X |  |  |  |
| Sol #19 |  |  |  |  | X |  |  |
| Sol #X |  |  | X |  |  |  |  |

* * * Second Change * * * *

## 8.x	Solution #x: Support for FL member grouping

This solution addresses Key Issue #3 on FL member grouping.

The mechanism proposed in this solution is to enable the group management of the entities serving as FL clients at the application enablement layer. Such group management is about the creation, monitoring and update of the groups based on the AI/ML operations, which are based on 1) the analytics event/service by ADAES or 2) the VAL requirement for FL support services.

### 8.x.1	Solution description

This solution covers the scenario that the group functionality is at GMS, and this functionality is utilized by the AIML enablement server who can be for example the FL server/aggregator.

The requirement for grouping the FL members by the VAL server (indirectly) or by the AIML enablement server (acting as consumer) is applicable to a specific VAL request or ML model ID or analytics event/ID. The creation of the group and the generation of group identities is performed by SEAL GMS for the FL members.

Figure 8.x.1-1 illustrates the procedure for supporting the FL member grouping.

Pre-conditions:

1.	VAL server is connected to AIML Enablement Server

2.	The candidate/selected FL member has registered to the FL member registry based on Solution #9.

Figure 8.x.1-1 FL member grouping procedure

1. The VAL server sends a request to the AIML Enablement server for supporting an FL process. Such request may include the ML model ID, the list of target UEs, the data requirements, the ML training or inference job description, the list of candidate FL participants (if known) or the list of selected FL participants (if the VAL server is the FL aggregator) etc.

2. The AIML Enablement server, based on the request, determines the need for creating a group consisting of the needed FL members. The FL aggregator can be either the VAL server or the AIML Enablement server himself, based on the request.

3. The AIML Enablement server sends a request to an FL group functionality at the Group Management Server (GMS) to undertake the group management for the FL process based on the request of step 1. Such request includes the ML model ID, the service ID or service area or service profile, as well as the KPIs for the FL process, the data requirement, the types of ML/FL entities required, whether model training and/or inference is needed, the model vendor information, the training job ID (if known) etc.

4. The GMS authorizes the request and fetches the FL candidate or available FL members for the given model ID/profile or for the given service or service area or for the given training/inference job ID. This can be fetched by a FL member registry/repository e.g., such as A-ADRF.

5. The GMS determines the group by configuring the group based on the selected FL members by the FL server/aggregator. GMS generates a group identity and creates/stores a mapping of the generated group ID to a set of FL member IDs within the group.

6. GMS notifies the group ID and the group member identities for the ML model ID / analytics ID (based on the request in step 1. Such notification may include a training job ID or a correlation ID or an FL process ID.

7. For VAL client UEs selected as FL members, the AIML Enablement client passes the group information to the VAL client and may also request its consent/confirmation.

8.  The AIML Enablement server sends a response to the VAL request indicating the group creation and the group information.

FL process is confirmed and starts.

9. The FL clients send the locally trained ML model outputs to the FL server using the group ID.

10. The SEAL GMS (FL group functionality) continuously checks/monitors the group status and the members’ availability / conditions.

11. The SEAL GMS (FL group functionality) detects a trigger for changing the group (new member joins or existing member leaves or existing member problematic/unavailable). Such trigger may come either from the VAL server or may be detected at the AIML Enablement Server. GMS then updates the group by changing the mapping to the member list (the group ID stays the same, but the member list changes).

12. The SEAL GMS sends notifications for the updated groups to the involved entities.

### 8.x.2	Architecture Impacts

The application enabler layer architecture impacts are the following:

-	AIML enablement server is introduced to provide support for triggering the grouping FL members.

-	SEAL GMS needs to be enhanced with FL group functionality (creating a group ID and monitoring the group for changes).

-	An FL member registry/repository needs to be enhanced to maintain and update group identity for an FL member group.

### 8.x.3	Corresponding APIs

This subclause provides a summary on the corresponding API for solution #x.

-	FL Group Management API (request / response model; API provider: AIML enablement server; known consumer: VAL server; corresponding to step 3 of 8.x.1.1).

-	FL group create API (request / response model; API provider: SEAL GMS; known consumer: AIML enablement server; corresponding to step 5 of 8.x.1.1).

-	FL group update API (request / response model; API provider: SEAL GMS; known consumer: AIML enablement server; corresponding to step 10 of 8.x.1.1).

### 8.x.4	Solution evaluation

This solution addresses Key Issue #3 and introduces the capability to support FL member grouping. This solution requires the enhancement of SEAL GMS for grouping the FL members. This solution also requires Sol#9 as pre-condition since the candidate FL members needs to be registered at the FL member repository.

This solution is feasible and doesn't introduce any dependency to 3GPP network systems.

* * * Next Change * * * *