3GPP TSG-SA WG6 Meeting #60	S6-231245

Changsha, China 15th – 19th April 2024	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0559 | rev |  | Current version: | 18.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Configuration for MC client on non-3GPP device |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCGWUE |  |  |  |  |  | Date: |  |  | 2024-04-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The MC client on the non-3GPP device should obtain the configurations from the MC server. However, in Annex A, those configuration tables are described to be configured to the MC service UE rather than the corresponding MC client in the non-3GPP device where the reference point is terminated. For the MC client is hosted in the non-3GPP device, such configurations are required to be available for the MC client on the N3GPP device. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce a NOTE to clarify the configuration should be delivered to the non--3GPP device host the MC client. Modify the descriptions in Annex A to capture the configurations are configured to the ‘client’ either on MC service UE or non-3GPP device. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The MC client host by the N3GPP device cannot obtain the configurations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.1, A.2, A.3, A.4, A.5, A.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | . |  |  |  |  |  |  |  |  |

# /************************ First change ******************************/

Annex A (normative):
Configuration data for MC services

A.1	General

This Annex provides information about the static data needed for configuration for MC services, which belong to one of the following categories:

-	MC service UE configuration data (see subclause A.2);

-	MC service user profile data (see subclause A.3);

-	MC service group configuration data (see subclause A.4);

-	MC service configuration data (see subclause A.5); and

-	Initial MC service UE configuration data (see subclause A.6).

NOTE:	For the MC client connects the MC server via the MC gateway UE, the above configuration data is delivered to the non-3GPP device host the MC client.

For each configuration category, data is split between configuration data that is applicable to both on-network and off-network, configuration data that is applicable to on-network only, and configuration data that is applicable to off-network only. The configuration data in each configuration category corresponds to a single instance of the category type i.e. the MC service UE, MC service group, MC service user and MC service configuration data refers to the information that will be stored against each MC service UE, MC service group, MC service user and MC service. This means that the three separate tables (on-network and off-network, on-network only, off-network only) for each configuration category represent the complete set of data for each configuration data category element.

The columns in the tables have the following meanings:

-	Reference: the reference of the corresponding requirement in 3GPP TS 22.280 [3] or 3GPP TS 22.281 [4] or 3GPP TS 22.282 [5] or 3GPP TS 22.179 [2] or the corresponding subclause from the present document.

-	Parameter description: A short definition of the semantics of the corresponding item of data, including denotation of the level of the parameter in the configuration hierarchy.

-	When it is not clear to which functional entities the parameter is configured, then one or more columns indicating this are provided where the following nomenclature is used:

-	"Y" to denote "Yes" i.e. the parameter denoted for the row needs to be configured to the functional entity denoted for the column.

-	"N" to denote "No" i.e. the parameter denoted for the row does not need to be configured to the functional entity denoted for the column.

Parameters within a set of configuration data have a level within a hierarchy that pertains only to that configuration data. The hierarchy of the configuration data is common across all the three tables of on-network and off-network, on-network only and off-network only. The level of a parameter within the hierarchy of the configuration data is denoted by use of the character ">" in the parameter description field within each table, one per level. Parameters that are at the top-most level within the hierarchy have no ">" character. Parameters that have one or more ">" characters are child parameters of the first parameter above them that has one less ">" character. Parent parameters are parameters that have one or more child parameters. Parent parameters act solely as a "grouping" of their child parameters and therefore do not contain an actual value themselves i.e. they are just containers for their child parameters.

Each parameter that can be configured online shall only be configured through one online reference point. Each parameter that can be configured offline shall only be configured through one offline reference point. The most recent configuration data made available to the MC service UE shall always overwrite previous configuration data, irrespective of whether the configuration data was provided via the online or offline mechanism.

A.2	MC service UE configuration data

MC service UE configuration data has to be known by an MC service UE or non-3GPP device host the MC client after MC service authorization. The CSC-4 reference point is used for configuration between the configuration management server and the configuration management client on the MC service UE when the MC service UE is on-network or non-3GPP device host the MC client.

MC service UE configuration data can be configured offline using the CSC-11 reference point.

MC service UE configuration data is specific to each MC service and is specified as follows:

-	MCPTT UE configuration data is specified in 3GPP TS 23.379 [16];

-	MCVideo UE configuration data is specified in 3GPP TS 23.281 [12]; and

-	MCData UE configuration data is specified in 3GPP TS 23.282 [13].

Within each MC service, the MC service UE configuration data can be the same or different across MC service UEs or across N3GPP devices host the MC client.

A.3	MC service user profile configuration data

The MC service user profile configuration data is stored in the MC service user database. The configuration management server is used to configure the MC service user profile configuration data to the MC service user database (CSC-13) and configuration management client (CSC-4) on the MC service UE or non-3GPP device host the MC client.

MC service user profile configuration data can be configured offline using the CSC-11 reference point.

MC service user profile configuration data is specific to each MC service and is specified as follows:

-	MCPTT user profile configuration data is specified in 3GPP TS 23.379 [16];

-	MCVideo user profile configuration data is specified in 3GPP TS 23.281 [12];

-	MCData user profile configuration data is specified in 3GPP TS 23.282 [13]; and

-	Location user profile configuration data is specified in clause A.8.

A.4	Group configuration data

The group configuration data is stored in the group management server. The group management server is used to configure the group configuration data to the group management client (CSC-2) on the MC service UE or the non-3GPP device  and the MC service server (CSC-3).

A single group can support one or multiple MC service configurations. Hence, the MC service group configuration data is divided into common group configuration data and MC service specific group configuration data.

The group configuration data can be configured offline using the CSC-12 reference point.

The common group configuration data are specified in table A.4-1. Tables A.4-1 and A.4-2 contain the group configuration required to support the use of on-network MC service. Tables A.4-1 and A.4-3 contain the group configuration required to support the use of off-network MC service.

The MC service related group configuration data specific to each MC service is specified as follows:

-	MCPTT related group configuration data is specified in 3GPP TS 23.379 [16];

-	MCVideo related group configuration data is specified in 3GPP TS 23.281 [12]; and

-	MCData related group configuration data is specified in 3GPP TS 23.282 [13];

Table A.4-1: Common group configuration data (on and off network)

| Reference | Parameter description | MC service UE | MC service Server | Group management server |
| --- | --- | --- | --- | --- |
| [R-5.17-004], [R-6.4.3-001], [R-6.4.3-002], [R-6.9-004] and [R-5.1.3-002] of 3GPP TS 22.280 [3] | MC service group ID | Y | Y | Y |
| [R-5.17-004], [R-6.4.3-001], [R-6.4.3-002] and [R-6.9-004] of 3GPP TS 22.280 [3] | Group Alias (Contact URIs) | Y | Y | Y |
|  | List of group members (see NOTE 1) (see NOTE 8) |  |  |  |
| [R-5.1.3-001], [R-5.1.5-001], [R-6.4.5-005] and [R-6.4.5-006] of 3GPP TS 22.280 [3] | > MCPTT |  |  |  |
|  | >> MCPTT ID (see NOTE 2) | N | Y | Y |
| 3GPP TS 33.180 [25] | >> KMSUri for security domain of the MCPTT ID (see NOTE 3) | N | Y | Y |
| [R-6.2.2-001] and [R-7.6-007] of 3GPP TS 22.280 [3] | >> User priority for the group | N | Y | Y |
| [R-5.1.3-001] of 3GPP TS 22.280 [3] | >> Participant type for the group (group membership information). The particpant type values are defined and configured by the Mission Critical Organisation (e.g.first responder, second responder, dispatcher, dispatch supervisor, MC service administrator). | N | Y | Y |
| [R-5.1.3-001], [R-5.1.5-001], [R-6.4.5-005] and [R-6.4.5-006] of 3GPP TS 22.280 [3] | > MCVideo |  |  |  |
|  | >> MCVideo ID (see NOTE 2) | N | Y | Y |
| 3GPP TS 33.180 [25] | >> KMSUri for security domain of the MCVideo ID (see NOTE 3) | N | Y | Y |
| [R-6.2.2-001] and [R-7.6-007] of 3GPP TS 22.280 [3] | >> User priority for the group | N | Y | Y |
| [R-5.1.3-001] of 3GPP TS 22.280 [3] | >> Participant type for the group (group membership information). The particpant type values are defined and configured by the Mission Critical Organisation (e.g.first responder, second responder, dispatcher, dispatch supervisor, MC service administrator). | N | Y | Y |
| [R-5.1.3-001], [R-5.1.5-001], [R-6.4.5-005] and [R-6.4.5-006] of 3GPP TS 22.280 [3] | > MCData |  |  |  |
|  | >> MCData ID (see NOTE 2) | N | Y | Y |
| 3GPP TS 33.180 [25] | >> KMSUri for security domain of the MCData ID (see NOTE 3) | N | Y | Y |
| [R-6.2.2-001] and [R-7.6-007] of 3GPP TS 22.280 [3] | >> User priority for the group | N | Y | Y |
| [R-5.1.3-001] of 3GPP TS 22.280 [3] | >> Participant type for the group (group membership information). The particpant type values are defined and configured by the Mission Critical Organisation (e.g.first responder, second responder, dispatcher, dispatch supervisor, MC service administrator). | N | Y | Y |
| [R-5.16.2-001] and [R-5.16.2-002] of 3GPP TS 22.280 [3] | Group’s owner (Mission Critical Organisation) | Y | Y | Y |
| Subclause 8.3.2, 10.2.3 | MC service specific configuration (see NOTE 4) |  |  |  |
|  | > MCPTT configuration (see NOTE 5) |  |  |  |
|  | > MCVideo configuration (see NOTE 6) |  |  |  |
|  | > MCData configuration (see NOTE 7) |  |  |  |
|  | List of subordinate groups |  |  |  |
| [R-5.2.2-002] of 3GPP TS 22.280 [3] | > MC service group ID | N | Y | Y |
|  | Group broadcast information |  |  |  |
| [R-5.2.2-001] of 3GPP TS 22.280 [3] | Level within group hierarchy | N | Y | Y |
| [R-5.2.3-001] of 3GPP TS 22.280 [3] | Level within user hierarchy | N | Y | Y |
| Subclause 10.6.2.9 of 3GPP TS 23.379 [16] | Group is a preconfigured regroup group | Y | Y | Y |
| [R-5.2.1-002] of 3GPP TS 22.280 [3] | Group is a broadcast group | Y | Y | Y |
| NOTE 1: The list of group members is provided to an authorized user only upon request and not by default. NOTE 2: MC service Ids for all configured MC services shall be provided for the configured group member. NOTE 3: If this parameter is absent, the KMSUri shall be that identified in the initial MC service UE configuration data (on-network) configured in table A.6-1. NOTE 4: An MC service specific configuration shall be provided for all of the MC services for which the group is enabled. At least one MC service shall be enabled for a group. NOTE 5: This parameter is a parent parameter whose child parameters are defined in 3GPP TS 23.379 [16]. NOTE 6: This parameter is a parent parameter whose child parameters are defined in 3GPP TS 23.281 [12]. NOTE 7: This parameter is a parent parameter whose child parameters are defined in 3GPP TS 23.282 [13]. NOTE 8: The list of group members comprises all group members in primary and in partner MC systems of the MC service group. |  |  |  |  |

Table A.4-2: Common group configuration data (on-network)

| Reference | Parameter description | MC service UE | MC service Server | Group management server |
| --- | --- | --- | --- | --- |
| [R-6.1-001] of 3GPP TS 22.280 [3] | Maximum number of group members (Nc6) | N | Y | Y |
| [R-6.1-003] of 3GPP TS 22.280 [3] | Enabled/disabled group (basic status) | Y | Y | Y |
| [R-6.6.1-006] of 3GPP TS 22.280 [3] | Temporary group indication | Y | Y | Y |
| [R-6.4.5-005] of 3GPP TS 22.280 [3] | List of authorised users who can request the list of members of an MC service group | Y | Y | Y |
|  | > MCPTT IDs |  |  | Y |
|  | > MCVideo IDs |  |  | Y |
|  | > MCData IDs |  |  | Y |
|  | MC service specific configuration |  |  |  |
|  | > MCPTT configuration (see NOTE 1) |  |  |  |
|  | > MCVideo configuration (see NOTE 2) |  |  |  |
|  | > MCData configuration (see NOTE 3) |  |  |  |
| [R-6.4.9-006] of 3GPP TS 22.280 [3] | Geographic area within which affiliation to the group is permitted (see NOTE 4) | Y | Y | Y |
| [R-6.4.9-006] of 3GPP TS 22.280 [3] | Geographic area outside which de-affiliation from the group is mandatory (see NOTE 4) | Y | Y | Y |
| Subclause 10.2.5 | List of authorised users who can retrieve the group information |  |  |  |
|  | > MCPTT IDs |  |  | Y |
|  | > MCVideo IDs |  |  | Y |
|  | > MCData IDs |  |  | Y |
| Subclause 5.2.3 | Associated relay service code (as specified in 3GPP TS 23.303 [14]) | Y | N | Y |
| Subclause 5.2.9 of 3GPP TS 23.280 [16 | List of partner MC systems from which interconnection is permitted for the group |  |  |  |
| Subclause 5.2.9 of 3GPP TS 23.280 [16 | > Identity of partner MCPTT system |  | Y | Y |
| Subclause 5.2.9 of 3GPP TS 23.280 [16 | > Identity of partner MCVideo system |  | Y | Y |
| Subclause 5.2.9 of 3GPP TS 23.280 [16 | > Identity of partner MCData system |  | Y | Y |
| [R-6.4.4-003] of 3GPP TS 22.280 [16] | List of functional aliases which prevents de-affiliating |  |  |  |
|  | > Functional alias | N | Y | Y |
| [R-6.4.4-004] of 3GPP TS 22.280 [16] | List of functional aliases which prevents de-affiliating, if only one user has bound the functional alias to the group |  |  |  |
|  | > Functional alias | N | Y | Y |
| NOTE 1: This parameter is a parent parameter whose child parameters are defined in 3GPP TS 23.379 [16]. NOTE 2: This parameter is a parent parameter whose child parameters are defined in 3GPP TS 23.281 [12]. NOTE 3: This parameter is a parent parameter whose child parameters are defined in 3GPP TS 23.282 [13]. NOTE 4: The geographic area may consist of a union of regions that are non-contiguous or contain holes. |  |  |  |  |

Table A.4-3: Common group configuration data (off-network)

| Reference | Parameter description | MC service UE | MC service server | Group management server |
| --- | --- | --- | --- | --- |
| Subclause 8.1.3.2 | ProSe layer-2 group ID (as specified in 3GPP TS 23.303 [14]) | Y | N | Y |
| Subclause 8.1.3.2 | ProSe group IP multicast address (as specified in 3GPP TS 23.303 [14]) | Y | N | Y |
| Subclause 8.1.3.2 | Indication of whether the UE shall use Ipv4 or Ipv6 for the MC service group (as specified in 3GPP TS 23.303 [14]) | Y | N | Y |
|  | MC service specific configuration |  |  |  |
|  | > MCPTT configuration (see NOTE 1) |  |  |  |
|  | > MCVideo configuration (see NOTE 2) |  |  |  |
|  | > MCData configuration (see NOTE 3) |  |  |  |
| NOTE 1: This parameter is a parent parameter whose child parameters are defined in 3GPP TS 23.379 [16]. NOTE 2: This parameter is a parent parameter whose child parameters are defined in 3GPP TS 23.281 [12]. NOTE 3: This parameter is a parent parameter whose child parameters are defined in 3GPP TS 23.282 [13]. |  |  |  |  |

A.5	MC service configuration data

The MC service configuration data is stored in the MC service server. The configuration management server is used to configure the MC service configuration data to the MC service server (CSC-5) and the configuration management client (CSC-4) on the MC service UE or non-3GPP device host the MC client.

The MC service configuration data can be configured offline using the CSC-11 reference point.

The MC service configuration data is specific to each MC service and hence the detailed list of MC service configuration data is listed as follows:

-	MCPTT service configuration data is specified in 3GPP TS 23.379 [16];

-	MCVideo service configuration data is specified in 3GPP TS 23.281 [12]; and

-	MCData service configuration data is specified in 3GPP TS 23.282 [13];

A.6	Initial MC service UE configuration data

The initial MC service UE configuration data is essential to the MC service UE or non-3GPP device host the MC client to successfully connect to the MC system. The initial MC service UE configuration data can be the same or different across MC service UEs or across non-3GPP devices host the MC client.

Data in table A.6-1 is provided to the MC service UE’s clients (e.g. MC service client, group management client, configuration management client, identity management client, key management client) during the bootstrap process (see subclause 10.1.1), and can be configured on the MC service UE or non-3GPP device host the MC client offline using the CSC-11 reference point or via other means e.g. as part of the MCPTT client’s provisioning on the MC service UE or non-3GPP device host the MC client, using a device management procedure.

Table A.6-1: Initial MC service UE configuration data (on-network)

| Reference | Parameter description |
| --- | --- |
| Subclause 10.1.1 | PDN connectivity information |
|  | > HPLMN ID and optionally VPLMN ID to which the data pertains |
|  | > MC services PDN |
|  | >> APN |
|  | >> PDN access credentials |
|  | > MC common core services PDN |
|  | >> APN |
|  | >> PDN access credentials |
|  | > MC identity management service PDN |
|  | >> APN |
|  | >> PDN access credentials |
| Subclause 10.1.1 | Application plane server identity information |
|  | > Identity management server |
|  | >> Server URI |
|  | > Configuration management server |
|  | >> Server URI |
|  | > Key management server |
|  | >> Server URI (also known as KMSUri for security domain managed by KMS) |
|  | > Indication of whether the UE shall use Ipv4 or Ipv6 for on-network MCPTT |
|  | > MCPTT Server |
|  | >> Server URI |
|  | > Indication of whether the UE shall use Ipv4 or Ipv6 for on-network MCData |
|  | > MCData Server |
|  | >> Server URI |
|  | > Indication of whether the UE shall use Ipv4 or Ipv6 for on-network MCVideo |
|  | > MCVideo Server |
|  | >> Server URI |
|  | > Location management server |
|  | >> Server URI |
| Subclause 10.1.1 | Operational information (NOTE 1) |
|  | > MC service UE label (see NOTE 2) |
| NOTE 1: This item is not required for the non-3GPP device host the MC client case. NOTE 2: The MC service label is optional. |  |