3GPP TSG-SA WG6 Meeting #60	S6-241267

Changsha, China 15th – 19th April 2024	(revision of S6-241xxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0623 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | ACR management event handling in ENS via leading ECSP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph3 |  |  |  |  |  | Date: |  |  | 2024-04-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The ENS service continuity via leading ECSP is not completed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add more procedure description for ENS service continuity via leading ECSP. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing procedures in supporting service continuity for ENS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Existing clauses: 6.7.2, 8.18.2.4.1 New clauses: 8.18.2.4.X1, 8.18.2.4.X2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Additional discussions:

This ACR mgmt events capability exposed by the EES supports:

1.	"User plane path change". This event supports to detect user plane path change for the application traffic and report the corresponding notification with user plane path change to the EAS.

2.	"ACR monitoring". This event supports to detect user plane path change for the application traffic, discover the T-EAS(s), and report the corresponding notification with the discovered T-EAS(s).

3.	"ACR facilitation". This event supports to detect user plane path change for the application traffic, make the decision for ACR, discover the T-EAS(s), influence the traffic for the selected T-EAS and report the corresponding notification with the selected T-EAS.

4.	"ACT start/stop". This event informs the EAS the need to start or stop an ACT towards or from another EAS for a particular UE. The "ACT start" can also inform the EAS of the ACR parameters.

5.	"ACR Selection". This event informs the EAS about the selected ACR scenario list for each AC using the EAS.

For 1 and 2, they are used in S-EAS decided ACR scenario. For 3, it is used in S-EES executed ACR scenario. For 4 and 5, they are used to convey information from the EEC and EES to EAS.

For 1, 2 and 3, they all depend on CN capability of UP path change. In ENS via leading ECSP, about which entity interacts with CN, there are two options:

|  | Overall signalling impact | T-EAS discovery (used in 2 and 3) | User plane path change detection (1, 2 and 3) and Traffic influence (used in 3) |
| --- | --- | --- | --- |
| a) S-EES of partner ECSP | Low. S-EES (P) does not need to relay the event subscription (and subsequent notifications) to S-EES (L) in 1, 2 and 3 | Easy to support Intra-EDN ACR, since S-EES (P) can directly identify local T-EAS(s) | Same cost |
| b) S-EES of leading ECSP | Medium | A bit more complex to support intra-EDN ACR, since S-EES (L) needs to interact with S-EES (P) to check possible T-EAS | Same cost |

In option a, allowing AF (EES, ECS) from partner to use CN service of leading ECSP is part of SLA of edge node sharing.  And Option a is used in solution below.

* * * First Change * * * *

### 6.7.2	APIs provided by the Edge Enabler Layer

Table 6.7.2-1 summarizes the APIs exposed by the ECS.

Table 6.7.2-1: APIs provided by the ECS

| API Name | Known Consumers | References |
| --- | --- | --- |
| Eecs_ServiceProvisioning | EEC | 8.3 |
| Eecs_EESRegistration | EES | 8.4.4 |
| Eecs_TargetEESDiscovery | EES, CES | 8.8.3.3 |
| Eecs_ECSRegistration | ECS | 8.17.2.2 |
| Eecs_ECSDiscovery | ECS | 8.17.2.3 |
| Eecs_ECSServiceProvisioning | ECS | 8.17.2.4 |
| Eecs_EASInfoManagement | EES | 8.20.2 |

Table 6.7.2-2 summarizes the APIs exposed by the EES.

Table 6.7.2-2: APIs provided by the EES

| API Name | Known Consumers | References |
| --- | --- | --- |
| Eees_EECRegistration | EEC | 8.4.2 |
| Eees_EASRegistration | EAS | 8.4.3 |
| Eees_EASDiscovery | EEC | 8.5 |
| Eees_UELocation | EAS | 8.6.2 |
| Eees_ACRManagementEvent | EAS, EES, CAS | 8.6.3 |
| Eees_AppClientInformation | EAS | 8.6.4 |
| Eees_UEIdentifier | EEC, EAS | 8.6.5 |
| Eees_SessionWithQoS | EAS | 8.6.6 |
| Eees_TrafficInfluenceEAS | EAS | 8.6.7 |
| Eees_TargetEASDiscovery | EAS, EES, CAS | 8.8.3.2 |
| Eees_AppContextRelocation | EEC, EAS, EES | 8.8.3.4 |
| Eees_ACREvents | EEC | 8.8.3.5 |
| Eees_EELManagedACR | EAS | 8.8.3.6 |
| Eees_SelectedTargetEAS | EAS, CAS, EES | 8.8.3.7 |
| Eees_ACRStatusUpdate | EAS, CAS, EES | 8.8.3.8 |
| Eees_ACRParameterInformation | EES, CES | 8.8.3.9 |
| Eees_EECContextPull | EES | 8.9.4.2 |
| Eees_EECContextPush | EES, CES | 8.9.4.3 |
| Eees_EASInformationProvisioning | EEC, EES | 8.15 |
| Eees_CommonEasAnnouncement | EES | 8.19 |

NOTE:	The event exposure related APIs (e.g. Eees_EASDiscovery and Eees_ACREvents) can be realized as single event subscription API.

Table 6.7.2-3 summarizes the APIs exposed by the CAS.

Table 6.7.2-3: APIs provided by the CAS

| API Name | Known Consumers | References |
| --- | --- | --- |
| Ecas_SelectedEES | EES | 8.8.3.10 |

Table 6.7.2-4 summarizes the APIs exposed the EEC.

Table 6.7.2-4: APIs provided by the EEC

| API Name | Known Consumers | References |
| --- | --- | --- |
| Eeec_ACRegistration | AC | 8.14.4.2 |
| Eeec_EASDiscovery | AC | 8.14.4.3 |
| Eeec_ACRTrigger | AC | 8.14.4.4 |
| Eeec_Services | AC | 8.14.4.5 |
| Eeec_UEId | AC | 8.14.4.6 |

Table 6.7.2-2 summarizes the EES APIs re-used by the CES.

Table 6.7.2-5: APIs re-used by the CES

| API Name | Known Consumers | References |
| --- | --- | --- |
| Eees_EASRegistration | CAS | 8.4.3 |
| Eees_UELocation | CAS | 8.6.2 |
| Eees_ACRManagementEvent | CAS | 8.6.3 |
| Eees_AppClientInformation | CAS | 8.6.4 |
| Eees_UEIdentifier | CAS | 8.6.5 |
| Eees_SessionWithQoS | CAS | 8.6.6 |
| Eees_TrafficInfluenceEAS | CAS | 8.6.7 |
| Eees_TargetEASDiscovery | EES | 8.8.3.2 |
| Eees_AppContextRelocation | CAS | 8.8.3.4 |
| Eees_ACREvents | EEC | 8.8.3.5 |
| Eees_SelectedTargetEAS | CAS | 8.8.3.7 |
| Eees_ACRStatusUpdate | CAS | 8.8.3.8 |
| Eees_ACRParameterInformation | EES | 8.8.3.9 |
| Eees_EECContextPush | EES | 8.9.4.3 |

* * * Next Change * * * *

##### 8.18.2.4.1	General

The AC connected EAS may be changed when the UE moves within the same partner ECSP' service coverage or to a different partner ECSP' service coverage or back to leading ECSP' service coverage.

S-EAS and T-EAS can be both provided by partner ECSP.

Some EEC request (over EDGE-1) or EAS request (over EDGE-3) may be relayed between leading EES and partner EES (over EDGE-9), with necessary modification (e.g. along with the ECSP ID of the sender). The following identifies a list of relayed procedures:

-	ACR launching

-	Selected T-EAS declaration

-	ACR status update

-	EAS Information provisioning

-	ACR management events

-	ACR parameter information

* * * Next Change * * * *

##### 8.18.2.4.X1	ACR management events

Upon receiving ACR management event subscribe request from the EAS (S-EAS or T-EAS depending on event):

-	For "ACT start/stop" or "ACR Selection" event, the partner EES uses the procedure as defined in clause 8.6.3.2.2 to send the subscribe request to the leading EES over EDGE-9 reference point. Upon receiving the response from the leading EES, the partner EES sends the response back to the EAS.

Upon receiving notification from the leading EES, the partner EES sends the notification to the S-EAS.

-	For "user plane path change" or "ACR monitoring" or "ACR facilitation" event, the partner EES interacts with the core network of the leading operator per event needs. For "ACR facilitation" event, once the partner EES selects T-EAS, the partner EES sends Selected T-EAS declaration request to the leading EES over EDGE-9 reference point using the procedure as defined in clause 8.8.3.7.

The partner EES further sends the notification to the S-EAS.

##### 8.18.2.4.X2	ACR parameter information procedure

Upon receiving the ACR parameter information request from the sending leading EES, the receiving leading EES uses the procedure as defined in clause 8.8.3.9 to send the request to the partner EES over EDGE-9 reference point. Upon receiving the response from the partner EES, the receiving leading EES sends the response back to the sending leading EES.

* * * END of Change * * * *