3GPP TSG-SA WG6 Meeting #60	S6-241271

Changsha, China 15th – 19th April 2024	(revision of S6-241xxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.436 | CR | 0030 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Edge computing preparation analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph3 |  |  |  |  |  | Date: |  |  | 2024-04-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In SA6#59 CR S6-240769 was agreed with the following Editor’s note: “Editor’s Note: Whether and how the ECS can obtain the EAS deployment time (e.g., from ADAES) is FFS”. In order to address this Editor’s note this paper proposes clarification in ADAES for using the OAM Network Resource Model to obtain edge deployment time information in a new procedure “Procedure for edge computing preparation analytics” for 3GPP TS 23.436. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new ADAES functionality, Procedure for edge computing analytics, is introduced The procedure specifies how edge (i.e., EAS, EES, and ECS) deployment time information can be obtained using the Network resource Model (specified in TS 28.623) and exposed to the analytics consumer. Historical deployment time can also be fetched from the A-ADRF in case of offline analytics or for predictions by the ADAE server. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Editor’s Note introduced in CR S6-240769 will not be solved and it will remain unclear how the ADAE server can obtain edge deployment time information. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Existing clauses: 2, 9.2.2, 9.3.2 New clauses: 6.x, 8.x, 8.x.1, 8.x.2, 8.x.2.1, 8.2.2.2, 8.x.3, 8.x.3.1, 8.x.3.2, 8.x.3.3, 8.x.3.4, 8.x.3.5, 8.x.3.6, 8.x.3.7, 8.x.3.8 9.2.x, 9.x.2.1, 9.x.2.2, 9.2.x.3, 9.2.x.4, 9.3.x, 9.2.x.1, 9.3.x.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| () |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

* * * First Change * * * *

## 6.x	Edge computing preparation analytics

This feature introduces exposure of edge computing preparation analytics of the EAS, EES, and/or ECS to the analytics consumer (e.g., the VAL server, ECS, EES). The ADAE server provides the edge computing preparation analytics based on collected edge deployment time information, historical edge computing preparation analytics, and registration time from the EDN.

* * * Next change * * * *

## 8.x	Procedure for edge computing preparation analytics

### 8.x.1	General

This clause describes the procedure for edge computing preparation analytics.

### 8.x.2	Procedure

#### 8.x.2.1	Subscribe-notify model

Figure 8.x.2.1-1 illustrates the procedure for edge computing preparation analytics.

Pre-conditions:

1.	ADAES is connected to A-ADRF.

2.	ADAES has subscribed to OAM for receiving management analytics.

Figure 8.x.2.1-1: ADAES support for edge computing preparation analytics subscribe-notify model

1.	The analytics consumer (e.g., VAL server, ECS, EES) of the ADAE server analytics service sends an Edge computing preparation analytics subscription request to the ADAE server.

2.	The ADAE server sends an Edge computing preparation analytics response as an ACK to the analytics consumer.

3.	The ADAE server utilizes OAM (Network Resource Model) for edge deployment time information for EAS, EES, or ECS. The deployment time information is determined from the Process Monitor that specifies, for instance, NOT_STARTED, RUNNING and start and/or end time for these states as described in 3GPP TS 28.623 [x].

4.	The ADAE server requests historical edge deployment time information of EAS, EES, and/or ECS from A-ADRF by sending an Edge computing preparation data request.

5.	Based on the request, the ADAE server receives historical edge deployment time information of EAS, EES, and/or ECS by receiving an Edge computing preparation data response from A-ADRF.

6.	The ADAE server collects EAS and/or EES registration time from the corresponding EES/ECS to determine when the EAS/EES service is ready.

7.	The ADAE server abstracts or correlates the data/analytics from step 3-6. Based on the request in step 1, ADAES may also provide predictions for the edge deployment time.

8.	The ADAE server sends an Edge computing preparation analytics notification to the analytics consumer, based on the request and the derived analytics in step 7. Such analytics indicate analytics and/or prediction of the edge deployment time for EAS, EES, and/or ECS.

#### 8.x.2.2	Request-response model

Figure 8.x.2.2-1 illustrates the procedure where the analytics consumer (e.g., VAL server, ECS, EES) requests edge computing preparation analytics using the request/response model.

Pre-conditions:

1.	ADAES is connected to A-ADRF.

2.	ADAES has subscribed to OAM for receiving management analytics.

Figure 8.x.2.2-1: ADAES support for edge computing preparation analytics request/response model

1.	The analytics consumer (e.g., VAL server, ECS, EES) sends an Edge computing preparation analytics retrieval request to the ADAES.

2.	Upon receiving the request, the ADAE server authenticates and authorizes the analytics consumer. If the analytics consumer is authorized, the ADAES performs step 3-7 of clause 8.x.2.1.

3.	The ADAES sends an Edge computing preparation analytics retrieval response to the analytics consumer.

### 8.x.3	Information flows

#### 8.x.3.1	General

The following information flows are specified for edge computing preparation analytics based on 8.x.2.

#### 8.x.3.2	Edge computing preparation analytics subscription request

Table 8.x.3.2-1 describes information elements for the edge computing preparation analytics subscription request from the analytics consumer (e.g., the VAL server, ECS, EES) to the ADAE server.

Table 8.x.3.2-1: Edge computing preparation analytics subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Analytics Consumer ID | M | The identifier of the analytics consumer (VAL server, ECS etc.). |
| Analytics ID | O | The identifier of the analytics event. This ID can be for example "edge computing preparation analytics". |
| Analytics type | M | The type of analytics for the event, e.g. statistics or predictions. |
| Reporting requirements | O | Requirements for analytics reporting. The requirements may include for example when the deployment information of the target (EAS, EES, ECS) is set to a specific state (such as "finished") or when the state is changed. |
| DNN/DNAI | O | DNN or DNAIs information of the EDN for which the subscription applies. |
| ECS provider ID | O | The identifier of the ECS provider |
| ECS endpoint | O | By providing the ECS endpoint, the consumer wants to subscribe to the EES instantiation info (EESs that will register in the ECS) |
| EES provider ID | O | The identifier of the EES provider |
| EES endpoint | O | By providing the EES endpoint, the consumer wants to subscribe to the EAS instantiation info (EASs that will register in the EES) |
| EAS provider ID | O | The identifier of the EAS provider |
| EAS ID | O | Identifies an EAS service. |
| EAS/EES/ECS resource requirements | O | The resources (i.e. available compute, graphical compute, memory, storage) needed for EAS/EES/ECS as described in Table 8.2.5-1 in 3GPP TS 23.558 [y]. |
| Preferred confidence level | O | The level of accuracy for the analytics service (in case of prediction). |
| Time validity | O | The time validity of the subscription request. |

#### 8.x.3.3	Edge computing preparation analytics subscription response

Table 8.x.3.3-1 describes information elements for the edge computing analytics preparation analytics subscription response from the ADAE server to the analytics consumer.

Table 8.x.3.3-1: Edge computing preparation analytics subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result of the analytics subscription request (positive or negative acknowledgement). |

#### 8.x.3.4	Edge computing preparation analytics notification

Table 8.x.3.4-1 describes information elements for the edge computing preparation analytics notification from the ADAE server to the analytics consumer.

Table 8.x.3.4-1: Edge computing preparation analytics notification

| Information element | Status | Description |
| --- | --- | --- |
| Analytics ID | O | The identifier of the analytics event. |
| Analytics Output | M | Represents the analytics output including prediction or statistics for the EAS/EES/ECS deployment time. |
| Confidence level | O | For predictive analytics, the achieved confidence level can be provided. |

#### 8.x.3.5	Edge computing preparation data request

Table 8.x.3.5-1 describes information elements for the edge computing preparation data request from the ADAE server to the A-ADRF/EES/ECS.

Table 8.x.3.5-1: Edge computing preparation data request

| Information element | Status | Description |
| --- | --- | --- |
| ADAE server ID | M | The identifier of the ADAE server. |
| Analytics ID | M | The identifier of the analytics event. |
| Target ECS information | O (NOTE) | This identifier shows the target ECS information |
| Target EES information | O (NOTE) | This identifier shows the target EES information |
| Target EAS information | O (NOTE) | This identifier shows the target EAS information |
| Time validity | O | The time validity of the request. |
| NOTE: At least one of these shall be present for A-ADRF as receiver. Target ECS information is included for collecting EES registration data and target EES information is included for collecting EAS registration data. |  |  |

#### 8.x.3.6	Edge computing preparation data response

Table 8.x.3.6-1 describes information elements for the edge computing preparation data response from the A-ADRF/EES/ECS to the ADAE server.

Table 8.x.3.6-1: Edge computing preparation data response

| Information element | Status | Description |
| --- | --- | --- |
| Analytics ID | M | The identifier of the analytics event. |
| Data Type | M | The type of reported data samples which can be network data, application data, edge data, or different granularities / abstraction of data (e.g., real time, non-real time). |
| Data Output | M | The reported data, which can be in form of measurements or offline/historical data on the requested parameter based on the request. The reported data includes historical ECS/EES/EAS deployment time information, EES/EAS registration data etc. |

#### 8.x.3.7	Edge computing preparation analytics retrieval request

Table 8.x.3.7-1 describes information elements for the Edge computing preparation analytics retrieval request from the analytics consumer to the ADAE server.

Table 8.x.3.7-1: Edge computing preparation analytics retrieval request

| Information element | Status | Description |
| --- | --- | --- |
| Analytics Consumer ID | M | The identifier of the analytics consumer (VAL server, ECS etc.). |
| Analytics ID | O | The identifier of the analytics event. This ID can be for example “edge computing preparation analytics”. |
| Analytics type | M | Whether analytics event is about prediction or statistics. |
| DNN/DNAI | O | DNN or DNAIs information of the EDN for which the subscription applies. |
| ECS provider ID | O | The identifier of the ECS provider |
| ECS endpoint | O | By providing the ECS endpoint, the consumer wants to subscribe to the EES instantiation info (EESs that will register in the ECS) |
| EES provider ID | O | The identifier of the EES provider |
| EES endpoint | O | By providing the EES endpoint, the consumer wants to subscribe to the EAS instantiation info (EASs that will register in the EES) |
| EAS provider ID | O | The identifier of the EAS provider |
| EAS ID | O | Identifies an EAS service. |
| EAS/EES/ECS resource requirements | O | The resources (i.e. available compute, graphical compute, memory, storage) needed for EAS/EES/ECS as described in Table 8.2.5-1 in 3GPP TS 23.558 [y]. |
| Preferred confidence level | O | The level of accuracy for the analytics service (in case of prediction). |

#### 8.x.3.8	Edge computing preparation analytics retrieval response

Table 8.x.3.8-1 describes information elements for the Edge computing preparation analytics retrieval response from the ADAE server to the analytics consumer.

Table 8.x.3.8-1: Edge computing preparation analytics retrieval response

| Information element | Status | Description |
| --- | --- | --- |
| Results | M | The result of the analytics data request (positive or negative acknowledgement). |
| Analytics ID | O | The identifier of the analytics event. |
| Analytics Output | M | Represents the analytics output including prediction or statistics for the EAS/EES/ECS deployment time. |
| Confidence level | O | For predictive analytics, the achieved confidence level can be provided. |

* * * Next change * * * *

### 9.2.2	ADAE server APIs

Table 9.2.2-1 illustrates the ADAE server APIs.

Table 9.2.2-1: List of ADAE server APIs

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_ADAE_VAL_performance_analytics | Subscribe | VAL server | Subscribe/Notify |
|  | Notify |  |  |
| SS_ADAE_slice_performance_analytics | Subscribe | VAL server | Subscribe/Notify |
|  | Notify |  |  |
| SS_ADAE_UE-to-UE_performance_analytics | Subscribe | VAL server | Subscribe/Notify |
|  | Notify |  |  |
| SS_ADAE_location_accuracy_analytics | Subscribe | VAL server | Subscribe/Notify |
|  | Notify |  |  |
| SS_ADAE_service_API_analytics | Subscribe | VAL server / Subscriber/ API invoker | Subscribe/Notify |
|  | Notify |  |  |
| SS_ADAE_slice_usage_pattern_analytics | Subscribe | VAL server / SEAL server | Subscribe/Notify |
|  | Notify |  |  |
| SS_ADAE_edge_analytics | Subscribe | VAL server / EAS / EES | Subscribe/Notify |
|  | Notify |  |  |
|  | Get | VAL server / EAS / EES | Request / Response |
| SS_ADAES_slice_usage_stats | Get | VAL server | Request / Response |
| SS_ADAES_edge_preparation_analytics | Subscribe | VAL server / ECS / EES | Subscribe/Notify |
|  | Notify |  |  |
|  | Get | VAL server / ECS / EES | Request / Response |

* * * Next change * * * *

### 9.2.x	SS_ADAE_edge_preparation_analytics API

#### 9.2.x.1	General

API description: This API enables the analytics consumer (e.g., the VAL server, ECS, EES) to communicate with the ADAE server for requesting or subscribing to edge computing preparation.

#### 9.2.x.2	Subscribe

API operation name: edge_preparation_analytics_subscribe

Description: The consumer subscribes for edge computing preparation analytics.

Inputs: See clause 8.x.3.2.

Outputs: See clause 8.x.3.3.

See clause 8.x.2.1 for details of usage of this operation.

#### 9.2.x.3	Notify

API operation name: edge_preparation_analytics_notify

Description: The consumer is notified by the ADAE server on the edge computing preparation analytics.

Inputs: -

Outputs: See clause 8.x.3.4.

See clause 8.x.2.1 for details of usage of this operation.

#### 9.2.x.4	Get

API operation name: edge_preparation_analytics_get

Description: The consumer requests edge computing preparation analytics

Inputs: -See clause 8.x.3.7

Outputs: See clause 8.x.3.8.

See clause 8.x.2.2 for details of usage of this operation.

* * * Next change * * * *

### 9.3.2	A-ADRF APIs

Table 9.3.2-1 illustrates the A-ADRF APIs.

Table 9.3.2-1: List of A-ADRF APIs

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_AADRF_Data_Collection | Subscribe | ADAES | Subscribe / Notify |
|  | Notify |  |  |
| SS_ AADRF_Historical_ServiceAPI_Logs | Get | ADAES | Request / Response |
| SS_AADRF_NetworkSlice_Data | Get | ADAES | Request / Response |
| SS_AADRF_Location_Accuracy_Data | Get | ADAES | Request / Response |
| SS_AADRF_EdgeData_Collection | Subscribe | ADAES | Subscribe / Notify |
|  | Notify | ADAES |  |
| SS_AADRF_Edge_Preparation_Data | Get | ADAES | Request / Response |

* * * Next change * * * *

### 9.3.x	SS_AADRF_Edge_Preparation_Data API

#### 9.3.x.1	General

API description: This API enables the ADAE server to communicate with the A-ADRF to request edge computing preparation data for the EAS, EES, and/or ECS.

#### 9.3.x.2	Get

API operation name: edge_preparation_data_get

Description: The consumer is receiving offline edge computing preparation data from the A-ADRF.

Inputs: See clause 8.x.3.5.

Outputs: See clause 8.x.3.6.

See clause 8.x.2.1 for details of usage of this operation.

* * * Next change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[3]	3GPP TS 26.531: "Data Collection and Reporting; General Description and Architecture"

[4]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[5]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics".

[6]	3GPP TS 23.435: "Procedures for Network Slice Capability Exposure for Application Layer Enablement Service".

[7]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[8]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs".

[9]	3GPP TS 23.501: “System architecture for the 5G System”.

[10]	GSMA NG.116 - Generic Network Slice Template.

[11]	3GPP TS 22.261: “Service requirements for the 5G system”.

[12]	3GPP TS 28.545: "Management and orchestration; Fault Supervision (FS)".

[13]	3GPP TS 23.433: "Service Enabler Architecture Layer for Verticals (SEAL); Data Delivery enabler for vertical applications".

[y]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

[x]	3GPP TS 28.623: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions".

* * * End of changes * * * *