3GPP TSG-SA WG6 Meeting #60	S6-241284

Changsha, China 9th – 13th April 2024	(revision of S6-24abcd)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.379 | CR | 0425 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Authorization check for MCPTT in-progress imminent peril group state cancel |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMC |  |  |  |  |  | Date: |  |  | 2024-02-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Only an authorized MCPTT user can cancel an MCPTT group’s in-progress imminent peril state. There is configuration exist in the user profile document for each user for authorization. However, the existing procedure is not validating the authorization of the user who is requesting for cancelling an MCPTT group’s in-progress imminent peril state. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added authorization validation when the request to cancel an MCPTT group's in-progress imminent peril state received by MCPTT server. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Any user cancels the MCPTT group’s in-progress imminent peril state. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.6.2.6.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

###### 10.6.2.6.2.3	MCPTT in-progress imminent peril group state cancel

NOTE 1:	In Rel-14 and Rel-13 versions of this specification the title of this subclause is "MCPTT imminent peril group call cancel".

The procedure focuses on the case where an authorized MCPTT user cancels an MCPTT group's in-progress imminent peril state.

Procedures in figure 10.6.2.6.2.3-1 are the signalling control plane procedures for the MCPTT client cancelling an MCPTT group's in-progress imminent peril state.

NOTE 2:	The end of the imminent peril call does not cancel the MCPTT group's in-progress imminent peril state. It is explicitly cancelled by an authorized user using this procedure.

Pre-conditions:

1.	The MCPTT group is previously defined on the group management server with MCPTT client 1, MCPTT client 2 and MCPTT client 3 affiliated to that MCPTT group.

2.	All members of the MCPTT group belong to the same MCPTT system.

3.	The MCPTT group is in an in-progress imminent peril state and has prioritized bearer support.

4.	MCPTT group members have been notified about the MCPTT group's in-progress imminent peril state.

5.	MCPTT client 1 previously initiated the in-progress imminent peril.

Figure 10.6.2.6.2.3-1: MCPTT in-progress imminent peril group state cancel

1.	The user at the MCPTT client 1 initiates an in-progress imminent peril group state cancel.

2.	MCPTT client 1 sends an MCPTT in-progress imminent peril group state cancel request to the MCPTT server.

3.	The MCPTT server verifies that the user of MCPTT client1 is authorized to cancel the imminent peril state of this MCPTT group. The MCPTT server cancels/resets the in-progress imminent peril state of the MCPTT group.

4.	The MCPTT server adjusts the priority of the underlying bearer; priority treatment is no longer required. The MCPTT server cancels/resets the in-progress imminent peril group state.

5.	MCPTT server resolves the MCPTT group ID to determine the members of that MCPTT group and their affiliation status, based upon the information from group management server.

6.	The MCPTT server sends an MCPTT imminent peril group state cancel request to the MCPTT group members.

7.	MCPTT group members are notified of the in-progress imminent peril group state cancel.

8.	The receiving MCPTT group members send the MCPTT in-progress imminent peril group state cancel response to the MCPTT server to acknowledge the MCPTT in-progress imminent peril group state cancel request. For a multicast scenario, these acknowledgements are not set.

9.	The MCPTT server sends the MCPTT in-progress imminent peril group state cancel response to the MCPTT user 1 to confirm the MCPTT in-progress imminent peril group state cancel request.

NOTE 3:	Step 9 can occur at any time following step 5, depending on the conditions to proceed with the call.

* * * * End of changes * * * *