3GPP TSG-SA WG6 Meeting #60	S6-241291

Changsha, China 15th – 19th April 2024	(revision of S6-241xxx)

Source:	Samsung

Title:	Pseudo-CR on AF leveraging S&F events information

Spec:	3GPP TR 23.700-01 V0.2.0

Agenda item:	8.6

Document for:	Approval

Contact:	Basavaraj (Basu) Pattan, basavarajjp@samsung.com

1. Introduction

This pCR proposes new KI on leveraging Store and Forward (S&F) Satellite operation events information at the application enablement layer.

2. Reason for Change

One of the objectives in FS_5GSAT_Ph3_App is to study the impacts to application enablers for supporting Store and Forward Satellite operation (when no simultaneous active feeder link connection to the ground segment).

When a UE registers to the network in S&F mode and is available to send/receive data on specific occasions only (e.g. when satellites fly-over and later connect to ground station), AF can leverage S&F events information. For e.g. if the UE is registered in S&F mode, the AF may want to adjust the frequency, size of data, message acknowledgement handling, ack timers etc. Another e.g. is AF can schedule delivery based on Feeder-Link availability.

Hence, it is desirable to study impacts to application enablers, while supporting S&F Satellite operation and S&F events information is shared to the AF.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-01 V0.2.0.

* * * First Change * * * *

## 4.x	Key issue #x: Usage of S&F events information for the application enablement

### 4.x.1	Description

3GPP SA1 has specified the service requirements for 5G for satellite access in TS 22.261[22.261] which includes supporting Store and Forward (S&F) Satellite Operation. And 3GPP SA2 is specifying the exposure of S&F events information to support these services requirements identified in SA1.

Services and application enablers defined by SA6 should also consider to take advantage of the output from SA1 and SA2 related to utilizing the S&F events information. When a UE registers to the network in S&F mode and is available to send/receive data on specific occasions only (e.g. when satellites fly-over and later connect to ground station), AF can leverage S&F events information. For e.g. if the UE is registered in S&F mode, the AF may want to adjust the frequency, size of data, message acknowledgement handling, ack timers etc. Another e.g. is AF can schedule delivery based on Feeder-Link availability etc.

Hence, it is desirable to study impacts to application enablers, while supporting S&F Satellite operation and S&F events information is shared to the AF.

### 4.x.2	Open issues

To support utilization of S&F events information by mission critical services and application enablers, the following aspects need to be studied:

-	What are the S&F events information that can be utilized by the application enablers?

-	How to use the S&F events information to optimise the application enablement behaviour?

-	What are the S&F events information that can be utilized by the mission critical services?

-	How to use the S&F events information to optimise the mission critical service behaviour?

* * * Next Change * * * *

<Proposed change in revision marks>

* * * Next Change * * * *

<Proposed change in revision marks>