3GPP TSG-SA WG6 Meeting #60	S6-241399

Changsha, China 15th – 19th April 2024	(revision of S6-241173)

Source:	Lenovo

Title:	Deployment scenarios

Spec:	3GPP TR 23.700-82 v0.3.0

Agenda item:	8.3

Document for:	Approval

Contact:	Emmanouil Pateromichelakis <epateromiche@lenovo.com>

1. Introduction

This paper includes the deployment scenarios for AIML enablement.

2. Reason for Change

This paper includes the deployment scenarios for AIML enablement.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-82 v0.3.0.

* * * First Change * * * *

# 9	Deployment scenarios

## 9.1	General

This clause provides the different deployment models for AIML enablement (AIMLE) services. There could be three deployment options:

-	AIMLE server can be deployed at a centralized cloud platform and collects data from multiple EDNs.

-	AIMLE server can be deployed at the edge platform.

-	Hierarchical AIMLE server deployment, where multiple AIML enablement services are deployed in edge or central clouds (e.g., in hierarchical architecture). Such deployment allows for local-global analytics for system wide optimization.

## 9.2	Deployment model #1: Cloud-deployed AIMLE server

In this deployment, as shown in Figure 9.2-1, the AIMLE server is centrally located and can provide support for AIML operations to the application and edge services (EAS/EES, VAL server, other SEAL services).

Figure 9.2-1 cloud deployed AIML enabler

## 9.3	Deployment model #2 Edge-deployed AIMLE server

In this deployment, as shown in Figure 9.3-1, the AIML enabler is located at the EDN and provides AIML enablement services to the EAS or other edge native applications at the edge platform. AIMLE services can be deployed by the ECSP or the MNO to provide value-add services related to AI/ML operations.

The ML support operations, that the edge deployed AIMLE Server provides, are applicable to the AIML enabler service areas (as shown in the example in Fig 9.3-1), which are equivalent to the EDN service areas.

Figure 9.3-1 edge deployed AIML enabler

## 9.4	Deployment model #3: Hierarchical AIMLE server deployment

In this deployment, multiple AIMLE servers can be located at different EDNs/DNs and can be deployed by the same provider.  Such hierarchical deployments allow the local – global ML operations (e.g. federated learning across domains).

The ML support services that the edge deployed AIML enaber correspond to the AIML enablement service areas (as shown in the example in Fig 9.4-1), which is equivalent to the EDN service areas. The central AIML enablement server covers all PLMN area and is used to coordinate the ML related operations (e.g. FL collaborator/server/aggregator) with the distributed AIML enabers.

Figure 9.4-1 hierarchical deployment of AIML enabler

* * * Next Change * * * *